# 3GPP TSG SA WG3 Meeting 103-e	S3- 211545

# Electronic meeting, 17-28 May 2021

Title:	Reply LS on Small data transmission

Response to:	LS S3-211426(R2-2104401) on Small data transmissions

Release:	Rel-17

Work Item:	NR_SmallData_INACTIVE-Core

Source:	SA3

To:	RAN2

Cc:

Contact person:	Abhijeet Kolekar

abhijeet.kolekar@intel.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	none

# 1	Overall description

SA3 thanks RAN2 for their LS on Small data transmission.

SA3 discussed the LS from RAN2 and acknowledge the security issues related to reusing the same I-RNTI and NCC with the same cell scenario or mobility scenarios as cell reselection. SA3 would like to provide the following answers to the questions.

Q1:  Can a CCCH message reusing the I-RNTI and resumeMAC-I be transmitted again in the same cell after SDT initiation, e.g. similar to legacy RRC Reject case (but without having received RRC Reject at the UE)?.

SA3 Answer: No, to avoid potential Replay Attack, UE should not send Resume Request message with the same I-RNTI and resumeMAC-I in the same cell after SDT initiation.

Q2: Can NCC and I-RNTI from a former cell in which an SDT procedure was initiated be reused to initiate a new SDT procedure in a new cell?.

SA3 Answer: No, reusing the same NCC and I-RNTI from a former cell to resume SDT procedure in the new cell violates the forward security principle, and such reuse should be avoided.

Solutions to get around this security issue could be possible, such as using horizontal key derivation from the old NCC.  SA3 has not discussed these.

# 2	Actions

To RAN2

ACTION: 	3GPP TSG SA WG3 asks RAN2 to take the above feedback into account.

# 3	Dates of next TSG CT WG1 meetings

TSG SA WG3 Meeting 103-Bis-e (TBC)	05-09 July 2021

TSG SA WG3 Meeting 104-e (TBC)	16-27 August 2021