3GPP TSG-SA3 Meeting #104-e 	S3-212413

# e-meeting, 16 - 27 August 2021

# Source:	3GPP CT1

Title:	LS reply on SDP attribute a=key-mgmt:mikey

# Document for:	Information, Discussion

Agenda Item:  3

# 3GPP TSG CT WG1 Meeting 130-e	TDoc C1-213548

# Electronic meeting, 20-28 May 2021

Title:	LS reply on SDP attribute a=key-mgmt:mikey

Response to:	LS R5-206283 (C1-212820) on SDP attribute a=key-mgmt:mikey from RAN5

Release:	Rel-17

Source:	CT1#130e

To:	RAN5

Cc:	SA3

Contact person:	Mike Dolan

michael.dolan@firstnet.gov

+1-571-524-1805

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

CT1 thanks RAN5 for their LS on SDP attribute a=key-mgmt:mikey.

Regarding the questions asked:

RAN5:	Can the key-mgmt attribute be a session level attribute or is it a media level attribute?

CT1:	Whether the key-mgmt attribute is used at the session level or media level, all instances of the attribute need to specify the same PCK, since 3GPP TS 33.180 does not define how to establish an MCVideo session using different keys for video and the associated audio. A session level attribute applies to all m-lines. If the attribute is not intended to apply to all m-lines the normal solution is to use media level attributes.

RAN5:	Should the same key-mgmt attribute be used for audio and video?

CT1:	Yes. A single PCK should be used for both.  3GPP TS 33.180 does not define how to establish an MCVideo session using different keys for video and the associated audio.

RAN5:	If the key-mgmt attribute is a session level attribute, how should media stream (floor control / pre-established call signaling...) be handled since the CSK should be used?

CT1:	Since the key-mgmt attribute is applicable for the media streams which transport the video and associated audio media, SRTCP, which is derived depending on the interface, is used for the media streams that include the floor control, media control, and pre-established session call control messaging. Detailed information can be found in 3GPP TS 24.581 subclause 13 on media plane security.

# 2	Actions

To: 	RAN WG5

ACTION: 	CT1 kindly asks RAN5 to take the above information into consideration in your work.

# 3	Dates of next TSG CT WG1 meetings

TSG-CT1 Meeting#131e 	 19-27 August 2021	Electronic Meeting

TSG-CT1 Meeting#132e 	 11-15 October 2021	Electronic Meeting