3GPP TSG-SA3 Meeting #104-e 	S3-212606

# e-meeting, 16 - 27 August 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS 33.512 | CR | 0013 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add reference to TS 33.512 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | eSCAS_5G |  |  |  |  |  | Date: |  |  | 2021-08-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Reference to TS 23.501 and TS 38.413 was not added before, so that the reference is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add reference and correct the references in the TS |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Reference is missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 4.2.2.1.3, 4.2.2.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*********** Start of Change***********

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 33.501 (Release 15): "Security architecture and procedures for 5G system".

[3]	3GPP TS 33.117: "Catalogue of general security assurance requirements".

[4]	3GPP TS 23.003: "Numbering, addressing and identification".

[5]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[6]	3GPP TR 33.926: "Security Assurance Specification (SCAS) threats and critical assets in 3GPP network product classes".

[7]	3GPP TS 33.501: "Security architecture and procedures for 5G system" (Release 16).

[8]	3GPP TS 23.501: "System Architecture for the 5G System".

[9]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)"

*********** End of 1st Change***********

*********** Start of 2nd Change***********

4.2.2.1.3	NAS based redirection from 5GS to EPS

Requirement Name: NAS based redirection from 5GS to EPS

Requirement Reference: TS 33.501 [2], clause 6.16.4. , TS 23.501 [8], clause 5.31.3.

Requirement Description: "When a UE initiates registration procedure with the AMF, the AMF may redirect the UE from 5GC to EPC by including a EMM cause indicating to the UE that it shall not use 5GC, as described in clause 5.31.3 in TS 23.501 [8]. The following requirements apply to Registration Reject message with an EMM cause which indicates to the UE that the UE shall not use 5GC:

-	the AMF shall only send such a Registration Reject message once NAS security has been established between the AMF and the UE; and

-	the UE shall only act upon such Registration Reject message if received integrity protected and if UE has verified the integrity of the Registration Reject message successfully.

NOTE 1:	This solution does not apply to unauthenticated emergency calls.

" as specified in TS 33.501 [2], clause 6.16.4. "

"In networks that support CIoT features in both EPC and 5GC, the operator may steer UEs from a specific CN type due to operator policy, e.g. due to roaming agreements, Preferred and Supported Network Behaviour, load redistribution, etc. Operator policies in EPC and 5GC are assumed to avoid steering UEs back and forth between EPC and 5GC.

" as specified in TS 23.501 [8], clause 5.31.3. "

Threat Reference:  TBD

Test Name: TC_AMF_REDIRCTION_5GS_EPS

Purpose:

Verify that AMF under test does not send a Registration Reject message containing an EMM cause indicating to the UE that the UE shall not use 5GC , if NAS security is not established. .

NOTE 2: This test case only applies to the AMF under test which supports the security handling in CIoT.

Pre-Conditions:

-	Test environment with UE. The UE may be simulated.

-	AMF under test is connected in emulated/real network environment.

-	Tester configures the operator policy of the AMF that all the UEs sending initial registration request should be redirected from 5GS to EPS.

Execution Steps

1. UE initiates initial registration procedure with the AMF.

2. The AMF under test determines that the UE shall not use 5GC, and needs to redirect the UE from 5GC to EPC.

3. The AMF under test sends a Registration Reject message with a 5GMM cause indicating to the UE that the UE shall not use 5GC.

Expected Results:

The NAS SMC is performed before sending the Registration Reject message.

Expected format of evidence:

Screenshot containing the operational results.

*********** End of 2nd Change***********

*********** Start of 3rd  Change***********

4.2.2.7	RRCRestablishment in Control Plane CIoT 5GS Optimization

Requirement Name: RRCRestablishment in Control Plane CIoT 5GS Optimization

Requirement Reference: TS 38.413 [9], clause 8.3.8.2

Requirement Description: "Upon receiving the RAN CP RELOCATION INDICATION message, the AMF shall authenticate the request using the NAS-level security information received in the UL CP Security Information IE and if the authentication is successful initiate the Connection Establishment Indication procedure including NAS-level security information in the DL CP Security Information IE.

In case the AMF cannot authenticate the UE's request, the CONNECTION ESTABLISHMENT INDICATION message does not contain security information, and the NG-RAN node shall fail the RRC Re-establishment.

In case of authentication failure, the NG-RAN node and the AMF should locally release the allocated NG resources, if any." as specified in TS 38.413 [9], clause 8.3.8.2.

Threat References: TR 33.926 [5], clause K.2.9.1 –Failed Verification of UE Identity during RRC Reestablishment Procedure for CP CIoT 5GS Optimization.

Test Case:

Test Name: TC_AMF_REEST_CP_CIOT

Purpose: To verify that the verification of RRC Reestablishment is applied correctly.

Pre-Condition:

Test environment with UE and ng-eNB, which may be simulated. The UE is using Control Plane CIoT 5GS Optimization.

Execution Steps:

A.	Test Case 1

1)	The UE sends the RRC Connection Reestablishment Request message to the ng-eNB.

2)	The ng-eNB sends RAN CP RELOCATION INDICATION message to the AMF.

B.	Test Case 2

1)	The UE sends the RRC Connection Reestablishment Request message to the ng-eNB.

2)	The ng-eNB sends RAN CP RELOCATION INDICATION message to the AMF. The ng-eNB modifies UL NAS MAC in UL CP Security Information

Expected Results:

For test case 1, the AMF sends CONNECTION ESTABLISHMENT INDICATION to the ng-eNB, and DL CP Security Information is included.

For test case 2, the AMF sends CONNECTION ESTABLISHMENT INDICATION to the ng-eNB, and DL CP Security Information is not included.

Expected format of evidence:

Evidence suitable for the interface, e.g., Screenshot containing the operational results.

*********** End of Change***********