3GPP TSG-SA3 Meeting #104-e 	S3-212825

# e-meeting, 16 - 27 August 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.535 | CR | 0097 | rev |  | Current version: | 17.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KAF refresh |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | 850021 |  |  |  |  |  | Date: |  |  | 2021-08-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | 17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As per the discussion paper (S3-212901), the KAF may not be refreshed after its lifetime expiry, until a new primary authentication takes place. This means a user may not be able to use an application (requiring authentication using AKMA) after the KAF expires and until a new primary authentication procedure takes place (which may happen after a very long period of time). The KAF should be refreshed when needed by the applications that are depending on the KAF and hence a key refresh mechanism is required for the AF to get a fresh KAF key, once the lifetime is expired. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR proposes: - Add a freshness parameter when deriving KAF from KAKMA. - Upon KAF lifetime expiry, AF requests AAnF to provide new KAF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support for explict triggering of AKMA key refresh, leads to unavailability of service, if there is no valid KAF exist in the UE and in the AF. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** Start of Changes ***

### 6.4.3	KAF refresh

Ua* protocol may support refresh of KAF. If the Ua* protocol supports refresh of KAF, the AF may refresh the KAF at any time using the Ua* protocol.

If Ua* protocol does not support refresh of KAF, the AF may initiate the KAF refresh procedure following the procedure described in clause 6.X.

*** Next Change ***

6.X	KAF refresh procedure

- When KAF is about to expire, AF requests AAnF for a new KAF, with A-KID, AF ID and a refresh indicator.

- Upon receving the key request with a refresh indicator from the AF, AAnF generates a freshness parameter and requests UDM to initiate UPU procedure carrying the generated freshness parameter.

Editor's Note: The service provided by UDM is FFS.

- UDM iniates UPU procedure as specified in clause 4.20, TS 23.501 [3], and transfers the freshness parameter as the UPU data to the UE.

- With a successful UPU procedure, UDM respondes AAnF with a response.

- AAnF generates the new KAF according to Annex A.4.

- AAnF sends the new KAF and its lifetime to AAnF via the Naanf_AKMA_ApplicationKey_Get_Response.

- AAnF sends the application key refresh notice to the UE, indicating the UE to generate a new KAF based on the freshness parameter.

*** Next Change ***

A.4	KAF derivation function

When deriving a KAF from KAKMA, the following parameters shall be used to form the input S to the KDF:

-	FC = 0x82;

-	P0 =AF_ID;

-	L0 = length of AF_ID;

-	P1 =freshness parameter;

-	L1 = length of freshness parameter;

The input key KEY shall be KAKMA.

AF_ID is constructed as follows:

AF_ID = FQDN of the AF || Ua* security protocol identifier, where the Ua* security protocol identifier is specified as Ua security protocol identifier in Annex H of TS 33.220 [4].