3GPP TSG-SA3 Meeting #105-e 	draft_S3-214017-r1

- e-meeting, 8 – 19 November 2021										              Revision of S3-21xxxx

- Source:	Huawei, HiSilicon

- Title:	User consent check

- Document for:	Approval

Agenda Item:	4.22

# 1	Decision/action requested

SA3 is kindly requested to approve the proposed changes on UDM services.

# 2	References

[1]	TR 33.867 v0.6.0 “Study on User Consent for 3GPP services”

# 3	Rationale

TR 33.867 [1] has concluded the UDM service for user consent check and revocation respectively in clause 8.5.1 and clause 8.5.3. Therefore, the contribution proposes to specify the conclusion in the annex for user consent accordingly.

# 4	Detailed proposal

*************** Start of 1st Change ****************

- X.2		Requirements

The UDM shall support the following services related to the user consent.

-	Retrieval of user consent parameters.

-	Notification of user consent parameters change.

The user consent parameters shall be stored in the UDM/UDR as subscription data. The user consent parameters shall be bound to a SUPI, and shall include the purpose of data processing and whether the user consent is granted or not.

The user consent parameters shall be effective only after the point in time that user consent was given, and they shall be effective until they are revoked.

NOTE:	UDM does not provide revocation service, it only provides notification.

*************** End of 1st Change ****************