3GPP TSG-SA3 Meeting #108e-AdHoc 	S3-222459

# e-meeting, 10 - 14 October 2022

# Source:	3GPP SA2

# Title:	Reply LS on FS_eEDGEAPP Solution for Support of NAT deployed within the edge

# Document for:	Information, Discussion

Agenda Item: 3

SA WG2 Meeting #S2-152E	S2-2207394

17 - 26 August, 2022, Electronic meeting

Title:	Reply LS on FS_eEDGEAPP Solution for Support of NAT deployed within the edge

Response to:	S2-2205458 / S6-221953

Release:	Release 18

Work Item:	FS_eEDGEAPP

Source:	SA2

To:	SA6

Cc:	SA3

Contact Person:

#### Name:	Sašo Stojanovski

E-mail Address:	saso stojanovski intel com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

SA2 thanks SA6 on their LS (S2-2205458 / S6-221953) and provides the following feedback on the SA6 questions.

- As mentioned in KI#16 about AF (e.g. EES) with a NATted UE IP address, can SA2 support taking NATted UE IP address as input and then exposing the UE External ID to EES?

Currently (in Rel-17) there is no specification support for NAT devices controlled by 5GS, as indicated in the following NOTE in TS 23.501 clause 5.6.10.1:

NOTE 2:	An operator can deploy NAT functionality in the network; the support of NAT is not specified in this release of the specification.

as well as in the following NOTE in TS 23.502 clause 4.15.10:

NOTE 3:	The case where UE IP address provided by the AF to the NEF corresponds to an IP address that has been NATed (Network and Port Address Translation) is not supported in this release.

As a consequence, there is currently no specification support for exposure of UE External ID based on the NATted UE IP address.

SA2 would like to point out that there is a related work as part of the ongoing Rel-18 study FS_UPEAS. However SA2 has not yet concluded or agreed any solution for normative work.

- In solution #23, EES (as AF) invokes Nnef_UEID API with UE private IP address allocated by 3GPP CN. In certain deployment when multiple UEs are allocated with the same private IP address, can SA2 support addressing IPv4 address overlap issue?

The case where multiple UEs are allocated with the same private 5GC IP address is currently addressed as follows

- when this same private IP address is allocated to different UE(s) for different DNN and S-NSSAI(s) by associating the AF with a DNN and S-NSSAI

- Otherwise and furthermore, the "ipDomain" attribute as defined in TS 29.514 clause 4.2.2.2 Note 3 may be leveraged

The above DNN/S-NSSAI or ipDomain attribute is not expected to be exposed outside the 5GC network.

2. Actions:

To SA6 group.

ACTION: 	SA2 respectfully asks SA6 to take into account the feedback above.

3. Date of Next TSG-SA WG2 Meetings:

3GPP SA2#153E			10 - 14 October 2022			e-meeting