3GPP TSG-SA3 Meeting #108e-AdHoc 	S3-222548

# e-meeting, 10 - 14 October 2022

# Source:	Huawei, HiSilicon

# Title:	NSSAA supporting temporay slices

# Document for:	Approval

Agenda Item:	5.12

# 1	Decision/action requested

Approve the new solution proposal to eNS3 TR33.886

# 2	References

[1]

# 3	Rationale

The contribution proposes a new solution to the KI#2.

# 4	Detailed proposal

pCR

***  BEGINNING OF CHANGES  ***

## 5.Y	Solution #Y: NSSAA supporting temporary slices

### 5.Y.1	Introduction

This solution addresses the Key Issue #2: temporary slice authorization and slice service area authorization.

In order to support a temporary slice and avoid abrupt PDU Session release, multiple solutions are being investigated in the TR23.700-41 [3]. These solutions share the same concept that the AMF will send a validity timer with the Allowed S-NSSAI. However, it is not addressed that how the timer is received by the AMF for network slices subject to the network-slice-specific authentication and authorization (NSSAA). This solution is to fill the gap providing the timer information for slices requiring NSSAA

### 5.Y.2	Solution details

This solution is also based on the idea of AMF seding a conditionally Allowed S-NSSAI with a validity timer to the UE. The main differences in this solution is to consider the network slices that are subject to NSSAA procedure and how the validity timer is delivered to the AMF before sending to the UE.

The detailed steps involving NSSAA are shown in the figure below.

1. UE sends AMF a Registration Request with a list of S-NSSAIs as specified in clause 16.2 of TS33.501 [4].

2. AMF receives subscription information from UDM. For temporary slices not requiring NSSAA, the termination times for each of the slices are included.

3. AMF sends the Registration Accept message. For the authorized temporary slices with their S-NSSAIs included in the allowed NSSAI, the corresponding termination times are also included.

4. AMF initiates NSSAA procedures for those slices requiring NSSAA.

5. AAA-S sends the NSSAA results to the NSSAAF, if successful, containing the authorized termination time.

6. AMF updates the UE with the UE Configuration Update procedure, where the Allowed NSSAI includes the corresponding termination times for the temporay slices.

Figure 5.Y.2-1: NSSAA supporting temporary slices

### 5.Y.3	Evaluation

***	END OF CHANGES	***