3GPP TSG-SA3 Meeting #108e-AdHoc 	S3-222808

# e-meeting, 10 - 14 October 2022																revision of S3-221854

# Source:	Nokia, Nokia Shanghai Bell

# Title:	KI3 EN resolution in sol12

# Document for:	Approval

Agenda Item:	5.24

# 1	Decision/action requested

Resolution of editor’s note in solution 12.

# 2	References

[1]	3GPP TR 33.875

# 3	Rationale

As resolution of

Editor’s Note: It is ffs how the solution works in case of notification target reselection as described in clause 6.3.1.0 of TS 23.501.

It is proposed to add a sentence on binding and reference the table in  TS 23.501, Table 6.3.1.0-1: Binding, selection and reselection provides details.

Binding can be used to indicate suitable target NF producer instance(s) for NF service instance selection, reselection and routing of subsequent requests associated with a specific NF producer resource (context) and NF service. This allows the NF producer to indicate that the NF consumer, for a particular context, should be bound to an NF service instance, NF instance, NF service set or NF set depending on local policies and other criteria (e.g. at what point it is in the middle of a certain procedure, considering performance aspects etc).

Binding can also be used by the NF consumer to indicate suitable NF consumer instance(s) for notification target instance reselection and routing of subsequent notification requests associated with a specific notification subscription and for providing binding indication for service(s) that the NF consumer produces for the same data context and the NF service producer is subsequently likely to invoke.

If NF_C is part of a NF Set (or NF Service Set), then another NF of the same set can be reselected, if CCA of NF_C or its certificate includes the NF Set Id (or NF Service Set Id). One potential use case is that NF_C is not reachable.

# 4	Detailed proposal

************* START OF CHANGES

## 6.12	Solution #12: Authorization of notification endpoint in “Subscribe-Notify” scenarios

### 6.12.1	Introduction

This solution addresses key issue #3: Service access authorization in the “Subscribe-Notify” scenarios by including “notification URI” in the token request.

This solution proposes to include “notification URI” and CCA of the NF which owns the “notification URI” at the access token request. The NRF verifies whether the notification URI is authorized to receive the service/notification that is requested in the access token request.

### 6.12.2 	Solution details

During access token request process, NF Service Consumer include "notification URI" in Nnrf_Access Token_Get Request when operation semantics of the requested service type is "Subscribe/Notify". NRF may verify whether the notification URI in the access token request match the corresponding information in the public key certificate of the NF Service Consumer or those in the NF profile of the NF Service Consumer. In addition, when "notification URI"(s) are included in the access token request, NRF may verify whether the "notification URI"(s) locate its location(s) with the same address(es) which are indicated by its own FQDN or IP address as registered in NRF. The NRF checks whether the NF Service Consumer is authorized to access the requested service(s).

When the NF Service Consumer is authorized, the NRF shall then generate an access token with appropriate claims which may include “notification URI” as requested by NF Service Consumer.

Figure 6.12.2.-1: Access token request for "Subscribe-Notify" NF Service illustration 1

A NF Service Consumer (NF_A) may request to subscribe to NF Service offered by a NF Service Producer (NF_B) on behalf of NF_C according to the Service Request from NF_C (e.g. Figure 4.15.3.2.2-1: Nudm_EventExposure_Subscribe, Unsubscribe and Notify operation in TS23.502).

The Figure 6.12.2-2 describe the solution to verify the service request from NF_A including Notification URI of NF_C.

Figure 6.12.2-2: Access token request for "Subscribe-Notify" NF Service illustration 2

Step 0.	The NF_C sends a Service Request to the NF_A for notification service on an event with an access token including notification URI and CCA of the NF_C.

Step 1.	The NF_A decides to subscribe a service of the NF_B associated to the Service Request received at step 0 and sends an Access Token Request to the NRF for a Service Request toward the NF_B including notification URI and CCA of the NF_C.

Step 2.	The NRF verifies whether the NF_A is authorized and whether the NF_C identified by the appended CCA is a valid Network Function and authorized to receive the requested NF Service from the NF_B. NRF may verify whether the notification URI in the access token request match the corresponding information in the public key certificate of the NF_C of the CCA of the NF_C, or those in the NF profile of the NF_C.

Step 3.	Upon successful verification, the NRF publishes an access token for a Service Request of the NF_A toward the NF_B. The access token includes notification URI of the NF_C in the claims as requested in step 2.

Step 4.	The NF_A sends a Service Request for a notification service toward NF_B including the access token received at step 3 and CCA of the NF_A.

Step 5.	The NF_B verifies the access token and CCA of the NF_A. During the verification, the NF_B can check whether notification URI in the Service Request are included in the access token. If the verification is successful, the NF_B may respond the NF_A and provide the NF_C with the notification service as requested by the NF_A. In case of notification target reselection as described in clause 6.3.1.0 of TS 23.501, binding can be used to indicate suitable NFs other than NF_C (see 23.501, Table 6.3.1.0-1 on binding, selection and reselection).

### 6.12.3 	Evaluation

TBD.

************* END OF CHANGES