3GPP TSG-SA3 Meeting #108e-AdHoc 	S3-222547

# e-meeting, 10 - 14 October 2022

# Source:	Huawei, HiSilicon

# Title:	New solution to KI#1 protecting slice-specific SoR information

# Document for:	Approval

Agenda Item:	5.12

# 1	Decision/action requested

Approve the new KI proposal to eNS3 TR33.886

# 2	References

[1]

# 3	Rationale

The contribution proposes a new solution to the key issue 1.

# 4	Detailed proposal

pCR

***  BEGINNING OF CHANGES  ***

## 5.Y	Key Issue #Y: protecting slice-specific SoR information

### 5.Y.1	Introduction

This solution addresses key issue 1, providing VPLMN slice information to roaming UE.

The solution provides integrity protection to the slice-specific Steering of Roaming (SoR) information sent to the roaming UE, adapting from the solution in TS33.501 [4].  It is applicable to either the procedure during roaming UE registration or a UDM initiated procedure.

### 5.Y.2	Solution details

The steps of the solution are described as follows, with reference to figure 5.Y.2-1.

1) The UDM decides to send the SoR Information, and obtains a list of preferred PLMN/access technology combinations for each identified slice S-NSSAI.

Editor’s Note: it is ffs on how to identify the slices, e.g. whether/what/when slice-related information is required for UE to send and which slices are authorized etc.

2-3) The UDM shall invoke Nausf_SoRProtection service operation message to the AUSF to get SoR-MAC-IAUSF and CounterSoR as described below. The UDM shall select the AUSF that holds the latest KAUSF of the UE.

Calculation of a SoR-MAC-IAUSF: the parameter P2 specified in A.17 of TS33.501 [4] shall be replaced with

P2 = [S-NSSAI-1, (list of preferred PLMN/access technology combinations per S-NSSAI-1), S-NSSAI-2, (list of preferred PLMN/access technology combinations per S-NSSAI-2) …]

NOTE: S-NSSAI-1, S-NSSAI-2 … are idendified slices in the step 1.

The inclusion of S-NSSAIs in the calculation of SoR-MAC-IAUSF allows the UE to verify that the received SOR Information is not tampered with or removed by the VPLMN.

CounterSoR: as specified in clause 6.14.2.3 of TS33.501 [4]

If the HPLMN decides that the UE is to acknowledge the successful security check of the received slice-specific SoR Information, then the UDM shall set accordingly the ACK Indication included in the Nausf_SoRProtection service operation message to signal that it also needs the expected SoR-XMAC-IUE

Figure 5.Y.2-1 Procedure for providing slice-specific SoR information

4)	The UDM sends a message Nudm_X to the VPLMN AMF, which shall include the slice-specific SoR information (P2 at step 2), SoR-MAC-IAUSF and CounterSoR. If the UDM requests an acknowledgement, it shall temporarily store the expected SoR-XMAC-IUE.

Calculation of a SoR-XMAC-IUE: as specified in A.18 of TS33.501 [4].

5) If the SoR information is received from the UDM, the VPLMN AMF shall include the received SoR information in a NAS message and send to the UE.

6) On receiving the NAS message with the SoR information from the AMF, the UE shall calculate the SoR-MAC-IAUSF in the same way as the AUSF (as step 3), and verifies whether it matches the SoR-MAC-IAUSF value received.

7) If the UDM has requested an acknowledgement from the UE and the verification is successful at step 6, the UE shall send a NAS message to the serving AMF. The UE shall generate the SoR-MAC-IUE as specified in Annex A.18 and includes the generated SoR-MAC-IUE in the NAS message.

8) The AMF sends a Nudm_Y message to the UDM, including the SoR-MAC-IUE.

9) If the HPLMN indicated that the UE is to acknowledge the successful security check of the received SoR  Information at step 4, the UDM shall compare the received SoR-MAC-IUE with the expected SoR-XMAC-IUE that the UDM stored temporarily in step 4.

### 5.Y.3	Evaluation

***	END OF CHANGES	***