3GPP TSG-SA3 Meeting #110Ad-Hoc-e 	draft_S3-232042-r1

- Electronic meeting, Online, 17 - 21 April 2023								(revision of S3-232042)

- Source:	Xiaomi

- Title:	33.893: Conclusion on Key Issue #4

- Document for:	Approval

Agenda Item:	5.19

# 1	Decision/action requested

SA3 is kindly asked to approve the proposed conclusion to be added for key issue #4 in TR 33.893.

# 2	References

[1]	TR 33.893 v0.6.0		Study on Security Aspects of Ranging Based Services and Sidelink Positioning

# 3	Rationale

This pCR proposes to conclude on key issue #4 of TR 33.893 [1].

# 4	Detailed proposal

*************** Start of the Change ****************

# 7	Conclusions

7.x	Conclusions on Key Issue #4

For protection of unicast direction communication, the following conclusions are made:

-	To establish secure SR5 direct communication for Ranging-based services provided by application providers, the security mechanisms defined for V2X unicast mode communication in TS 33.536 [5] and for 5G ProSe unicast mode Direct Communication in TS 33.503 [6] are reused.

-	To establish secure SR5 direct communication for network assisted SL positioning services provided by operators, the security mechanisms defined for 5G ProSe UE-to-Network Relay communication in TS 33.503 [6] are reused.

-	To protect RSPP control messages on SR5 reference point, SR5 security policies are to be enforced on PDCP layer. SR5 security policy for integrity protection of RSPP messages is set as “REQUIRED” only. SR5 security policy for confidentiality protection of RSPP messages can be set as “REQUIRED”, “PREFERRED” or “NOT NEEDED” depending on operator’s configuration.

NOTE: 	There is only one set of security policies configured for one service (i.e. PC5 security policies for ProSe/V2X service, SR5 security policies for Ranging/SL positioning service).

*************** End of the Change ****************