3GPP TSG-SA3 Meeting #112	S3-233605

# Goteborg, Sweden, 14th Aug 2023 - 18th Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.512 | CR | 0040 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of Replay Protection of NAS signalling messages |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | BSI (DE) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SCAS_5G_Ph2 |  |  |  |  |  | Date: |  |  | 2023-08-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The 1st Execution Step orders the tester to capture the NAS Security Mode Command procedure. In the following steps other arbitrary NAS signalling messages are mentioned. To be consistent, the test should focus on the NAS Security Mode Command procedure. (A more general approach would be to check for replay protection of every UE originating message of every NAS signalling procedure – this would significantly enhance the tests security statement but also its scale.) Replaying a filtered packet is less error prone than crafting a packet. The corresponding Execution Step can be simplified. The content of Execution Steps 5 and 6 can be transferred to Step 4. Since checking for Replay Protection of NAS signalling at the AMF, only UE originating messages could be replayed. The wording of the Expected Results implies a bidirectional replay protection, which is out of scope for the AMF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removed non 3GPP abbreviation SMC from Execution Steps. Modified Execution Step 3 to simplify instructions. Added content of Execution Step 5 and 6 to Step 4. Modified wording of Expected Results. Modified Expected format of evidence. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Execution Steps are not clear and lead to non-comparable or non-repeatable execution of the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.2.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

********** START OF 1st CHANGE **********

##### 4.2.2.3.1	Replay protection of NAS signalling messages

Requirement Name: Replay protection of NAS signalling messages

Requirement Reference: TS 33.501 [7], clause 5.5.2.

Requirement Description: " The AMF shall support integrity protection and replay protection of NAS-signalling." as specified in TS 33.501 [2], clause 5.5.2.

Threat References: TR 33.926 [6], clause K.2.3.1, Bidding Down

Test case:

Test Name: TC_NAS_REPLAY_AMF

Purpose:

Verify that the NAS signalling messages are replay protected by AMF over N1 interface between UE and AMF.

Procedure and execution steps:

Pre-Condition:

-	AMF network product is connected in emulated/real network environment.

-	Tester shall have access to the NAS signalling packets sent between UE and AMF over N1 interface.

-	Tester shall ensure that integrity protection algorithm other than NIA0 is used.

Execution Steps:

1.	The tester shall capture the NAS Security Mode Command procedure taking place between UE and AMF over N1 interface using any network analyser.

2.	The tester shall filter the NAS Security Mode Complete message by using a filter.

3.	The tester shall replay the captured NAS Security Mode Complete message.

4.	The tester shall check whether the replayed NAS Security Mode Complete message was not processed by the AMF by capturing traffic over the N1 interface to see if no corresponding response message was sent by the AMFIf applicable, AMF application logs could be checked for the rejection of the replayed NAS Security Mode Complete message.

Expected Results:

The NAS signalling messages sent from the UE to the AMF over N1 interface are replay protected.

Expected format of evidence:

Evidence suitable for the interface, e.g., Screenshot containing the operational results.

********** END OF CHANGE **********