3GPP TSG-SA3 Meeting # 112	S3-233847

# Gothenburg, Sweden, 14 -18 August 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | 1742 | rev | - | Current version: | 17.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Authentication result removal |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17 |  |  |  |  |  | Date: |  |  | 2023-08-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | During the study of authentication enhancements in the 5G, SA3 has addresses a key issue on attacks due to expired authentication result in the UDM. SA3 concluded the work on this key issue by sending an LS to CT4 to resolve the issue without the need to record any normative requirement in stage 2 specifications. The feature has been afterwards addressed and captured in CT4’s specification. Nevertheless, for the sake of completeness and future reference, it is better to include the necessary requirements covering stage 3 work. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding two services in AUSF and UDM to perform the authentication result removal procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Invalid authentication result stored in UDM |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New clause |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** Start of Changes****

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

…

[X]	3GPP TS 29.509: "5G System; Authentication Server Services".

**** Next Changes****

6.1.4.1a	Linking authentication confirmation to Nudm_UECM_Registration procedure from AMF

The information sent from the AUSF to the UDM that a successful or unsuccessful authentication of a subscriber has occurred, shall be used to link authentication confirmation to subsequent procedures. The AUSF shall send the Nudm_UEAuthentication_ResultConfirmation service operation for this purpose as shown in figure6.1.4.1a-1.

Figure 6.1.4.1a-1: Linking increased Home control to subsequent procedures

1.	The AUSF shall inform UDM about the result and time of an authentication procedure with a UE using a Nudm_UEAuthentication_ResultConfirmation Request. This shall include the SUPI, a timestamp of the authentication, the authentication type (e.g. EAP method or 5G-AKA), and the serving network name.

NOTE: 	It may be sufficient for the purposes of fraud prevention to send only information about successful authentications, but this is up to operator policy.

2.	The UDM shall store the authentication status of the UE (SUPI, authentication result, timestamp, and the serving network name).

3.	UDM shall reply to AUSF with a Nudm_UEAuthentication_ResultConfirmation Response.

4.	Upon reception of subsequent UE related procedures (e.g. Nudm_UECM_Registration_Request from AMF) UDM may apply actions according to home operator’s policy to detect and achieve protection against certain types of fraud (e.g. as proposed in clause 6.1.4.2).

6.1.4.X	Authentication result removal

In the case of  purge of subscriber data in AMF after the UE deregisters from the network or the NAS SMC fails following the successful authentication the registration procedure as specified in clause 5.2.2.2.5 of TS 29.509 [x], the AMF needs to inform the UDM through the AUSF to remove the authentication result. In order to achiev this, the AMF shall invoke the Nausf_ UEAuthenticationResultRemoval service (see clause 14.1.x) by sending a Nausf_ UEAuthenticationResultRemoval Request message to the AUSF. The Nausf_ UEAuthenticationResultRemoval Request message shall contain the SUPI and serving network name.

The AUSF shall send a Nudm_ UEAuthenticationResultRemoval Request (see clause 14.2.x) to the UDM for removing the authentication result of the UE after receiving the above message. The Nudm_ UEAuthenticationResultRemoval Request sent from AUSF to UDM includes the SUPI and the serving network name.

Upon reception of the Nudm_ UEAuthenticationResultRemoval Request, the UDM shall mark the status of the authentication as invalid while keep the authentication event to record the AUSF ID.

**** Next Changes****

14.1.X	Nausf_UEAuthenticationResultRemoval service operation

Service operation name: Nausf_ UEAuthenticationResultRemoval

Description: Remove the authentication result in UDM in the case that the Purge of subscriber data in AMF after the UE deregisters from the network or the NAS SMC fails following the successful authentication the registration procedure.

Input, Required: SUPI, serving network name

Input, Optional: None

Output, Required: None

Output, Optional: None

**** Next Changes****

14.2.X	Nudm_UEAuthenticationResultRemoval service operation

Service operation name: Nudm_UEAuthenticationResultRemoval

Description: Remove the authentication result in UDM in the case that the Purge of subscriber data in AMF after the UE deregisters from the network or the NAS SMC fails following the successful authentication the registration procedure.

Input, Required: SUPI, serving network name

Input, Optional: None

Output, Required: None

Output, Optional: None

**** End of Changes****