# 3GPP TSG-SA3 Meeting #112 	S3‑234345

# Goteborg, Sweden, 14 - 18 August 2023	(revision of S3-233718)

# Source:	Huawei, HiSilicon, Xiaomi

# Title:	Authorization for the SL Positioning Client UE

# Document for:	Approval

Agenda Item:	4.26

1	Decision/action requested

Approve the changes in TS 33.533.

2	References

[1]	TR 33.893

3	Rationale

Based on the conclusion on Key Issue #2 in TR 33.893, the procedure for authorization of UE for Ranging/SL positioning service exposure is added.

4	Detailed proposal

************ START OF CHANGES ************

6.3.X	Procedure for authorization of UE for Ranging/SL positioning service exposure

6.3.x.1	General

According to TS 23.586 [2] clause 5.6.2, Ranging/SL Positioning service can be exposed to the SL Positioning Client UE through PC5 or through 5GC control plane. The SL Positioning Client UE shall be authorized for Ranging/SL Positioning service exposure.

6.3.x.1	Authorization procedure for Ranging/SL positioning service exposure through 5GC control plane

For Ranging/SL Positioning service exposure through the network via control plane (i.e. clause 6.7.1.2.3 of TS 23.586 [2]), the MO-LR authorization as specified in TS 23.273 [3] is reused for the authorization of the SL positioning Client UE.

If the Client UE is not authorized, the Ranging/SL Positioning service request shall be rejected.

6.3.x.2	Authorization procedure for Ranging/SL positioning service exposure through PC5

For Ranging/SL Positioning service exposure through PC5 (i.e. clause 6.7.1.1 of TS 23.586 [2]), the SL Positioning Client UE authorization is triggered by the Reference/Target UE during PC5 link establishment. The authorization can be performed by the network via the SLPKMF for ProSe capable UEs or by the Reference/Target UE if the authorization information is available in the UE.

If the Client UE is not authorized, the Ranging/SL Positioning service request shall be rejected.

************ END OF CHANGES************