3GPP TSG-SA3 Meeting #112 	S3-233541

# Goteborg, Sweden, 14 - 18 August 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.512 | CR | 0038 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | AMF redirection to EPS remove CIoT precondition |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SCAS_5G_Ph2 |  |  |  |  |  | Date: |  |  | 2023-07-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | It is not needed the AMF supports CIoT, could support, but it is not mandatory for this behaviour. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the precondition, added the threat reference and other minor changes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The pre-conditions to execute the test case are wrong |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.2.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Change related with Tdoc S3-233542 in threats documennt |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

******** START OF CHANGE *********

##### 4.2.2.1.3	NAS based redirection from 5GS to EPS

Requirement Name: NAS based redirection from 5GS to EPS

Requirement Reference: TS 33.501 [2], clause 6.16.4, TS 23.501 [8], clause 5.31.3.

Requirement Description: As specified in TS 33.501 [2], clause 6.16.4, when a UE initiates registration procedure with the AMF, the AMF may redirect the UE from 5GC to EPC by including a EMM cause indicating to the UE that it shall not use 5GC, as described in clause 5.31.3 in TS 23.501 [2]. The following requirements apply to Registration Reject message with an EMM cause which indicates to the UE that the UE shall not use 5GC:

-	the AMF only sends such a Registration Reject message once NAS security has been established between the AMF and the UE; and

-	the UE only acts upon such Registration Reject message if received integrity protected and if UE has verified the integrity of the Registration Reject message successfully.

NOTE 1:	Void

In addition, in networks that support CIoT features in both EPC and 5GC, the operator may steer UEs from a specific CN type due to operator policy, e.g. due to roaming agreements, Preferred and Supported Network Behaviour, load redistribution, etc. Operator policies in EPC and 5GC are assumed to avoid steering UEs back and forth between EPC and 5GC.

Threat Reference:  TR 33.926 [6], clause K.2.8, NAS based redirection from 5GS to EPS

Test Name: TC_AMF_REDIRECTION_5GS_EPS

Purpose:

Verify that AMF under test does not send a Registration Reject message containing an EMM cause indicating to the UE that the UE shall not use 5GC, if NAS security is not established.

NOTE 2: Void

Pre-Conditions:

-	Test environment with UE. The UE may be simulated.

-	AMF under test is connected in emulated/real network environment.

-	Tester configures the operator policy of the AMF that all the UEs sending initial registration request should be redirected from 5GS to EPS.

Execution Steps

1. 	The tester triggers the UE to initiate an initial registration procedure with the AMF.

2. The AMF under test determines that the UE shall not use 5GC and needs to redirect the UE from 5GC to EPC.

3. The AMF under test sends a Registration Reject message with a 5GMM cause indicating to the UE that the UE shall not use 5GC.

Expected Results:

The NAS SMC is performed before sending the Registration Reject message.

Expected format of evidence:

Evidence suitable for the interface, e.g., Screenshot, packet captures or application log files containing the operational results.

******** END OF CHANGE *********