3GPP TSG-SA3 Meeting #112 	S3-233542

# Goteborg, Sweden, 14 - 18 August 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.926 | CR | 0075 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NAS based redirection from 5GS to EPS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SCAS_5G_Ph2 |  |  |  |  |  | Date: |  |  | 2023-07-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The threat not only apply to CIoT |  |  |  |  |  |  |  |  |
| Summary of change: |  | CIoT mention needs to be remove/updated |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The threat could be incomplete, because it affects to any UE |  |  |  |  |  |  |  |  |
| Clauses affected: |  | K.2.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Change related with Tdoc S3-233541 in SCAS AMF |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

********** START OF CHANGE ***********

## K.2.8	NAS based redirection from 5GS to EPS

-	Threat name: NAS based redirection from 5GS to EPS

-	Threat Category: Denial of Service, Information disclosure.

-	Threat Description: In NAS based redirection from 5GS to EPS in 5G, when a UE initiates registration procedure with the AMF, the AMF may redirect the UE from 5GC to EPC with a Registration Reject message sent to the UE, and if the Registration Reject message with an EMM cause which indicates to the UE that the UE shall not use 5GC is not protected, the attacker can modify the cause and the UE will try to connect to the EPS. This will lead to a bidding down attack to the UE.

-	Threatened Asset: Sufficient Processing Capability, N1 interface, Mobility Management data.

********** END OF CHANGE ***********