3GPP TSG-SA3 Meeting #112 	S3-233806

# Goteborg, Sweden, 14 - 18 August 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | 1734 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Home control for Network Slice Admission Control (NSAC) procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Nokia, Nokia Shanghai Bell, LGE, Xiaomi, ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | Dummy |  |  |  |  |  | Date: |  |  | 2023-08-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | It was concluded in the eNS3 study (TR33.886) that a fraudulent local NSACF in specific area(s) of a PLMN with low security protection may send fake messages to the primary NSACF (e.g. at HPLMN) indicating that the total number of registered UEs/PDU sessions has reached the quota. This may cause the primary NSACF to re-distribute the quotas and impact services of other areas. This CR is to provide a means to prevent this fraud. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new subclause in clause 16 to protect NSAC procedures for the hierarchical NSAC architecture. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The NSAC procedure is not secured. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

************************** Start of changes ************************

## 16.X	Increase home control for Network Slice Admission Control (NSAC) procedures

### 16.x.1	Introduction

The NSACF procedures are specified in clause 4.2.11 of TS23.502 [8] for an S-NSSAI subject to Network Slice Admission Control (NSAC). In case of a hierarchical NSAC architecture (i.e. the Option 3) is deployed as described in clause 4.2.11.2a and clause 4.2.11.4a of TS23.502 [8], the feature of increased home control is to prevent certain types of fraud, e.g. the fraudulent Nnsacf_NSAC_NumOfUEsUpdate/Nnsacf_NSAC_NumOfPDUsUpdate NSAC request from an NSACF to the primary NSACF.

The Primary NSACF should verify the number of UEs per network slice and the number of PDU sessions per network slice before performing NSAC for the network slice as follows.

### 16.x.2	Home control procedure of Number of UEs per network slice for Hierarchical NSACF

This procedure follows the clause 4.2.11.2a of TS23.502 [8] with the following additions:

-	Before the Primary NSACF performs NSAC for the indicated S-NSSAI at step 6, the Primary NSACF shall verify the validity of the information carried in the update request.

### 16.x.3	Home control procedure of Number of PDU Sessions per network slice availability check and update procedure

This procedure follows the clause 4.2.11.4a of TS23.502 [8] with the following additions:

-	Before the Primary NSACF determines whether to accept or reject the request to update local maximum PDU session number from NSACF at step 6, the Primary NSACF shall verify the validity of the information carried in the update request.

Editor's note:	The information required for the verification by the Primary NSACF is to be specified.

******************************* End of changes *********************************