3GPP TSG-SA3 Meeting #112 	draft_S3-234210-r2 was S3-234035

# Goteborg, Sweden, 14 - 18 August 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.256 | CR | draftCR | rev |  | Current version: | V17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to A2X Direct C2 Communication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | UAS_Ph2 |  |  |  |  |  | Date: |  |  | 2023-08-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The A2X Direct C2 Communication in Clause 5.Y.1, has the following EN: Editor's note: Enhancements to the UUAA (5.2) and/or Pairing (5.4) procedures related to unicast security establishment for Direct C2 are FFS. Further, Clause 5.Y.2 for Unicast mode direct C2 communication cites clause 5.X.2 for the procedures, wherein clause 5.X.2.3 Security procedure cites 33.536. The actual baseline procedure in TS 33.536 can be reused for A2X, but the aspects used in the procedure should be specific to A2X service rather than V2X service similar to TS 23.256 Clause 5.4.4 Procedure for Direct C2 Communication establishment, which has the following NOTE. NOTE: Security aspects of Direct C2 Communication is defined by SA WG3. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Resolves the EN in Clause 5.Y.1 and clarifies the A2X service aspects for considering aspects in TS 33.536 as baseline. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The A2X security procedure will not be complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.X.2, 5.Y.1 and 5.Y.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | n | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | n | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | n | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** Start of Change 1 ****

## 5.Y	A2X Direct C2 Communication

### 5.Y.2	Unicast mode Direct C2 Communication

Unicast mode Direct C2 Communication has the same requirements and procedures as unicast mode A2X Direct Communication (see clause 5.X.2 of the present specification) with the following exception:

-	The A2X Policy Provisioning is done based on TS 23.256 Clause 4.2.1.2.2 and the A2X Policy includes A2X security policy for each A2X services (e.g., C2 and DAA). The C2 service specific security policy available as part of A2X security policy is used for the security establishment (i.e. included in the direct communication request and is further replayed in the Direct security mode command to provide protection against bidding down attacks as described in TS 33.536 [7]), where the signalling and user plane confidentiality and integrity are set as required based on local policy.

**** End of Change 1 ****