3GPP TSG-SA3 Meeting #117	S3-243313

Maastricht, Netherlands  19 - 23 August 2024

# Source:	CableLabs

# Title:	Addressing ENs in solution #12 in TR 33.700-32

# Document for:	Approval

Agenda Item:	5.10

# 1	Decision/action requested

It is requested to approve the new solution in this pCR.

# 2	References

[X]	3GPP TR 23.700-32 " Study on User Identities and Authentication Architecture"

[Y]	3GPP TR 33.700-32 " Study on security aspects of User Identities and Authentication"

# 3	Rationale

This is to address ENs in solution 12.

EN1 is deleted with the difference from clause 7B.3 added.

EN2 is deleted based on the evalution that this solution is not consistent with SA2 architecture assumption.

EN3 is converted to a NOTE explaining why the SUCI of non-3GPP device is used in N2 message.

# 4	Detailed proposal

SA3 is kindly requested to agree to the below pCR to TR 33.700-32 [Y].

**** START OF CHANGE ****

## 6.12	Solution #12: authorization of non-3GPP devices behind 5G-RG

    - Introduction

This solution addresses Key Issue #3 on the authorization of non-3GPP devices behind 5G-RG. It is based on the authentication of FN-RG in clause 7B.3 of TS 33.501 [3], with following changes: a) it is the 5G-RG, not W-AGF, which registers non-3GPP devices to the 5GC; and b) it adds authorization check that the non-3GPP device is under the control of an RG which has been successfully authenticated by 5GC. This ensures that an RG can only represent a non-3GPP device allowed by the RG subscription.

### 6.12.2	Solution details

Figure 6.12.2-1. authorization of non-3GPP devices behind 5G-RG

1. A layer-2 (L2) connection is established between the non-3GPP device and the 5G-RG using local authentication (e.g., WPA personal).

2. The 5G-RG sends an AAA message to the W-AGF to indicate that a device with a non-3GPP device identifier has been successfully authenticated locally.

NOTE 1 : The identifier of the non-3GPP device is defined by BBF or CableLabs and is out of scope of 3GPP.

3. The W-AGF sends back a confirmation AAA message to the RG.

NOTE 2 : The AAA messages used between the 5G-RG and the W-AGF in steps 2-3 are defined by BBF or CableLabs and out of scope of 3GPP.

4. The W-AGF shall perform initial registration on behalf of the non-3GPP device. The W-AGF shall generate a Registration Request message and send it to the AMF over N2. The Registration Request message contains the SUCI of the non-3GPP device and the SUCI of the 5G-RG. The N2 message contains an indication that the RG has authenticated the non-3GPP device.

NOTE1: the SUCI of non-3GPP device is included in the N2 message to be consistent with clause 7B.3 of TS 33.501 [3] where the SUCI of FN-RG is included in the N2 message.

5. The AMF shall select an AUSF based on the received SUCI. The AMF shall send a Nausf_UEAuthentication_Authenticate Request message to the AUSF. It contains the SUCI of the non-3GPP device, the SUCI of the 5G-RG, and the SN-name. It also contains the authenticated indication generated by the W-AGF.

6. The AUSF shall send a Nudm_UEAuthentication_Get Request to the UDM. It contains the SUCI of the non-3GPP device, the SUCI of the 5G-RG, the SN-name, and the authenticated indication.

7. The UDM shall invoke the SIDF and maps the SUCIs to the SUPIs. The UDM shall verify that the 5G-RG has been successfully authenticated and the non-3GPP device is under the control of the RG based on the subscription profiles of the 5G-RG.  The UDM decides the authentication by the home network is not required for the non-3GPP device and the non-3GPP device has been authorized.

8. The UDM shall send a Nudm_UEAuthentication_Get Response to the AUSF. It contains the SUPI of the non-3GPP device and an indication that authentication by the home network is not required.

9. After checking the indication set by the UDM, The AUSF shall not perform authentication and shall send a Nausf_UEAuthentication_Authenticate Response to the AMF. It contains the SUPI of the non-3GPP device and the indication that authentication by the home network is not required set by the UDM.

This response from AUSF indicates that authentication is not required, and no KSEAF is included.

10. After checking the indication to make sure that the authentication by the home network is not required, the AMF shall estabilish the NAS security for the non-3GPP device between AMF and W-AGF with NULL encryption and NULL integrity protection.

11. The AMF shall send Registration Accept message to the W-AGF. This message contains 5G-GUTI and other parameters.

12. The W-AGF shall send a Registration Complete message back to the AMF. The W-AGF shall store the 5G-GUTI for use in later NAS procedures.

13. The W-AGF and the 5G-RG may establish a PDU session for the non-3GPP device. This is defined by CableLabs and BBF and is out the scope of 3GPP.

### 6.12.3	Evaluation

This solution is not consistent with the SA2 architecture assumption that non-3GPP devices is not registered to the 5GC.

**** END OF CHANGE ****