3GPP TSG-SA3 Meeting #117	S3-243488

Maastricht, Netherlands 19 - 23 August 2024

# Source:	Huawei, HiSilicon

# Title:	Update to Sol#1

# Document for:	Approval

Agenda Item:	5.4

# 1	Decision/action requested

Approve to include pCR proposal for the TR 33.776

# 2	References

[1]	3GPP TR 33.776 	Study of Automatic Certificate Management Environment (ACME) for the Service Based Architecture (SBA)

# 3	Rationale

The contribution updates the solution #1 to include initial trust eatablishment and update evaluation in TR 33.776 [1].

# 4	Detailed proposal

pCR

***  BEGINNING OF CHANGES ***

## 6.1	Solution #1:	Using NF FQDN as ACME identifier

### 6.1.1	Introduction

This solution addresses the key issue #3.

The ACME protocol defined in the RFC 8555 [2] was designed to help a web server to get a domain name certificate from a CA automatically. However, in the current operator networks, an NF instance ID certificate is preferred since the NF instance ID is used to uniquely identify an NF. In this solution, the NF FQDN is linked to the NF instance ID so that the ACME protocol with domain name can be re-used for NF certificate management.

### 6.1.2	Solution Details

In 5G SBA, an NF is uniquely identified by an NF instance ID. The NF profile can also contain a FQDN.  In order to link the NF instance ID with its FQDN and re-use the ACME protocol based on a domain name, it is proposed to use an NF instance ID to form part of the NF’s domain name, e.g. NF_instance_ID. NF_types.operators_name.3gpp.org, etc. The format of NF_instance_ID is as defined in TS 29.571 [13].

For example, according to TS 23.003 [17], an FQDN for an NF, e.g. AMF, can be constructed as:

-	AMF: <AMF-id>.amf.5gc.mnc<MNC>.mcc<MCC>.3gppnetwork.org

Similarly, the FQDN of an NF in this solution with NF instance ID can be constructed the same way as:

-	< NF_instance_ID >. <NFType>.5gc.mnc<MNC>.mcc<MCC>.3gpp.org. e.g. for an AMF, <AMF’s NF_instance_ID>.amf.5gc.mnc123.mcc456.3gpp.org.

Through the challenge and response procedure of the ACME protocol RFC 8555 [2], the ACME client is able to prove to the ACME Server that it is authoritative for its FQDN.

### 6.1.2.1	Procedure

When the http-01 challenge type is used, it is required that the ACME client is authorized to control the /.well-known/acme-challenge/ directory on the http server corresponding to the domain name.

Figure 6.1.2.1 shows a high-level procedure for NF to obtain certificates from CA with ACME procedures (for simplicity, it is assumed that an NF takes the role of the ACME client). The procedure is as follows:

1. 	After the NF is deployed, it starts the ACME client and performs following the steps for certificate issuing based on RFC 8555 [2].

2. 	The ACME client on the NF chooses a CA as configured by the OAM and creates an ACME account as in RFC 8555 [2].

3. 	The ACME client creates a certificate order on the CA. To confirm that the ACME client is authorized to delegate the identifiers, the ACME server at the CA side generates challenges for the ACME client to complete.

4. 	The ACME client downloads the challenge from the ACME server, choose one of the challenge types, e.g. http-01 and completes the challenge according to RFC 8555 [2].

5. 	After the ACME client completes the challenge successfully, the ACME client is authorized to request and receive a certificate for its FQDN. To receive the certificate, the ACME client needs to send a Certificate Signing Request (CSR) to the ACME server.

6. 	After receiving the CSR, CA issues the certificates and puts it under the relevant directory on the ACME server. The FQDN in the certificate contains the NF_instance_ID.

7. 	The ACME client downloads the certificate from the ACME server.

ACME Server/CA

NF

2. ACME client creates an account

3. ACME client creates a certificate order for the domain name

1. NF starts the ACME client

Standard procedures from RFC 8555

4. ACME client completes the authorizations (http-01/dns-01) of the order

5.  ACME client sends the CSR to the server

6.  CA issues the certificate

7.  ACME client download the certificate from server

Figure 6.1.2.2: ACME procedure for NF certificate management

It is worthwhile mentioning that the solution can be extended to support the dns-01 challenge type as well. This solution with the dns-01 challenge is feasible only if the DNS resource record can be updated by the ACME client and that DNS server should be accessible (i.e., record can be retrieved) by the ACME server. In order for an NF to modify its DNS records, the NF needs to be granted such privileges.

### 6.1.3	Evaluations

This solution addresses the key issue #3.

The solution is limited to NF producers if it assumes control over HTTP resources for the http-01 challenge. The solution can be extended to support the dns-01 challenge if the DNS resource record can be updated by the NF or OAM.

This solution with the dns-01 challenge is feasible only if the DNS resource record can be updated by the ACME client and that DNS server should be accessible (i.e., record can be retrieved) by the ACME server. In order for an NF to modify its DNS records, the NF needs to be granted such privileges.

In order to not impact ACME, the solution requires changes to the current SBA certificate profiles so that an FQDN formed based on the NF instance ID can be used as an identifier value for the challenge. Observe that the standard impact is not only limited to the profile since there are also requirements for NF instance ID checks based on what is included in the certificate for example in TS 33.501 [8].

***   END OF CHANGES   ***