3GPP TSG-SA3 Meeting #120	S3-250680

# Athens, Greece, 17 - 21 February 2025

Source:	Huawei, HiSilicon

Title:	Add Security flow of Multi-hop UE-to-Network Relay security establishment procedure to ProSe draftCR

Document for:	Approval

Agenda item:	4.15

Spec:	3GPP TS 33.503

Version:	18.5.0

Work Item:	5G_ProSe_Sec_Ph3

Comments

Accept the following changes to add security flow of multi-hop UE-to-Network security establishment procedures to the ProSe Ph3 draft CR.

* * * First Change * * * *

6.3.Z.2	Security procedure for 5G ProSe Multi-hop UE-to-Network Relay communication

The security procedure for 5G ProSe Multi-hop UE-to-Network Relay communication includes the PC5 security establishment procedures between Remote UE and Intermediate UE-to-Network Relay, between Intermediate UE-to-Network Relays, and between Intermediate UE-to-Network Relay and UE-to-Network Relay. For Multi-hop UE-to-Network Relay security establishment, both the UP-based procedure as specified in clause 6.3.3.2 and the CP-based procedures as specified in clauses 6.3.3.3 are supported.

The detailed procedures are listed as follows:

0.	The 5G ProSe Remote UE, Intermediate UE-to-Network Relay, and 5G ProSe UE-to-Network Relay are provisioned with the discovery security materials associated with an RSC based on the procedure specified in clause 6.1.3.2.

0a.	If the UEs are discovered using multi-hop UE-to-Network Relay discovery with model A procedure, the 5G ProSe Intermediate UE-to-Network Relay receives an Announcement message from 5G ProSe UE-to-Network Relay or the upstream 5G ProSe Intermediate UE-to-Network Relay. If the Intermediate UE-to-Network Relay does not have an existing PC5 link with the UE-to-Network Relay or the upstream intermediate UE-to-Network relay when it successfully processes the Announcement message, the Intermediate UE-to-Network Relay may establish a PC5 link with the 5G ProSe UE-to-Network Relay or the upstream intermediate UE-to-Network relay using the security procedures over User Plane or Control Plane as specified in clause 6.3.3.

Once the PC5 link is established between the upstream Intermediate UE-to-Network Relay or the 5G ProSe UE-to-Network Relay, the Intermediate UE-to-Network updates the Announcement message as specified in clause 6.3.2.5.2 of TS 23.304 [2], protects and sends the updated Announcement message.

1.	After the multi-hop Relay discovery procedure, the Remote UE initiate a Direct Communication Request (DCR) message to request the security establishment between the intermediate relay at the next hop (denoted as the Intermediate UE-to-Network Relay-A), including the RSC, CP/UP-PRUK ID or SUCI of the Remote UE as defined in clause 6.3.3.

2. 	Upon received the DCR message, the Intermediate UE-to-Network Relay-A checks whether or not has an existing PC5 link with the UE-to-Network Relay or the upstream Intermediate UE-to-Network Relay, and the upstream Intermediate UE-to-Network Relay is in-coverage.

NOTE 1: 	The upstream Intermediate UE-to-Network Relay is considered in-coverage if it has connection to the network.

If PC5 link is not established with the UE-to-Network Relay or the upstream Intermediate UE-to-Network Relay, the Intermediate UE-to-Network Relay-A plays the role of the Remote UE to establish secured link with the UE-to-Network Relay or with the upstream Intermediate UE-to-Network Relay, reusing the CP/UP-based procedure as specified in the in clauses 6.3.3. The DCR in this step includes the RSC, CP/UP-PRUK ID or SUCI of the Intermediate UE-to-Network Relay-A as defined in clause 6.3.3.

Each of the Intermediate UE-to-Network Relay needs to establish secured PC5 link with the upstream node (Intermediate UE-to-Network Relay or the UE-to-Network Relay) before it can serve the Remote UE, or the Intermediate UE-to-Network Relay acting the role of the Remote UE. The Intermediate UE-to-Network Relay’s UDM/PKMF checks whether the Intermediate UE-to-Network Relay is authorised to offer multi-hop UE-to-Network relay service based on the RSC.

3. 	The Intermediate UE-to-Network Relay-A uses the protected PC5 link (i.e. established in step 0a or step 2) to send the parameters in the DCR in step 1 to the network. Based on the steps 4a-4e of 6.3.3.2.2 or steps 3-13 of 6.3.3.3.2), the Intermediate UE-to-Network Relay-A interacts with the network, in order to get the KNR_ProSe/KNRP and freshness parameter to set up connection with the Remote UE.

NOTE 2: 	It is assumed that the Intermediate UE-to-Network Relay can use the dedicated DNN for UE-to-Network Relay to communicate with its HPLMN’s PKMF.

If Layer-3 connection is setup using CP-based mechanism, the Intermediate UE-to-Network Relay-A sends/receives the Intermediate key request/response to the UE-to-Network Relay. All the NAS connection with the network will be sent to the UE-to-Network Relay’s AMF by the UE-to-Network Relay, instead of sending to the Intermediate UE-to-Network Relay’s AMF by the Intermediate UE-to-Network Relay.

4.	The Intermediate UE-to-Network Relay-A uses the KNR_ProSe/KNRP and freshness parameter to establish secure PC5 link with the Remote UE.

* * * End of Changes * * * *