3GPP TSG-SA3 Meeting #120	S3-250683

# Athens, Greece, 17 - 21 February 2025

Source:	Huawei, HiSilicon

Title:	Add Security flow of L3 Multi-hop UE-to-UE Relay security establishment procedure to ProSe draftCR

Document for:	Approval

Agenda item:	4.15

Spec:	3GPP TS 33.503

Version:	18.5.0

Work Item:	5G_ProSe_Sec_Ph3

Comments

Accept the following changes to add security flow of Layer-3 multi-hop UE-to-UE security establishment procedures to the ProSe Ph3 draft CR.

* * * First Change * * * *

6.6.XX.2	Security procedure for 5G ProSe Layer-3 Multi-hop UE-to-UE Relay communication

The security procedures for 5G ProSe Layer-3 Multi-hop UE-to-UE Relay covers the procedures of security establishment of both IP PDU type and non-IP PDU type.

* * * Second Change * * * *

6.6.XX.2.1	Security procedure for security establishment of 5G ProSe Layer-3 Multi-hop UE-to-UE Relay of IP PDU type

The security procedure in clause 6.2 is used to establish a secure PC5 link between the End UE and the UE-to-UE Relay, and between UE-to-UE Relays with the following modifications.

-	The RSC is included in the Direct Communication Request (DCR) message.

-	The DCR message is protected reusing the security mechanism defined in clause 6.3.5 with a modification that the length of the UP-PRUK ID/CP-PRUK ID is set to zero in clause 6.3.5.2.

* * * Third Change * * * *

6.6.XX.2.2	Security procedure for security establishment of 5G ProSe Layer-3 Multi-hop UE-to-UE Relay Discovery of non-IP PDU type

If 5G ProSe UE-to-UE Relays are out of coverage of the network, the security procedure specified in clause 6.2 is reused to establish a secure PC5 link for each hop among 5G ProSe End UEs and 5G ProSe UE-to-UE Relays, with the following modifications:

-	The RSC is included in the Direct Communication Request (DCR) message.

-	The DCR message is protected reusing the security mechanism defined in clause 6.3.5 with a modification that the length of the UP-PRUK ID/CP-PRUK ID is set to zero in clause 6.3.5.2.

Both UP-based and CP-based security procedures specified in clauses 6.3.3.2 and 6.3.3.3 are reused to establish secure PC5 link establishment for each hop among 5G ProSe End UEs and 5G ProSe UE-to-UE Relays if 5G ProSe UE-to-UE Relays are in-coverage.

* * * End of Changes * * * *