3GPP TSG-SA3 Meeting #120	S3-250858

# Athens, Greece, 17 - 21 February 2025

Source:	Xiaomi

Title:	Pseudo-CR on TR 33.700-22 KI#1.2 conclusion update

Document for:	Approval

Agenda item:	5.19

Spec:	3GPP TR 33.700-22

Version:	0.5.0

Work Item:	FS_CAPIF_Ph3-sec

Comments

This contribution proposes to update TR 33.700-22 KI#1.2 conclusion.

The following Editor’s Note is related to the second security requirement of KI#1.2.

Editor’s Note: Conclusion for authorization of the resource owner to provide resource owner authorization is FFS.

Since the resource owner can only provide authorization information/authorization revocation information related to himself/herself, the following part is added to address the Editor’s Note.

The CCF/authorization function accepts resource owner authorization and authorization revocation information only if the authenticated resource owner ID is identical to the resource own ID in the authorization information/authorization revocation information.

* * * First Change * * * *

7.1.2	Conclusions for KI#1.2 Resource owner authorization management

The following statements are agreed as a basis for normative work.:

7.1.2.1	Authentication and authorization of the end points and security of transferred authorization data

Authorization information/authorization revocation information is transferred between the ROF and the CCF via secure CAPIF-8 reference point.

The resource owner is authenticated before accepting resource owner authorization and authorization revocation information. How to authenticate the resource owner is left to implementation.

The CCF/authorization function accepts resource owner authorization and authorization revocation information only if the authenticated resource owner ID is identical to the resource own ID in the authorization information/authorization revocation information.

NOTE: Authentication between the ROF and the CCF is addressed in KI#1.1.

7.1.2.2	Resource owner authorization data

Authorization information includes Resource Owner Identifier. Other information to be obtained and stored for the resource owner authorization is to be determined in the normative work.

7.1.2.3	Revocation

The existing mechanisms for the revocation in TS 33.122 are reused. What revocation information is sent by the ROF for the CCF to identify the revoked RNAA-related token is to be determined in the normative work.

* * * End of Change * * * *