3GPP TSG-SA3 Meeting #120	S3-250932

Athens, Greece, 17 - 21 February 2025

Source: 	Sateliot, Novamint

Title: 	DP on handling NAS counters in S&F Mode when the UE registration is valid in multiple satellites

Document for: 	Discussion/Approval

Agenda Item: 	4.19

Work Item / Release:	5GSAT_Ph3_SEC

Abstract of the contribution: This paper discusses an issue with the use of the NAS counters in S&F Satellite operation when UE registration is expected to be valid in multiple satellites, as is the case of split-MME architectures, and outlines a couple of solution approaches to handle it. This contribution is primarily intended to trigger discussion on this topic and agree on a possible way forward within CT WG1 and/or SA WG3 to address it.

# 1	Discussion

Support for Store and Forward Satellite operation is being added to Rel-19 specs (TS 23.401 v19.1.0 §4.3.9). When a satellite supports Store and Forward Satellite operation and is operating in S&F Mode, the end-to-end exchange of signalling/data traffic is handled in a sequence of steps reflecting the intermittent availability of the service link when the satellite can exchange data with the UE and of the intermittent availability of the feeder link when the satellite can exchange data with the core network on the ground. To support Store and Forward Satellite operation, the eNB together with some network functionality needs to be deployed on the satellite payload.

One deployment option considered within SA2 to support S&F satellite operation is the split-MME architecture documented in Annex O.2 in 3GPP TS 23.401 v19.1.0. Under such split-MME architecture (see Figure 1), the MME entity is split into two functions:

- MME-onboard: the MME part which is onboard the satellite. MME-onboard is in charge of (1) handling the S1 interface with the onboard eNB and (2) handling the NAS protocol signalling from/to UEs via the onboard eNB.

- MME-ground: the MME part which is on the ground network. MME-ground is in charge of handling the rest of interfaces towards other CN functions (e.g., S6a towards HSS, SGd towards SMS-GMSC/IWMSC /SMS Router, T6a towards SCEF, T6ai towards IWF-SCEF, S11 towards SGW).

which adhere to the following principles:

- An MME-ground is associated with at least one MME-onboard. An MME-onboard is associated with a Satellite ID identifier. The MME-ground together with the associated MME-onboard(s) behave jointly as a single MME entity.

- How MME-onboard(s) interacts with MME-ground and how synchronization of the UE context between them is done is outside the scope of 3GPP.

NOTE:	Based on implementation, the UE context can be uploaded on a subset of MME-onboard(s) or all MME-onboard(s) associated with MME-ground.

Figure 1: "Split-MME" architecture for supporting S&F satellite operation for SMS and CP CIoT services (Source: Figure O.2-1 in [1])

As it can be observed in Figure 1, the split-MME architecture allows use cases where multiple satellites can be involved in the S&F operation. Indeed, as stated in a LS reply to SA3 [2], S&F operation involving multiple satellites of the same PLMN to serve a given UE is considered to be the general case, while the use of a single serving satellite per UE it’s just a particular case. Enabling a UE to be served by multiple satellites of the network is key to reduce the revisit time and so improve service performance [3].

Observation 1: S&F Satellite operation involving multiple satellites of the same PLMN is considered to be the general case, while the use of a single serving satellite per UE it’s just a particular case. Enabling a UE to be served by multiple satellites of the network is key to reduce the revisit time and improve service performance [3].

To handle S&F Satellite operation with multiple satellites under the split-MME architecture, the following assumptions apply:

- It is assumed that NAS ciphering/deciphering and integrity protection functions are handled in the MME-onboard when only service link is available (see [2]). In this way, during a satellite contact window, the UE and the MME-onboard can execute the successive steps of different NAS procedures (e.g., Attach, TAU, CPSR, etc.) as in normal mode, whenever all the necessary information is available within the satellite (e.g., authentication vectors, subscription data, etc.). Accordingly, the UE context in the MME-onboard is expected to change during the course of interaction with the UE (e.g., at least NAS Counters of the security context will have increased).

- Once a satellite contact window is over, the NW needs to propagate the changes occurred in the UE context of the MME-onboard towards the MME-onboard(s) hosted in the other satellites that may interact with the UE in a subsequent satellite contact window.  In this way, when the UE accesses the network via a next serving satellite, the MME-onboard residing in that satellite, is able to smoothly resume the interaction with the UE (i.e., the change of MME-onboard remains unnoticed to the UE) without implying the detach and consequent re-attach of the UE.

- The propagation and synchronisation of the UE context across the MME-onboard(s) and associated MME-ground has been left out of the scope of 3GPP specification in Rel-19 and based on implementation. The exchange of the UE context information will take place over the feeder link connectivity, whose capacity and connectivity pattern (i.e., scheduling of feeder link passes between the satellites and the ground stations) should be properly arranged based on network operator policy.

Observation 2: For split-MME architecture, involving multiple satellites means that the NW needs to cope with UE context synchronisation between the multiple MME-onboard(s) and the associated MME-ground so that the change of serving MME-onboard across subsequent passes remains unnoticed to the UE. How UE context synchronisation is performed has been left out of 3GPP specs.

While multi-satellite S&F Satellite operation benefits from a higher number of satellites (i.e. the UE will be able to interact with the network more often), the need to propagate and synchronise the UE context after each UE-satellite contact across other satellites that can serve the UE in a next pass can become challenging in terms of feeder link capacity dimensioning and operation. To cope with this issue, the specification provides two tools that are instrumental for the NW to be able to prevent access from a UE until the UE context is properly synchronised in the serving satellites:

- S&F Wait Timer. If a UE indicates Store and Forward capability, the MME may provide the UE with a S&F Wait Timer when accepting or rejecting a NAS procedure. When the S&F Wait Timer expires, the UE may perform a NAS procedure, which can be a subsequent NAS procedure or a reattempt of a NAS procedure previously rejected with a S&F reject cause. On the network side, the Wait Timer can be adjusted so that the NW has enough time to complete the UE context synchronisation before that the UE retries access.

- S&F Monitoring List. This is a list of satellites (identified by satellite ID(s), see Figure 1) for the same PLMN. The Satellite ID is transmitted over the SIB information broadcast by eNB. The list indicates the UE the satellites in which interactions are allowed (i.e. satellites in which the UE context is going to be handled/available). This list allows the NW to restrict the operation of a UE to a subset of satellites of the PLMN (e.g., a satellite operator with a network of M=100 satellites operating in S&F mode may decide to serve a particular UE using only N=10 of those satellites).

The use of above two tools is key to cope with the issue of UE context synchronisation, especially when the changes in the UE context are significant (e.g., a UE has completed an attach/registration in a given MME-onboard and the new resulting UE context needs to be propagated entirely to the other satellites of the “List of Satellite IDs”).

Observation 3: For split-MME architecture, the “S&F Wait Timer” and “S&F Monitoring List” are two fundamental tools to cope with the issue of UE context synchronisation on the NW side, especially when the changes introduced in the UE context during a satellite pass are related to EMM/ESM status changes, establishment of new identifiers and/or establishment of new security keys.

However, keeping aside the NAS procedures related to the management of EMM / ESM state changes which can be handled by previously two mentioned tools, a common case for IoT devices is expected to be the case where:

An IoT device attaches to the network and registration state is expected to be kept for a long period of time (e.g., days or longer).

While registered, an IoT device can perform Mobile Originated (MO) or Mobile Terminated (MT) data transfers as long as satellite passes becomes available, without needing to re-attach or re-establish security context in each pass.

In such a situation, only performing MO and MT data transfers during a satellite pass means that the only changes to occur in the UE context are the increase of the NAS Counters.

Observation 4: However, a common case with IoT devices is expected to be that where the IoT device, once properly registered into the network, it’s only going to use forthcoming satellite pass to send or receive data without needing to re-attach or update the UE context parameters related to EMM/ESM states. In such a case, the only changes that will occur in the UE context are the increase of the NAS Counters.

For such cases, having a solution that prevents the network from having to synchronise the UE context only because of the change of NAS Counters can importantly contribute to improve the service (e.g., increase of revisit time) and reduce the signalling overhead in the feeder link (e.g., less UE context updates between MME-onboards).

Proposal#1. CT1 and/or SA3 to consider specifying a solution to avoid UE context synchronisation across the set of satellites in S&F Mode serving a given UE (i.e., the set of satellites in which a valid UE context is maintained and kept synchronised between the satellites), when the only difference between the UE contexts held in each of the satellites is the value of the NAS Counters.

Two possible solution approaches to Proposal#1 are outlined in the following. The main intention is to illustrate the sort of solution approaches that in our view could be feasible to address, while not advocating for any specific solution nor disregarding other approaches.

Solution approach#1: EPS security context in the UE includes separate pairs of NAS counters per SatelliteID.

This solution approach considers that separate pairs of NAS counters for each of the satellites in which the registration is valid can be included in the EPS security context in the UE. Such an approach could be based on the following principles:

In S&F Satellite operation, the subset of satellites operating in S&F Mode in which a given UE registration is valid are expected to maintain a synchronised UE context (i.e., UE context consistency across satellites is assumed: however, the synchronisation mechanism is outside the scope of 3GPP). The only exception to this synchronization is the NAS counters, which may or may not be synchronised between the satellites.

When a UE registration is valid in multiple satellites operating in S&F mode, the UE includes separate pair of NAS Counters to interact with each one of the satellites in which the registration is valid. Each pair of NAS Counters is associated with a SatelliteID.

Solution approach#2: New specific rules to handle NAS Counter Overflow in S&F mode.

Another potential solution approach is to maintain a single pair of NAS Counters while introducing specific rules for managing the NAS counters in S&F Satellite operation. This approach could be based on the following principles:

In S&F Satellite operation, the subset of satellites operating in S&F Mode in which a given UE registration is valid, are expected to maintain a synchronised UE context (i.e., UE context consistency across satellites is assumed: however, the synchronisation mechanism is outside the scope of 3GPP). The only exception to this synchronization is the NAS counters, which may or may not be synchronised between the satellites.

When a UE registration is valid in multiple satellites operating in S&F mode, each time the UE interacts with one of these satellites, the UL NAS Overflow Counter (OC) stored in the UE may be higher than the UL NAS OC stored in the serving satellite. This discrepancy can arise due to previous interactions between the UE and other serving satellites where the UL NAS OC in the UE was incremented when the UL NAS SQN wrapped around. In such case, if the MME fails to verify the MAC integrity of a received NAS packet using the last stored UL NAS OC, the MME may attempt to validate integrity using a series of consecutively incremented UL NAS OC values. If successful, the MME updates its stored UL NAS OC accordingly.

Similarly, when a UE registration is valid in multiple satellites operating in S&F mode, the DL NAS Overflow Counter (OC) stored in the UE may be higher than the last DL NAS OC stored in the serving satellite. This can result from prior interactions between the UE and other satellites where the DL NAS OC in the UE was increased when the DL NAS SQN wrapped around. In such cases, if the UE fails to verify the MAC integrity of a received NAS packet using the last stored DL NAS OC, it may attempt integrity validation using a series of consecutively decremented DL NAS OC values.

# 2	Observations and Proposals

Observation 1: S&F Satellite operation involving multiple satellites of the same PLMN is considered to be the general case, while the use of a single serving satellite per UE it’s just a particular case. Enabling a UE to be served by multiple satellites of the network is key to reduce the revisit time and improve service performance [3].

Observation 2: For split-MME architecture, involving multiple satellites means that the NW needs to cope with UE context synchronisation between the multiple MME-onboard(s) and the associated MME-ground so that the change of serving MME-onboard across subsequent passes remains unnoticed to the UE. How UE context synchronisation is performed has been left out of 3GPP specs.

Observation 3: For split-MME architecture, the “S&F Wait Timer” and “S&F Monitoring List” are two fundamental tools to cope with the issue of UE context synchronisation on the NW side, especially when the changes introduced in the UE context during a satellite pass are related to EMM/ESM status changes, establishment of new identifiers and/or establishment of new security keys.

Observation 4: However, a common case with IoT devices is expected to be that where the IoT device, once properly registered into the network, it’s only going to use forthcoming satellite pass to send or receive data without needing to re-attach or update the UE context parameters related to EMM/ESM states. In such a case, the only changes that will occur in the UE context are the increase of the NAS Counters.

Proposal#1. CT1 and/or SA3 to consider specifying a solution to avoid UE context synchronisation across the set of satellites in S&F Mode serving a given UE (i.e., the set of satellites in which a valid UE context is maintained and kept synchronised between the satellites), when the only difference between the UE contexts held in each of the satellites is the value of the NAS Counters.

Not excluding others, two potential solution approaches are:

Solution approach#1: EPS security context in the UE includes separate pairs of NAS counters per SatelliteID.

Solution approach#2: New specific rules to handle NAS Counter Overflow in S&F mode.

# References

- 3GPP TS 23.401 v 19.1.0, “General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access (Release 19), December 2024

- S2-2411250, Reply LS on FS_5GSAT_Ph3_ARCH conclusions, 3GPP TSG-WG SA2 Meeting #165, 14 - 18 October, 2024, Hyderabad, India

- R2-2410596, “Considerations on multi-satellite for S&F Satellite operation”, NOVAMINT, Sateliot, Thales, 3GPP TSG-RAN WG2 Meeting #128, Orlando, US, 18 - 22 November 2024

The specification of the S&F List of Satellite IDs and/or S&F Monitoring List is not yet concluded in SA2 and is being treated as part of the Exception Sheet work in Q1 25.

As per 3GPP TS 24.301 §4.4.3, each EPS security context has two independent NAS COUNTs (Uplink NAS COUNT and Downlink NAS COUNT) of 24 bits, split into the NAS SQN (8 LSBs) that wraps around from 0xFF back to 0x00) and the NAS overflow counter (16 MSBs), which increments once the NAS SQN wraps.