3GPP TSG-SA WG3 Meeting #120	S3-251097

# Athens, Greece, 17th  – 21st  February 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.180 | CR | 0217 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | × |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | IdMS - PKCE Downgrade Attack |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Leonardo SpA |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCXSec3 |  |  |  |  |  | Date: |  |  | 2025-02-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The code_challenge parameter is mandatorily required in the authentication request in order to support the PKCE protection, as explained in Table B.4.2.2-1. However, required could mean that when the authentication request is originated, it should contain the code_challenge parameter, but it is not checked whether it is removed or not by an attacker who intercepted and tampered the request. For this reason, there is a need to explicitly mention the possibility of the PKCE Downgrade Attack. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add clarifications where the IdMS shall ensure that if there was no code_challenge in the authorization request, a request to the token endpoint containing a code_verifier should be rejected. An alternative could be to reference the Section 4.8 of the OAuth 2.0 Security Best Current Practice document. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | This could results in a bypass of the PKCE protection, especially deriving from third-party OIDC libraries adjusted to be standard compliant. Additionally, this would facilitate the process of obtaining an access_token on behalf of a victim, if combined with other vulnerabilities. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, B.4.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | × | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | × | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | × | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** START OF 1st CHANGE ***

### B.4.2.2	Authentication request

As described in OpenID Connect 1.0, the IdM client constructs a request URI by adding the following parameters to the query component of the authorization endpoint's URI using the "application/x-www-form-urlencoded" format, redirecting the user's web browser to the authorization endpoint of the IdM server. The standard parameters shown in table B.4.2.2-1 are required by the MCX Connect profile. Other parameters defined by the OpenID Connect specification are optional.

Table B.4.2.2-1: Authentication Request standard required parameters

| Parameter | Values |
| --- | --- |
| response_type | REQUIRED. For native clients the value shall be set to "code". |
| client_id | REQUIRED. The identifier of the client making the API request. It shall match the value that was previously registered with the IdM server of the MCX service provider. |
| scope | REQUIRED. Scope values are expressed as a list of space-delimited, case-sensitive strings which indicate which MCX resource servers the client is requesting access to (e.g. MCPTT, MCVideo, MCData, KMS, etc.). If authorized, the requested scope values will be bound to the access token returned to the client. The scope value "openid" is defined by the OpenID Connect standard and is mandatory, to indicate that the request is an OpenID Connect request, and that an ID token should be returned to the client. This profile further defines the following additional authorization scopes: - "3gpp:mc:ptt_service" - "3gpp:mc:video_service" - "3gpp:mc:data_service" - "3gpp:mc:ptt_key_management_service" - "3gpp:mc:video_key_management_service" - "3gpp:mc:data_key_management_service" - "3gpp:mc:ptt_config_management_service" - "3gpp:mc:video_config_management_service" - "3gpp:mc:data_config_management_service" - "3gpp:mc:ptt_group_management_service" - "3gpp:mc:video_group_management_service" - "3gpp:mc:data_group_management_service" - "3gpp:mc:location_management_service" Others may be added in the future as new MCX resource servers are introduced by 3GPP (see note). |
| redirect_uri | REQUIRED. The URI of the client to which the IdM server will redirect the authentication response in order to return the authorization code. The URI shall match the redirect URI registered with the IdM server during the client registration phase. |
| state | REQUIRED. An opaque value used by the client to maintain state between the authentication request and authentication response. The IdM server includes this value in its authentication response. |
| acr_values | REQUIRED. Space-separated string that specifies the acr values that the IdM server is being requested to use for processing this authentication request, with the values appearing in order of preference. For minimum interoperability requirements, a password-based ACR value is mandatory to support. "3gpp:acr:password" as per the OpenID Connect 1.0 specification [21]. |
| code_challenge | REQUIRED. The base64url-encoded SHA-256 challenge derived from the code verifier, to be verified against later. |
| code_challenge_method | REQUIRED. The hash method used to transform the code verifier to produce the code challenge. This profile current requires the usage of "S256" |
| NOTE: The order in which they are expressed does not matter. |  |

An example of an authentication request for MCX Connect might look like:

EXAMPLE:

GET/as/authorization.oauth2?response_type=code&client_id=idm_client&scope=openid

3gpp:mc:ptt_service&redirect_uri=http://3gpp.mcptt/cb&state=abc123&acr_values=3gpp:acr:password&code_challenge=0x123456789abcdef&code_challenge_method=S256

HTTP/1.1
Host: IdMS.server.com:9031
Cache-Control: no-cache
Content-Type: application/x-www-form-urlencoded

Upon receiving the authentication request from the client, the IdM server performs user authentication. Note that user authentication should be completely opaque to the MC services on the client.

The IdMS rejects the authentication request if the code_challenge parameter is not present. This security practice is RECOMMENDED.

*** END OF 1st CHANGE ***