3GPP TSG-SA3 Meeting #120	S3-251101

Athens, Greece, 17 - 21 February 2025

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.511 | CR | 0077 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of gNB SCAS wording |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | BSI |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SCAS_5G_Maint |  |  |  |  |  | Date: |  |  | 2025-02-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The current wording of some test steps does not make it clear, if a test step is to be executed by the tester or that system behaviour is to be observed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | No changes in test content or intention. Added “The tester triggers/<verb>” if a test step is to be executed by the tester – adapted the remaining sentence accordingly. Some formatting fixes regarding tabs and numbering style B1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear test step wording. Incoherent formatting. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.2.1.1, 4.2.2.1.2, 4.2.2.1.6, 4.2.2.1.7, 4.2.2.1.12, 4.2.2.1.13, 4.2.2.1.14, 4.2.2.1.15, 4.2.2.1.18, 4.2.2.1.19, 4.2.2.1.22, 4.2.2.1.23 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

********** START OF 1st CHANGE **********

##### 4.2.2.1.1	Integrity protection of RRC-signalling

Requirement Name: Integrity protection of RRC-signalling

Requirement Reference: TS 33.501 [2], clause 5.3.3

Requirement Description: The gNB supports integrity protection and replay protection of RRC-signalling as specified in TS 33.501 [2], clause 5.3.3.

Threat References: TR 33.926 [5], clause D.2.2.2 – Control plane data integrity protection.

Test Case:

Test Name: TC_CP_DATA_INT_RRC-SIGN_gNB

Purpose: To verify that the RRC-signalling data sent between UE and gNB over the NG RAN air interface are integrity protected.

Pre-Condition:

-	The gNB network product shall be connected in emulated/real network environments. UE may be simulated.

-	Tester shall have access to the integrity algorithm and the integrity protection keys.

-	The tester can capture the message via the NG RAN air interface, or can capture the message at the UE.

-	The NIA0 is disabled at UE and gNB.

Execution Steps:

1.	The tester triggers the gNB to send AS SMC message to the UE, and UE responses AS SMP.

2.	The tester checks any RRC message sent by gNB after sending AS SMC and before UE enters CM-Idle state is integrity protected.

Expected Results:

Any RRC-signalling over the NG RAN air interface is integrity protected after gNB sending AS SMC.

Expected format of evidence:

Evidence suitable for the interface, e.g. Screenshot containing the operational results.

##### 4.2.2.1.2	Integrity protection of user data between the UE and the gNB

Requirement Name: Integrity protection of user data between the UE and the gNB.

Requirement Reference: TS 33.501 [2], clause 5.3.3

Requirement Description: The gNB supports integrity protection and replay protection of user data between the UE and the gNB as specified in TS 33.501 [2], clause 5.3.3.

NOTE: 	This requirement does not apply to the gNB that is used as a secondary node connecting to the EPC.

Threat References: TR 33.926 [5], clause D.2.2.4 – User plane data integrity protection.

Test Case:

Test Name: TC-UP-DATA-INT_gNB

Purpose: To verify that the user data packets are integrity protected over the NG RAN air interface.

Pre-Condition:

-	The gNB network product shall be connected in emulated/real network environments. UE may be simulated.

-	Tester shall enable the user plane integrity protection and ensure NIA0 is not used.

-	Tester shall have knowledge of integrity algorithm and integrity protection keys.

-	The tester can capture the message via the NG RAN air interface, or can capture the message at the UE.

-	The NIA0 is disabled at the UE and gNB.

Execution Steps:

1.	The tester triggers the gNB to send RRCConnectionReconfiguration with integrity protection indication "on".

2.	The tester checks any User data sent by gNB after sending RRCConnectionReconfiguration and before UE enters CM-Idle state is Integrity protected.

Expected Results:

Any user plane packets sent between UE and gNB over the NG RAN air interface after gNB sending RRCConnectionReconfiguration is integrity protected.

Expected format of evidence:

Evidence suitable for the interface e.g. Screenshot containing the operational results.

********** START OF 2nd CHANGE **********

##### 4.2.2.1.6	Ciphering of RRC-signalling

Requirement Name: Ciphering of RRC-signalling

Requirement Reference: TS 33.501 [2], clause 5.3.2

Requirement Description: The gNB supports ciphering of RRC-signalling as specified in TS 33.501 [2], clause 5.3.2.

Threat References: TR 33.926 [5], clause D.2.2.1 – Control plane data confidentiality protection.

Test Case:

Test Name: TC-CP-DATA-CIP-RRC-SIGN_gNB

Purpose: To verify that the RRC-signalling data sent between UE and gNB over the NG RAN air interface are confidentiality protected.

Pre-Condition:

-	The gNB network product shall be connected in emulated/real network environments. The UE may be simulated.

-	The tester shall have access to the NG RAN air interface or can capture the message at the UE.

Execution Steps:

1.	The tester triggers the UE to send a Registration Request to the AMF.

2.	The AMF sends a KgNB and the UE security capability to the gNB.

3.	The gNB selects an algorithm and sends AS SMC to the UE.

4.	The gNB receive AS SMP from the UE.

Expected Results:

Control plane packets sent to the UE after the gNB sends AS SMC is ciphered.

Expected format of evidence:

Evidence suitable for the interface, e.g. Screenshot containing the operational results.

##### 4.2.2.1.7	Ciphering of user data between the UE and the gNB

Requirement Name: Ciphering of user data between the UE and the gNB

Requirement Reference: TS 33.501 [2], clause 5.3.2

Requirement Description:  The gNB supports ciphering of user data between the UE and the gNB. as specified in TS 33.501 [2], clause 5.3.2.

Threat References: TR 33.926 [5], clause D.2.2.3 – User plane data confidentiality protection at gNB

Test Case:

Test Name: TC-UP-DATA-CIP_gNB

Purpose: To verify that the user data packets are confidentiality protected over the NG RAN air interface.

Pre-Condition:

-	The gNB network product shall be connected in emulated/real network environments. The UE may be simulated.

-	The tester shall have access to the NG RAN air interface or can capture the message at the UE.

Execution Steps:

1.	The tester triggers the UE to send PDU session establishment Request to the SMF.

2.	The SMF sends a UP security policy with UP ciphering required or preferred to the gNB.

3.	The gNB sends RRCConnectionReconfiguration with ciphering protection indication "on".

4.	The tester checks any user data sent by the gNB after sending RRCConnectionReconfiguration and before the UE enters into CM-Idle state.

Expected Results:

The user plane packets sent to the UE after the gNB sends RRCConnectionReconfiguration is confidentiality protected.

Expected format of evidence:

Evidence suitable for the interface e.g. Screenshot containing the operational results.

********** START OF 3rd CHANGE **********

##### 4.2.2.1.12	AS algorithms selection

Requirement Name: AS algorithms selection

Requirement Reference: TS 33.501 [2], clause 6.7.3.0 and clause 5.11.2.

Requirement Description: The gNB selects the algorithms to use dependent on: the UE security capabilities of the UE, the configured allowed list of security capabilities of the currently gNB as specified in TS 33.501 [2], clause 5.11.2.

Each gNB is configured via network management with lists of algorithms which are allowed for usage. There is one list for integrity algorithms, and one for ciphering algorithms. These lists are ordered according to a priority decided by the operator as specified in TS 33.501 [2], clause 6.7.3.0.

Threat References: TR 33.926 [5], D.2.2.5 – AS algorithm selection and use

Test Case:

Test Name: TC-AS-alg-select_gNB

Purpose:

Verify that the gNB selects the algorithms with the highest priority in its configured list.

Pre-Conditions:

Test environment with the gNB has been pre-configured with allowed security algorithms with priority.

Execution Steps

1)	The tester triggers the UE to send registration request message to the gNB.

2)	The gNB receives UE context setup request message.

3)	The gNB sends the AS SECURITY MODE COMMAND message.

4)	The UE replies with the AS SECURITY MODE COMPLETE message.

Expected Results:

The gNB initiates the SECURITY MODE COMMAND message that includes the chosen algorithm with the highest priority according to the ordered lists and is contained in the UE NR security capabilities.

The MAC in the AS SECURITY MODE COMPLETE message is verified, and the AS protection algorithms are selected and applied correctly.

Expected format of evidence:

Sample copies of the log files.

##### 4.2.2.1.13	Key refresh at the gNB

Requirement Name: Key refresh at the gNB

Requirement Reference: TS 33.501 [2], clause 6.9.4.1; TS 38.331 [6], clause 5.3.1.2

Requirement Description: Key refresh is possible for KgNB, KRRC-enc, KRRC-int, KUP-enc, and KUP-int (if available), and is to be initiated by the gNB/ng-eNB when a PDCP COUNTs are about to be re-used with the same Radio Bearer identity and with the same KgNB. as specified in TS 33.501 [2], clause 6.9.4.1.

The network is responsible for avoiding reuse of the COUNT with the same RB identity and with the same key, e.g. due to the transfer of large volumes of data, release and establishment of new RBs, and multiple termination point changes for RLC-UM bearers and multiple termination point changes for RLC-AM bearer with SN terminated PDCP re-establishment (COUNT reset) due to SN only full configuration whilst the key stream inputs (i.e. bearer ID, security key) at MN have not been updated. In order to avoid such re-use, the network e.g. uses different RB identities for RB establishments, change the AS security key, or an RRC_CONNECTED to RRC_IDLE/RRC_INACTIVE and then to RRC_CONNECTED transition as specified in TS 38.331 [6], clause 5.3.1.2.

Threat References: TR 33.926 [5], clause D.2.2.7 Key Reuse

Test Case :

Test Name: TC_GNB_KEY_REFRESH_DRB_ID

Purpose:

Verify that the gNB performs KgNB refresh when DRB-IDs are about to be reused under the following conditions:

-	the successive Radio Bearer establishment uses the same RB identity while the PDCP COUNT is reset to 0, or

-	the PDCP COUNT is reset to 0 but the RB identity is increased after multiple calls and wraps around.

Pre-Conditions:

The UE, AMF and SMF may be simulated.

Execution Steps

1)  The tester triggers the gNB to send the AS Security Mode Command message to the UE.

2)	The UE responds with the AS Security Mode Complete message.

3)	A DRB is set up.

4)	The tester sets up and tears down the DRB for multiple times within one active radio connection without the UE going to idle (e.g. by triggering the UE to make multiple IMS calls, or by triggering the SMF to request PDU session modification and deactivation via the AMF), until the DRB ID is reused.

Expected Results:

Before DRB ID reuse, the gNB takes a new KgNB into use by e.g. triggering an intra-cell handover or triggering a transition from RRC_CONNECTED to RRC_IDLE or RRC_INACTIVE and then back to RRC_CONNECTED.

NOTE:	Random Access Procedure defined in clause 9.2.6 of TS 38.300[8] runs in the above procedures.

Expected format of evidence:

Part of log that shows all the DRB identities and the corresponding procedure. This part can be presented, for example, as a screenshot.

##### 4.2.2.1.14	Bidding down prevention in Xn-handovers

Requirement Name: Bidding Down Prevention

Requirement Reference: TS 33.501 [2], clause 6.7.3.1

Requirement Description: In the Path-Switch message, the target gNB/ng-eNB sends the UE's 5G security capabilities received from the source gNB/ng-eNB to the AMF. as specified in TS 33.501 [2], clause 6.7.3.1.

Threat References: TR 33.926 [5], clause D.2.2.6 Bidding Down on Xn-Handover

Test Case:

Test Name: TC-Xn-handover_bid_down_gNB

Purpose:

Verify that bidding down is prevented in Xn-handovers.

Pre-Conditions:

Test environment with source gNB and target gNB, and the source gNB may be simulated.

Execution Steps:

The tester triggers the target gNB to send the path-switch message to the AMF.

Expected Results:

The UE NR security capabilities are in the path-switch message.

Expected format of evidence:

Snapshots containing the result.

##### 4.2.2.1.15	AS protection algorithm selection in gNB change

Requirement Name: AS protection algorithm selection in gNB change.

Requirement Reference: TS 33.501 [2], clauses 6.7.3.1 and 6.7.3.2

Requirement Description: The target gNB/ng-eNB selects the algorithm with highest priority from the received 5G security capabilities of the UE according to the prioritized locally configured list of algorithms (this applies for both integrity and ciphering algorithms). The chosen algorithms are indicated to the UE in the Handover Command message if the target gNB/ng-eNB selects different algorithms compared to the source gNB/ng-eNB as specified in TS 33.501 [2], clause 6.7.3.1, and clause 6.7.3.2.

Threat References: TR 33.926 [5], D.2.2.5 – AS algorithm selection and use

Test Case:

Test Name: Alg_select_change_gNB

Purpose:

Verify that AS protection algorithm is selected correctly.

Pre-Conditions:

Test environment with source gNB, target gNB and AMF. Source gNB and AMF may be simulated.

Execution Steps:

Test Case 1:

1.	The tester triggers the source gNB to transfer the ciphering and integrity algorithms used in the source cell to the target gNB in the handover request message.

2.	Target gNB verifies the algorithms and selects AS algorithms which have the highest priority according to the ordered lists. Target gNB includes the algorithm in the handover command.

Test Case 2:

1.	The tester triggers the AMF to send the UE NR security capability to the Target gNB.

2.	The target gNB selects the AS algorithms which have the highest priority according to the ordered lists in the HANDOVER COMMAND.

The above test cases assume that the algorithms selected by the target gNB are different from the ones received from the source gNB.

Expected Results:

For both test cases:

1.	The UE checks the message authentication code on the handover command message.

2.	The MAC in the handover complete message is verified, and the AS integrity protection algorithm is selected and applied correctly.

Expected format of evidence:

Snapshots containing the result.

********** START OF 4th CHANGE **********

##### 4.2.2.1.18	Key update at the gNB on dual connectivity

Requirement Name: Key update at the gNB on dual connectivity

Requirement Reference: TS 33.501 [2], clause 6.10.2.1; clause 6.10.2.2.1; clause 6.10.3.1.

Requirement Description: When executing the procedure for adding subsequent radio bearer(s) to the same SN, the MN is expected to, for each new radio bearer, assign a radio bearer identity that has not previously been used since the last KSN change. If the MN cannot allocate an unused radio bearer identity for a new radio bearer in the SN, due to radio bearer identity space exhaustion, the MN is expected to increment the SN Counter and compute a fresh KSN, and then is expected to perform a SN Modification procedure to update the KSN as specified in TS 33.501 [2], clause 6.10.2.1.

The MN is expected to refresh the root key of the 5G AS security context associated with the SN Counter before the SN Counter wraps around. Refreshing the root key is done using intra cell handover as described in subclause 6.7.3.3 of TS 33.501 [2]. When the root key is refreshed, the SN Counter is reset to '0' as defined above. in that same clause; as specified in TS 33.501 [2], clause 6.10.3.1.

NOTE:	The following testcases are only tested when the NR-NR DC, NE-DC and EN-DC scenarios are deployed.

Threat References: TR 33.926 [5], clause D.2.2.7 Key Reuse

Test Case 1:

Test Name: TC_GNB_DC_KEY_UPDATE_DRB_ID

Purpose:

Verify that the gNB under test acting as a Master Node (MN) performs KSN update when DRB-IDs are about to be reused.

Pre-Conditions:

-	Test environment with a gNB or ng-eNB acting as the Secondary Node (SN), which may be simulated

-	Test environment with a UE, SMF and AMF, which may be simulated

Execution Steps

1.	The tester triggers the gNB under test to establish RRC connection and AS security context with the UE.

2.	The gNB under test establishes security context between the UE and the SN for the given AS security context shared between the gNB under test and the UE; and generates a KSN sent to the SN.

3.	A SCG bearer is set up between the UE and the SN.

4.	The tester triggers the gNB under test to execute the SN Modification procedure to provide additional available DRB IDs to be used for SN terminated bearers (e.g. by triggering the UE to make multiple IMS calls, or by triggering the SMF to request PDU session modification and deactivation via the AMF), until the DRB IDs are reused.

Expected Results:

-	Before DRB ID reuse, the gNB under test generates a new KSN and sends it via the SN Modification Request message to the SN.

Expected format of evidence:

Evidence suitable for the interface, e.g. text representation of the captured SN Modification Request message.

Test Case 2:

Test Name: TC_GNB_DC_KEY_UPDATE_SN_COUNTER

Purpose:

Verify that the gNB under test acting as a Master Node (MN) performs KNG-RAN (AS root key) update when SN COUNTER is about to wrap around.

Pre-Conditions:

-	Test environment with a gNB or ng-eNB acting as the Secondary Node (SN), which may be simulated

-	Test environment with a UE, SMF and AMF, which may be simulated.

Execution Steps

1.	The tester triggers the gNB under test to establish RRC connection and AS security context with the UE.

2.	The gNB under test establishes security context between the UE and the SN for the given AS security context shared between the gNB under test and the UE; and generates a KSN sent to the SN and increases the value of SN Counter.

3.	A SCG bearer is set up between the UE and the SN.

4.	The tester triggers the gNB under test to execute the SN Modification procedure to provide updated KSN to SN, until the SN Counter value wraps around.

Expected Results:

-	Before SN Counter wraps around, the gNB under test takes a new KNG-RAN into use by e.g. triggering an intra-cell handover or triggering a transition from RRC_CONNECTED to RRC_IDLE or RRC_INACTIVE and then back to RRC_CONNECTED.

NOTE:	Random Access Procedure defined in clause 9.2.6 of TS 38.300[8] runs in the above procedures.

Expected format of evidence:

Part of log that shows the SN Counter values before and after wrapping around and the corresponding procedure. This part can be presented, for example, as a screenshot.

##### 4.2.2.1.19	UP security activation in Inactive scenario

Requirement Name: UP security activation in Inactive scenario

Requirement Reference: TS 33.501 [2], clause 6.8.2.1.3.

Requirement Description: If the UP security activation status can be supported in the target gNB/ng-eNB, the target gNB/ng-eNB uses the UP security activations that the UE used at the last source cell. Otherwise, the target gNB/ng-eNB responds with an RRC Setup message to establish a new RRC connection with the UE as specified in TS 33.501 [2], clause 6.8.2.1.3.

Threat Reference:  TR 33.926 [5], clause D.2.2.9 State transition from inactive state to connected state.

Test Name: TC_GNB_INACTIVE_TO_ACTIVE

Purpose:

Verify that the target gNB/ng-eNB uses the UP security activation status to activate the UP security.

Pre-Conditions:

-	The gNB network product shall be connected in emulated/real network environments.

-	The UE may be simulated.

Execution Steps

1.	The tester shall complete a Registration Procedure and PDU Session establishment procedure to make sure the gNB configure the UP security, and get the UP security activation status.

2.	The gNB sends RRC Release message with a suspend config to the UE.

3.	The tester deletes the UP security activation status of the UE.

4.	The tester triggers the UE to send RRC Resume message.

Expected Results:

The gNB sends RRC Setup message to the UE.

Expected format of evidence:

Screenshot containing the operational results.

********** START OF 5th CHANGE **********

##### 4.2.2.1.22	Checking expiry certificate

Requirement Name: Expired Certificate checking at base station

Requirement Reference: TS 33.310 [10], clause 7.2

Requirement Description:

Certificate Management Protocol v2 (CMPv2) [11] may be the supported protocol to provide certificate lifecycle management capabilities for TLS entities. All TLS entities and TLS CAs may support initial enrolment by the TLS entity to the TLS CA via CMPv2, i.e. receiving a certificate from the TLS CA, and updating the key of the certificate via CMPv2 before the certificate expires.

Threat References: TBD

Test case:

Test Name: TC_EXPIR_CERT_CHCK

Purpose:

Verify that the gNB can check whether its certificate issued by operator CA is about to expire and to act accordingly.

Pre-Conditions:

-	If the gNB under test does not support handling certificates as defined in clause 9 of TS 33.310[10], this test does not apply.

-	The gNB network product is connected in emulated/real network environments.

-	A TLS CA may be emulated, if needed.

-	The gNB is configured the necessary information to connect with the CA server.

-	Optionally, the vendor may provide necessary information in a document, e.g. describing how to handle the case when a gNB checks the operator certificate is about to be expired.

Execution Steps

1.	The tester configures the gNB with certificates that is assigned by operator CA.

2.	The tester configures the UTC timer of gNB to the time when its own certificate is about to be expired.

3.	The gNB initiates the CMPv2 procedure to get the new operator certificate.

Expected Results:

-	The gNB raises an alarm or requests a new certificate from the CA.

-	The gNB received a new operator certificate.

Expected format of evidence:

The logs and the communication flow in a .pcap file.

##### 4.2.2.1.23	Peer certificate checking

Requirement Name: Peer certificate checking at base station

Requirement Reference: TS 33.310 [10], clause 9.5.1

Requirement Description:

-	The base station may be pre-provisioned with the operator root CA certificate.

-	If the base station is not pre-provisioned with the operator root CA certificate, then the base station takes the operator root certificate from the certificates received in the initialization response. The selection is based on checking which root certificate can be used to validate the received base station certificate.

NOTE1: Examples on validation factors defined in clause 6.3.1 of TS 33.310[10] can be taken into consideration, for example, whether the certificated has been revoked, expired, on the CRL distribution point is empty, etc.

Test case:

Test Name: TC_PEER_CERT_CHCK

Purpose:

Verify that the gNB has the ability to check the peer certificate is valid or not.

Pre-Conditions:

-	If the gNB under test does not support handling certificates as defined in clause 9 of TS 33.310[10], this test does not apply.

-	The gNB is configured with or has obtained an operator root CA certificate.

-	The gNB network product shall be connected in emulated/real network environments.

-	A peer, e.g., AMF, SEG, gNB may be emulated.

NOTE2: according to 5GS, only AMF, SEG/UPF, gNB can connect to a gNB. The peer means the network function that provides the operator assigned certificate to the gNB for establishing the N2, N3, Xn interfaces.

-	The gNB is configured the necessary information to connect with the peer.

-	Optionally, the vendor may provide necessary information in a document, e.g., describing the checking factors and how to handle the case when a gNB validate the certificate is invalid.

Execution Steps:

1.	The tester configures the peer with an invalid certificate e.g., by using a wrong signature or a certificate that has expired.

2.	The tester triggers the gNB to connect to the peer.

Expected Results:

-	The gNB does not connect with the peer and may raise an alarm at the same time.

Expected format of evidence:

The logs and the communication flow in a .pcap file.