3GPP TSG-SA3 Meeting #120	S3-251105-r3

# Athens, Greece, 17 - 21 February 2025	revision of S3-250623

Source:	China Mobile

Title:	Pseudo-CR on New solution on AIoT privacy

Document for:	Approval

Agenda item:	5.9

Spec:	3GPP TR 33.713

Version:	0.6.0

Work Item:	FS_Ambient_IoT_Sec

Comments

This contribution proposes a new Temp ID based privacy protection solution. This solution proposes to generate Temp ID by AIoT device. Compared to the solution with Temp Id generated by 5GC, this solution can avoid recurring of writing signaling due to failure of writing operation or AIoT device out of power. Frequent writing operations shall be avoided based on LS from RAN1(R1-2405796)

Proposed Changes

* * * First Change * * * *

6.Y	Solution #Y: Temp ID based privacy protection for Ambient IoT device identifier

6.Y.1	Introduction

This solution addresses key issue #3 on privacy by protecting AIoT device identifiers. It is based on a temp ID generated by Ambient IoT device. Compared to the solutions with temp ID assigned by 5GC, this solution can avoid frequent sync-up procedures (writing operations) between 5GC and Ambient IoT Device due to failure of writing operation or AIoT device out of power. The main points of this solution are as follows:

1)  The ambient IoT device assigns the temp ID, not 5GC.

2)  The Ambient IoT temp ID is synced between the ambient IoT device and 5GC by standard inventory procedure.  Thus, there is no writing signaling between 5GC and the Ambient IoT Device.

3)  5GC will obtain and maintain the binding relationship between temp ID and permanent device ID via standard inventory procedure.

4)  5GC will add an “ID response indication” parameter in the standard inventory message to instruct ambient IoT devices to respond with only Temp ID or both Temp ID and permanent device ID to realize uplink privacy protection

5) 5GC can use Temp ID to replace permanent device ID as the target ambient device filter info in inventory message from 5GC to ambient IoT devices to realize downlink privacy protection

6.Y.2	Solution details

Temp ID generation and sync up procedure between device and 5GC

Figure x: Procedure for Temp ID generation and sync up between device and 5GC

Procedure:

- When AF wants to trigger inventory request to an AIoT device, it sends request with AIoT permanent device ID and AF ID.

- When 5G network receives the request, it enables privacy protection based on AF ID. After that, it checks whether there is temporary ID mapped to the permanent device ID. If no, it sends inventory request with AIoT permanent device ID and ID response indication, which is set to represent "report both Temp ID and Permanent device ID".

- When AIoT device receives this request, it generates temp ID based on the indication, and write it locally. After that, it sends inventory response with its permanent device ID and temp ID together.

- 5G network gets the binding relationship between permanent ID and temp ID from received response. Then 5G network forward permanent ID to AF in inventory response message.

Editor’s note: It’s not clear how the permanent ID is protected in up and downlink.

Editor’s note: It is FFS how to keep uniqueness of temp ID.

Editor’s note: The sequence number of message needs to be added in Figure.

Downlink and Uplink Privacy protection based on Temp ID

Figure y: Procedure for Downlink and Uplink Privacy protection based on Temp ID

Procedure:

- When AF wants to trigger inventory request to an AIoT device, it sends request with AIoT permanent device ID and AF ID.

- When 5G network receives the request, it enables privacy protection based on AF ID. After that, it checks whether there is temporary ID mapped to the permanent device ID. If yes, it sends inventory request with temp ID to provide privacy protection, and ID response indication which is set to represent "report Temp ID only".

- When AIoT device receives this request based on its temp ID, it sends inventory response with its temp ID.

- 5G network gets the binding relationship between permanent ID and temp ID from received response retrieve its permanent ID by using temp ID from received response. Then 5G network forward permanent ID to AF in inventory response message.

4a. If 5G network could not retrieve AIoT device permanent ID or failed to get response. It needs to relaunch inventory request with AIoT permanent device ID and "report both Temp ID and Permanent device ID" ID response indication .

Editor’s note: The sequence number of message needs to be added in Figure.

Temp ID update and sync up procedure

Figure z: Procedure for Temp ID update and sync up

Procedure:

- When AF wants to trigger inventory request to an AIoT device, it sends request with AIoT permanent device ID and AF ID.

- When 5G network receives the request, it enables privacy protection based on AF ID. After that, it checks whether there is temporary ID mapped to the permanent device ID. If yes, it may decide to update temp ID based on local policy. It sends inventory request with temp ID to provide privacy protection, and ID response indication which is set to represent "update temp ID, and report both temp ID and permanent device ID ".

- When AIoT device receives this request based on its temp ID, it calculates  a new temp ID and save it locally, then sends inventory response with its new temp ID and permanent ID together.

- 5G network gets the binding relationship between permanent ID and temp ID from received response and update its mapping table. Then 5G network forward permanent ID to AF in inventory response message.

4a. If 5G network fails to get response. It needs to relaunch inventory request with AIoT permanent device ID and "report both Temp ID and Permanent device ID" ID response indication.

Editor’s note: It is not clear how the ID is stored in the solution.

Editor’s note: Tracking or tracing of AIoT device using the unprotected ID response indicator is not addressed in the present document.

Editor’s note: It is FFS how to keep uniqueness of temp ID.

Editor’s note: The sequence number of message needs to be added in Figure.

6.Y.3	Evaluation

Editor’s Note: Further evaluation is FFS

* * * End of Changes * * * *