3GPP TSG-SA3 Meeting #120	S3-250370

# Athens, Greece, 17 -21 February 2025

# Source:	KDDI

# Title:	Technical Content for 3GPP Cryptographic Inventory

# Document for:	Approval

Agenda Item:	5.20

# 1	Decision/action requested

The SA3 is kindly asked to review and approve the proposed changes to the 3GPP cryptographic inventory TR.

# 2	References

None.

# 3	Rationale

In the NIST’s report impact of quantum computing on common cryptographic algorithms is summarised.

# 4	Detailed proposal

******************* Start of Change ****************

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[X]		UK NCSC, Next steps in preparing for post-quantum cryptography
https://www.ncsc.gov.uk/whitepaper/next-steps-preparing-for-post-quantum-cryptography

[Y]		US NSA, The Commercial National Security Algorithm Suite 2.0

[Y1]	S3-200929, ETSI SAGE, Observations and questions on 256-bit security goals

[Y2]	NIST Special Publication 800-38B, Recommendation for Block Cipher Modes of Operation: The CMAC Mode for Authentication

[Z]		S3‑242363, Recent attack on polynomial based MACs with short tag

[Z1]	3GPP TS 33.501, Security architecture and procedures for 5G System

******************* Next Change ****************

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Cryptographically Relevant Quantum Computer (CRQC): 
“Quantum computers use properties of quantum mechanics to compute in a fundamentally different way from today's digital, 'classical', computers. They are, theoretically, capable of performing certain computations that would not be feasible for classical computers.” [2]

“If such a computer could exist in the future, most traditional public key cryptography (PKC) algorithms in use today will be vulnerable to attacks from it.” [2]

Traditional Public Key Cryptography (PKC) Algorithms:
“These traditional PKC algorithms include:

- algorithms based on integer factorisation such as RSA

- algorithms based on the discrete logarithm problem such as Finite Field Diffie-Hellman, ECDH, DSA, ECDSA, EdDSA” [2]

Classical Computer / Traditional Computer:
In the context of quantum computing and in comparison, to a CRQC, in classical/traditional computers the bits of data can exist either on zero (0) or one (1).

Hybrid Mode:
Implementation and coexistence of traditional cryptographic algorithms and post-quantum cryptographic algorithms.

******************* Next Change ****************

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AES		Advanced Encryption Standard

AKA		Authentication and Key Agreement

CRQC		Cryptographically Relevant Quantum Computer

DSA		Digital Signature Algorithm

ECDH		Elliptic Curve Diffie-Helman

ECDHE		Elliptic Curve Diffie-Helman Ephemeral

ECDSA		Elliptic Curve	Digital Signature Algorithm

EdDSA		Edwards-Curve Digital Signature Algorithm

FIPS		Federal Information Processing Standards

GSM		Global System for Mobile Communications

HMAC		Hash-based Message Authentication Code

MAC		Message Authentication Code

NCSC UK	National Cyber Security Centre United Kingdom

NIST US		National Institute of Standards and Technology United States

PKC		Public Key Cryptography

RSA		Rivest-Shamir-Adleman

SECG		Security Gateway

SHA		Secure Hash-Algorithm

******************* Next Change ****************

# 4	List of security algorithms and protocols

## 4.1	Symmetric Cryptographic Algorithms

Table 4.1-1 provides an overview of symmetric key and hash algorithms which are used in the 5G System. All current algorithms can continue to be used, but 256-bit algorithms are preferred.

Table 4.1-1: Symmetric key algorithms

| Cryptographic algorithms | Type | Specification/ Profile | Purpose | Impact from large-scale quantum computer |
| --- | --- | --- | --- | --- |
| 128-NEA1 | Symmetric key | 3GPP TS 35.215 | Confidentiality | Larger key size preferred (Note 1) |
| 128-NIA1 | Symmetric key | 3GPP TS 35.215 | Integrity | Larger key size preferred (Note 1, Note 2) |
| 128-NEA2 | Symmetric key | NIST FIPS PUB 197 NIST Special Publication 800-38A | Confidentiality | Larger key size preferred (Note 1) |
| 128-NIA2 | Symmetric key | NIST FIPS PUB 197 NIST Special Publication 800-38B | Integrity | Larger key size preferred (Note 1, Note 2) |
| 128-NEA3 | Symmetric key | 3GPP TS 35.221 | Confidentiality | Larger key size preferred (Note 1) |
| 128-NIA3 | Symmetric key | 3GPP TS 35.221 | Integrity | Larger key size preferred (Note 1, Note 2) |
| 256-NEA4 | Symmetric key | 3GPP TS 35.240 | Confidentiality | Secure |
| 256-NIA4 | Symmetric key | 3GPP TS 35.240 | Integrity | Larger output size preferred (Note 3, Note 4) |
| 256-NEA5 | Symmetric key | 3GPP TS 35.243 | Confidentiality | Secure |
| 256-NIA5 | Symmetric key | 3GPP TS 35.243 | Integrity | Larger output size preferred (Note 3, Note 4) |
| 256-NEA6 | Symmetric key | 3GPP TS 35.246 | Confidentiality | Secure |
| 256-NIA6 | Symmetric key | 3GPP TS 35.246 | Integrity | Larger output size preferred (Note 3, Note 4) |
| MILENAGE-128 | Symmetric key | 3GPP TS 35.205 | AKA | Larger key size preferred (Note 1) |
| MILENAGE-256 | Symmetric key | 3GPP TS 35.235 | AKA | Secure |
| TUAK-128 | Symmetric key | 3GPP TS 35.231 | AKA | Larger key size preferred (Note 1) |
| TUAK-256 | Symmetric key | 3GPP TS 35.231 | AKA | Secure |
| HMAC-SHA-256 | MAC | RFC 2104 NIST FIPS 180-2 | Key derivation | Larger output size preferred |

Note 1: NCSC have declared that “security of symmetric cryptography is not significantly impacted by quantum computers, and existing symmetric algorithms with at least 128-bit keys [...] can continue to be used” [X]. However, from a security perspective, 256-bit algorithms are preferrable [Y].

Note 2: All three 128-NIA algorithms generate 32-bit MAC tags. From a security perspective, larger MAC lengths are preferrable [Y1] [Y2].

Note 3: All three 256-NIA algorithms generate 128-bit MAC tags which are truncated to 32 bit. From a security perspective, larger MAC lengths are preferrable [Y1] [Y2].

Note 4: Though there are three algorithms for integrity protection, all algorithms are based on the same design, i.e., GCM based, and vulnerable to the attack proposed in [Z]. Having new algorithms based on a different design principle is preferred to avoid a situation where all algorithms are broken by a single attack.

## 4.2	Asymmetric Cryptographic Algorithms

Table 4.2-1 provides an overview of asymmetric key algorithms which exist in 5G System. Unlike the symmetric key algorithms, all asymmetric key algorithms will become unsecure when a CRQC is realised.

Table 4.2-1: Asymmetric key algorithms

| Cryptographic algorithms | Type | Specification/ Profile | Purpose | Impact from large-scale quantum computer |
| --- | --- | --- | --- | --- |
| ECIES | Asymmetric, symmetric key, hash & MAC | SECG SEC 1 | Confidentiality | No longer secure (Note 1) |
| RSA | Asymmetric key | RFC 8017 | Confidentiality | No longer secure |
|  |  | RFC 8017 | Digital signature | No longer secure |
| ECDHE | Asymmetric key | NIST SP 800-56A | Key exchange | No longer secure |
| ECDSA | Asymmetric key | NIST FIPS 186-5 | Digital signature | No longer secure |

Note 1: ECIES uses AES-128-CTR for symmetric key encryption algorithm, SHA-256 for hash and HMAC-SHA-256 for MAC. The resulting MAC tag is truncated to 64 bits [Z1]. According to NCSC, “[t]he security of hash functions such as SHA-256 is also not significantly affected, and secure hash functions can also continue to be used.” However, as noted in section 4.3.1, larger keys and MAC lengths are preferrable. EC Diffie-Hellman, which is used for asymmetric key encryption, requires to be migrated to adequate post-quantum algorithms.

******************* End of Change ****************