3GPP TSG-SA3 Meeting #120	S3-250405

# Athens, Greece, 17 - 21 February 2025

Source:	Huawei, HiSilicon

Title:	Pseudo-CR on inventory of 5G security protocols by organizations

Document for:	Approval

Agenda item:	5.20

Spec:	3GPP TR 33.938

Version:	0.0.0

Work Item:	FS_CryptoInv

Comments

This contribution provides inventory of security protocols used in 5G systems for TR 33.938.

* * * First Change * * * *

## 4.Y	Security protocol inventory by organizations

This clause provides inventory of security protocols according to the custodian organizations.

Table 4.y-1. Inventory of 3GPP Security protocols by organizations

| Organi-zations | External cryptography specification | Protocol/cryptography used in 3GPP | 3GPP specifications, profiles, clauses | Type of cryptography | Cryptography usage type |
| --- | --- | --- | --- | --- | --- |
| Internal |  |  |  |  |  |
| 3GPP /ETSI | - | 5G AKA | TS 33.501 | symmetric | Authentication |
|  | - | MILEANAGE/TUAK | TS 33.501 TS 35.206 TS 35.231 | symmetric | Authentication, Integrity |
|  | - | SNOW 3G | TS 33.501 TS 33.215 | symmetric | Integrity, Confidentiality |
|  | - | ZUC 128 | TS 33.501 TS 33.221 | symmetric | Integrity, Confidentiality |
| External |  |  |  |  |  |
| NIST | FIPS PUB 197 | AES-128 | TS 33.501 | symmetric | Integrity, Confidentiality |
| SECG | Elliptic Curve Integrated Encryption Scheme (ECIES) | UE SUPI privacy | TS 33.501 | symmetric and asymmetric | Integrity, Confidentiality |
| IETF | RFC 5448 RFC 9048 | EAP-AKA’ | TS 33.501 (6.1.3.2) | symmetric | Authentication |
|  | RFC 3748 | EAP framework | TS 33.501 (6.1.1.2, 11.1, 16.3) | symmetric and asymmetric | Authentication |
|  | RFC 5216 | EAP-TLS | TS 33.501 (Annex B) | symmetric and asymmetric | Authentication, Integrity, Confidentiality |
|  | RFC 8446 | TLS | TS 33.501 (clauses 13, 12, 15) | symmetric and asymmetric | Authentication, Integrity, Confidentiality |
|  | RFC 9113 (HTTP2) |  | HTTPS |  | TS 33.501 (clauses 13, 12, 15) |  | symmetric and asymmetric symmetric and asymmetric |  | Authentication, Integrity, Confidentiality |  |
|  | RFC 2818 (HTTP over TLS) |  |  |  |  |  |  |  |  |  |
|  | RFC 6749 |  | OAuth2.0 |  | TS 33.501 (clauses 12, 13, 15) |  | symmetric and asymmetric |  | Authorization |  |
|  | RFC 4301 |  | IPsec |  | TS 33.501 (clauses 7, 9) |  | symmetric and asymmetric |  | Authentication, Integrity, Confidentiality |  |
|  | RFC 7296 |  | IKEv2 |  | TS 33.501 (clauses 7, 9) |  | symmetric and asymmetric |  | Authentication, Integrity, Confidentiality |  |
|  | RFC 6083 |  | DTLS |  | TS 33.501 (clauses 7, 9) |  | symmetric and asymmetric |  | Authentication, Integrity, Confidentiality |  |
|  | RFC 7858 RFC 8310 |  | DNS over TLS |  | TS 33.510 (Annex T) |  | symmetric and asymmetric |  | Authentication, Integrity, Confidentiality |  |
|  | RFC 5280 | Certificate X.509v3 | TS 33.210 TS 33.310 | asymmetric | Authentication, Integrity, Confidentiality |
|  | RFC 5280 | CRL | TS 33.210 TS 33.310 | asymmetric | Revocation |
|  | RFC 6960 | OCSP | TS 33.210 TS 33.310 | asymmetric | Revocation |
|  | RFC 2986 | PKCS #10 | TS 33.210 TS 33.310 | asymmetric | Authentication, Integrity, Confidentiality |
|  | RFC 4210 RFC 4211 RFC 6712 RFC 9480 RFC 9481 | CMPv2 | TS 33.210 TS 33.310 | asymmetric | Authentication, Integrity, Confidentiality |
|  | RFC 7515 | JWS | TS 33.210 TS 33.310 TS 33.501 | asymmetric | Authentication, Integrity, Confidentiality |
|  | RFC 7516 | JWE | TS 33.210 TS 33.310 TS 33.501 | asymmetric | Authentication, Integrity, Confidentiality |
|  | RFC 7518 | JOSE JWA | TS 33.210 TS 33.310 TS 33.501 | asymmetric | Authentication, Integrity, Confidentiality |

* * * End of Changes * * * *