3GPP TSG-SA3 Meeting #120	S3-250467

Athens, GR, 17 - 21 February 2025

Source:	Johns Hopkins University APL, Cisco Systems

Title:	Updates to Annex I.2 -- pCR to ACME-SBA living document

Document for:	Approval

Agenda item:	4.18

Spec:	3GPP TS 33.310

Version:	19.3.0

Work Item:	ACME_SBA

Comments

This document proposes updates to Annex I.2 to address an editorial error and to include support for ACME for NF certificate updates in TS 33.310.

Proposed Changes

* * * Start of Changes #1 * * * *

# 2	References

[XX]	IETF RFC 8555: "Automatic Certificate Management Environment (ACME)".

* * * End of Change #1* * * *

* * * Start of Changes #2 * * * *

## I.2	NF Certificate Updates

The normal procedure of update and renewal of 5GC NF certificates is managed by CMP protocol as described in clause 10.3.1. Automated Certificate Management Environment (ACME) [XX] protocol may optionally be supported, as described in Annex YY.

Nevertheless, the certificate management framework can be severely impacted by special critical circumstances, which can derive in simultaneous updates of vast number of certificates, causing a potential partial or complete disruption of the service. For example, a compromised security algorithm, the disclosure of broken cryptographic primitives, the revocation of CA root certificates or multiple certificates with same expiration data, are some of the special circumstances triggering the certificate update procedure.

This clause lists a few practical recommendations to be considered in NF certificate update procedure with the aim of mitigating potential issues or disruptions due to outages or overload situations. These recommendations can be deployed and implemented via internal configuration, operator policies and other mechanisms and functionalities in the operator PKI infrastructure, OAM systems, orchestration systems, etc.

-	The NF certificate updates can be configured in the operator PKI, and consequently the procedure can be initiated in advance before the certificate expiration time. For example, making use of different time interval/periodicity based on the NF type when configuring certificate update policies. Observe that the NF type is included in the certificates as per the profile in clause 6.1.3c and hence can be checked there while configuring such policies.

-	The operator PKI does not have to update the certificates with the same or similar expiration time simultaneously. Furthermore, the certificate update policies can take into consideration the expiration time and the triggers of the procedure being configured in advance. Certificate updates policies can be configured, for example in the operator PKI, to create different batches of certificates to be updated sequentially or with certain prioritization criteria.

-	Certificate expiry related alarms reported by network management systems, operator CA announcements for certificate revocations (e.g., via CRL, OCSP, etc.), and any other type of certificate related event, can be monitored with the purpose of mitigating the risk of service unavailability due to above mentioned special circumstances.

* * * End of Change #2* * * *