3GPP TSG-SA3 Meeting #120	S3-250468

Athens, GR, 17 - 21 February 2025

Source:	Johns Hopkins University APL, Cisco Systems, U.S. National Security Agency

Title:	pCR to ACME-SBA living document: certificate revocation

Document for:	Approval

Agenda item:	4.18

Spec:	3GPP TS 33.310

Version:	19.3.0

Work Item:	ACME_SBA

Comments

This document proposes guidance and procedures to support ACME-based certificate revocation in 5G SBA to be included as an informative annex in TS 33.310.

Proposed Changes

* * * Start of Changes #1 * * * *

2	References

[XX]	IETF RFC 8555: "Automatic Certificate Management Environment (ACME)".

* * * End of Change #1 * * * *

* * * Start of Changes #2 (All Next Text) * * * *

YY.6	Certificate revocation for 5GC NFs

### YY.6.1 Introduction

The ACME protocol [XX] supports an automated revocation of ACME enrolled and renewed certificates using established authenticated and authorized credentials (i.e., key pair) verified during ACME client account activation and certificate issuance. An end entity (e.g., ACME client in the NF) can use its account key pair or the key pair of the issued certificate to request revocation of its certificate by the CA (i.e., ACME server).

There are several practices associated with certificate revocation that are recommended.

- The ability to revoke certificates is limited to the original enrolling ACME client or if the ACME client has knowledge of the certificate private key.

- When the ACME client is co-located with the NF in 5G SBA, the ACME client should not have privileges to request certificate revocation for other NFs.

- The ACME client should maintain the valid account key pair for the NF Instance ID for which the certificate was issued and/or access to the key pair of the issued certificate being requested for revocation to properly sign the revocation request. Maintenance of the account key pair in a secure manner is left to implementation.

- The certificate being requested for revocation should not have expired. Since expired certificates can not be used, there is no justification to revoke expired certificates.

- If the NF has been compromised and the ACME client is co-located, access to the ACME client may not be possible. In such instances, certificate revocation would use existing server-side operator’s implementation.

NOTE: This client-side certificate revocation procedure does not impact existing CA or OAM initiated revocation mechanisms, which are based on operator’s implementation and outside the scope of this solution. The CA operator will continue to have the ability to revoke certificates that have been issued. In addition, production and distribution of the revocation status messages (i.e., via CRL or OCSP) of the revoked certificates are solely dependent on CA operator’s implementation.

### YY.6.2 ACME Certificate Revocation Procedure

Prerequisites of the ACME certificate revocation procedure:

- Initial trust has been established per Annex YY clause YY.2 (Initial trust)

- Certificate has been enrolled or renewed by the CA (i.e., ACME server) and has not expired

The revocation request contains the certificate to be revoked and, optionally, the reason code for certificate revocation. Valid revocation reason codes are defined in CRLReason in RFC 5280 [14].

NOTE: To deny or accept revocation requests based on which reasonCode is left to the operator’s implementation. Use of the reasonCode is strongly encouraged.

NOTE: RFC 8555 [XX] includes optional revocation reason codes, such as keyCompromise. These codes could provide an indication to the CA and further to the OAM in case that the CA is under control of the OAM.

To reduce potential impact of a DoS attack using compromised credentials, the CA should be configured to deny request for certificate revocation using ACME per Annex YY clause YY.3.

* * * End of Change #2 * * * *