3GPP TSG-SA3 Meeting #120	S3-250472

Athens, Greece, 17 - 21 February 2025

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.558 | CR | 0019 | rev | <> | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Requirements for secure retrieval of 5G system UE lds and privacy related information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGE_Ph3 |  |  |  |  |  | Date: |  |  | 2024-02-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The KI#2.1 of TR 33.749 related to the security retrieval of 5G system UE Ids and privacy related information was concluded with the agreement of a set of solution design principles to be followed in normative phase. This CR implements those principles as requirements specifically for the UE Identifier API defined in the context of the enchanced architecture for enabling edge applications. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New security requirements applied to the retrieval of 5G system UE Ids and privacy relate information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The usage of the UE Identifier API could compromise the security and privacy of the UE |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

# 5	Security requirements

## 5.1	General security requirements

The Edge application architecture defined in the TS 23.558 [5] shall satisfy the following requirements.

### 5.1.1	Authentication and authorization

Authentication and Authorization between Edge Enabler Client (EEC) and Edge Configuration Server (ECS): Edge Configuration Server (ECS) shall be able to provide mutual authentication with Edge Enabler Client (EEC) over EDGE-4 Interface. ECS shall determine whether EEC is authorized to access ECS's services.

Authentication and Authorization between EEC and EES: Edge Enabler Server (EES) shall provide mutual authentication with EEC over EDGE-1 Interface. EES shall determine whether EEC is authorized to access EES's services.

Authentication and Authorization between Edge Enabler Server (EES) and ECS: ECS shall provide mutual authentication with EES over EDGE-6 Interface. ECS shall determine whether EES is authorized to access ECS's services.

Authentication and Authorization between EESs: EES shall provide mutual authentication with another EES over EDGE-9 Interface. EES shall determine whether peer EES is authorized to access EES's services.

Authentication and Authorization in EES capability exposure to EAS: EES shall provide mutual authentication with EAS over EDGE-3 Interface. EES shall determine whether EAS is authorized to access EES's services and expose EEC Capabilities. The Edge application architecture shall support EASs to obtain the user's authorization to access sensitive information (e.g. user's location).

NOTE1: The corresponding security requirements defined in TS 23.558 [5] is AR-5.2.6.2-a/b/d/e/f/g.

Authentication and Authorization between Application Client (AC) and EEC: EEC should provide mutual authentication with the Application Client over EDGE-5 interface, and the EEC should determine whether Application client is authorized to access EEC’s service.

Authentication and Authorization between V-ECS and H-ECS：V-ECS shall provide mutual authentication with H-ECS over EDGE-10 Interface. V-ECS shall determine whether H-ECS is authorized to access V-ECS's services.

Authentication and Authorization between ECS and ECS-ER: ECS-ER shall provide mutual authentication with ECS over EDGE-10 interface. ECS-ER shall determine whether ECS is authorized to access ECS-ER's services.

Authentication and Authorization between ECS-ERs: ECS-ERs shall provide mutual authentication over EDGE-10 interface. ECS-ER shall determine whether other ECS-ERs are authorized to access ECS-ER's services.

### 5.1.2	Interface security

Confidentiality, integrity, and replay protection shall be supported on the EDGE-1-4 and EDGE 6-10 interfaces.

NOTE 1: The interfaces are defined in the Figure 6.2.4 of TS 23.558 [5]. The corresponding security requirement defined in TS 23.558 [5] is AR-5.2.6.2-c.

NOTE 2: The security requirement of EDGE 5 is out of the scope of this specification, since its details are out of the scope of this release of this specification, according to TS 23.558 [5].

The privacy requirements AR-5.2.6.2-h defined in TS 23.558 [5] are implicitly supported, since all the interfaces will be confidentiality and integrity protected.

### 5.1.3	User consent requirements

User consent for edge computing shall comply with TS 33.501 [3] (Annex V).

If EES, trusted by the 3GPP Core Network, is utilizing 5GC services without NEF, the EES acts as the consent enforcing entity. Otherwise, if the EES is utilizing 5GC services via NEF, the NEF acts as the consent enforcing entity.

User consent architecture in the present document is only applicable when EES or NEF and data provider are operated by the same entity.

### 5.1.X	Secure retrieval of 5G system UE lds and privacy related information

The information used in the procedures of calling UE identifier API in EES (Eees_UEIdentifier) shall not include permanent or semi-permanent UE identifiers.

The entity responsible for the generation of the information used in Eees_UEIdentifier API call invocation shall be part of the 5G system in HPLMN.  That information can be also generated by both the 5G System and the UE.

5G system in HPLMN shall verify the information used in Eees_UEIdentifier API call invocation.

NOTE 1: The API invoker may not need to obtain the permanent 5G UE ID, which improves the privacy.

* * * End of First Change * * * *