3GPP TSG-SA3 Meeting #120	S3-250475

Athens, GREECE, 17-21 February 2025

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.122 | CR | 0082 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Security requirements on CAPIF-8 reference points |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | CAPIF_Ph3_sec |  |  |  |  |  | Date: |  |  | 2025-02-21 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Till Rel-18 requirements on CAPIF-8/8e were out of scope. Now to be addressed with Rel-19 work item. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Requirements on CAPIF-8/8e |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete spec |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

******* START OF CHANGES

## 4.8	Security requirements on the CAPIF-8reference points

CAPIF-8 is the interface between the CCF and a client (ROF) supporting RO interaction.

The transport of messages over the CAPIF-8 reference point shall be integrity protected.

The transport of messages over the CAPIF-8 reference point shall be protected from replay attacks.

The transport of messages over the CAPIF-8 reference point shall be confidentiality protected.

Mutual authentication between the authorization server and the resource owner shall be supported.

CAPIF RNAA shall support to authorize the resource owner to provide resource owner authorization.

CAPIF RNAA shall support authorization of API invoker based on resource owner authorization and should support revocation of the resource owner authorization.

CAPIF RNAA shall support finer granular authorization and revocation when API invoker access resource(s) of the resource owner provided by the service API.

******* END OF CHANGES