3GPP TSG-SA3 Meeting #120	S3-250483

Athens, Greece, 17 - 21 February 2025

Title:	LS reply on Issues related to Analytics context transfer between AnLF(s)

Response to:	LS S2-2409441 on Issues related to Analytics context transfer between AnLF(s) from SA2

Release:	Rel-18

Work Item:	Enablers for Network Automation for 5G phase 3 (eNA_Ph3)

Source:	Nokia, to be SA3

To:	SA2

Cc:	-

Contact person:	German Peinado

german.peinado@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

SA3 would like to thank SA2 for their reply-LS on Issues related to Analytics context transfer between AnLF(s). Regarding the questions from SA2, SA3 has the following observation and answers:

Observation:

The following text in clause 6.1B.2.2 of TS 23.288 needs to be corrected:

“If the file address(es) of the trained ML Model(s) is provided and if the NWDAF containing MTLF is part of the locally configured set of NWDAFs containing MTLF, the target NWDAF may retrieve the ML Model using the file address of the trained ML Model”

The reasoning is that, in Clause X.10 procedure of TS 33.501, there is no independent static authorization mechanism proposed. That means, there is no way to fetch a ML model without an access token (steps 4a, 4b).

Question 1 from SA2: Can SA3 indicate if there are security issues in the scenario above?

SA3 has identified security issues, so SA3 kindly requests SA2 to not send the model address in analytics context transfer based on local information, and follow the procedure as defined in the clause X.10 of TS 33.501.

Question 2 from SA2: For scenarios other than the above, when the source NWDAF contains both AnLF and MTLF, is there any security issue for the source NWDAF to provide the file address(es) of the trained ML Model(s) to the target NWDAF?.

Same answer provided for Question 1 applies here, regardless the deployment scenario.

# 2	Actions

To SA WG2

ACTION: 	SA3 kindly asks SA2 to correct the text in 6.1B.2.2, and accordingly not to send the model address in analytics context transfer based on local information.

# 3	Dates of next TSG SA WG 3 meetings

SA3#121	7 – 11 April 2025		Goteborg, Sweden

SA3#122	19 - 23 May 2025		Fukuoka, Japan