3GPP TSG-SA3 Meeting #120	S3-250694

# Athens, Greece, 17 - 21 February 2025

Source:	Huawei, HiSilicon

# Title:	Content to clause 4.X

Document for:	Approval

Agenda item:	4.16

Spec:	TS 33.545

Version:	0.0.0

Work Item:	5G_Femto_Sec

Comments

This contribution proposes to add new content to clause 4.X.

* * First Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[A]	3GPP TS 38.300: "NR; NR and NG-RAN Overall Description".

[B]	3GPP TS 23.501: "System Architecture for the 5G System".

[C]	3GPP TS 33.320: "Security of Home Node B (HNB) / Home evolved Node B (HeNB)".

[D]	3GPP TS 32.593: "Telecommunication management; Procedure flows for Type 1 interface H(e)NB to H(e)NB Management System (H(e)MS) ".

* * * Next Change * * * *

4.X	Security Architecture of NR Femto

In 5GS, an NR Femto node connects to 5GC directly via NG interface or optionally connects to 5GC via NR Femto Gateway (NR Femto GW) as specified in TS 38.300 [A] and in Annex V of TS 23.501[B]. Additionally, other functions that will be used for security purpose are further depicted in Figure 4.X.1.

Figure 4.X.1: Security Architecture of NR Femto

The system architecture is described in TS 33.320[C] with the following modifications:

- Replace the H(e)NB with the NR Femto.

- Replace the HNB-GW with the NR Femto GW.

- Replace the H(e)MS and HMS with the NR Femto Management System.

- L-GW is replaced with the UPF that is not drawn in Figure 4.X.1, but can be seen in Annex V of TS 23.501[B].

Editor’s Note: whether AAA needs to be supported is FFS.

Editor’s: whether AUSF is needed is FFS.

* * * End of Changes * * * *