3GPP TSG-SA3 Meeting #120	S3-250764

Athens, Greece, 17 - 21 February 2025	(revision of xx-yyxxxx)

# Source:	China Mobile

# Title:	New SID on enhanced security management service about security policy provisioning

# Document for:	Approval

# Agenda Item:	6

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: Study on enhanced security management service about security policy provisioning

Acronym:	FS_SPE(TBD)

{Propose an acronym. Final acronym to be confirmed at the plenary. The sign "-" is a level separator between (Feature)-(Building Block)-(Work Task). The sign "_" can be freely used. Studies have to start by "FS_". Each acronym level has to be simple and short, 7 characters max recommended}

Unique identifier:	(TBD)

{A number to be provided by MCC at the plenary}

Potential target Release:	Rel-20

1	Impacts

{For Normative work, identify the anticipated impacts. For a Study, identify the scope of the study}

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | X |  |
| No | X | X | X |  |  |
| Don't know |  |  |  |  |  |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a …

{Tick one or more box(es). The full structure of all existing Work Items is shown in the 3GPP Work Plan in https://ftp.3gpp.org/Information/WORK_PLAN}

| X | Study |
| --- | --- |
|  | Normative – Stage 1 |
|  | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| N/A | N/A | N/A | N/A |

2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| N/A | N/A | N/A |

3	Justification

- Current network security are mainly focus on how to protect connections between NFs, but lack of consideration on how to provision network security policies to execute such security procedures. It is important to utilize such ability to network operator to improve the security protection on networks.

- There are some cases needs to be considered to enhance the security management service about security policy provisioning.

- On-demand Security Enabler to Protect 3GPP NFs

- ToB service is crucial for mobile operators. Some 3GPP NFs in 5G have been deployed in the ToB dedicated network, e.g. PLMN NPN.

- Due to the differentiated security requirements of enterprises across various industries, the operators need to be able to deploy the security capabilities on-demand to protect 3GPP NFs. Some of them are related to NFs, e.g. the interfaces, OS.

- Security Configuration Update

- When some security attacks are detected, e.g. from compromised NF(s), the network needs to update the security configuration on other NFs to reject communication with such compromised NF in order to mitigate attacks. And it would be also necessary to update back when such compromised NF recovered.

- Security Incident Response Synchronization

- The security operation of ToB network and ToC mobile network in one operator are largely independent from each other. However, the connectivity between ToB and ToC networks is still possible. It provides a potential path for the attacker to implement lateral movement between them.

- When some suspicious attacks (OS configuration changes, abnormal processes, etc.) are detected in one sub-network(ToB or ToC), which result a security response execute in the network, such response also needs to be synchronized to other sub-networks via the security collaboration mechanism, so that the mobile operators can prevent potential attacks. For example, if there is an suspicious attack happened in a sub-network, it may needs to block NFs in such sub-network to visit NFs in other sub-networks temporarily.

- Security Policy Consistency Co-operation

Some security attacks (e.g., GTP Door) target the 3GPP NFs. Mitigating the attacks requires cooperation and coordination among 3GPP NFs on multi-stakeholder cloud.

The general security response policy can be decomposed into multiple different sub-policies. The NFs on multiple clouds need to be coordinated to avoid a lack of security response policy or insufficient, isolated security policies, as these can hinder the overall effectiveness of attack protection.

So based on above cases, it is necessary to study the security requirements about security policy execution and how to execute. Furthermore, it also needs to study which kind of security policy should be impact.

4	Objective

It is proposes to study:

WT1:

- the use cases about security policy provisioning management

- whether additional security requirements about security policy provisioning management is needed

  - if yes, how security requirements are defined.

- WT2:

- which kind of security policy should be impact.

TU estimates and dependencies

| Work Task ID | TU Estimate (Study) | TU Estimate (Normative) | RAN Dependency (Yes/No/Maybe) |
| --- | --- | --- | --- |
| WT1: Security requirements | 2 | 0.5 | No |
| WT2: Impact security policy | 3 | 0.5 | No |

Total TU estimates for the study phase:   5

Total TU estimates for the normative phase:  1

Total TU estimates: 6

5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| Internal TR | 33.abc | Study on enhanced security management service about security policy execution | SA#110 (December 2025) | SA#112 (June 2026) | TBD |

6	Work item Rapporteur(s)

TBD

7	Work item leadership

SA3

8	Aspects that involve other WGs

N/A

9	Supporting Individual Members

Supporting IM name

China Mobile