3GPP TSG-SA3 Meeting #120	S3-250811

Athens, Greece, 17 - 21 February 2025

Title:	Draft_LS on Checking PLMNID of NFc in interconnect scenario

Response to:

Release:	Release-16

Work Item:

Source:	Ericsson to be SA3

To:	CT4

Cc:	CT3

Contact person:	mohsin.a.khan@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	S3-24xxxx, S3-24xxxx, S3-24xxxx

# 1	Overall description

Rel-15 work in SA3 and CT4:

SA3 identified a security requirement, described in an LS sent to CT3 and CT4 (S3-183789), to prevent an attack by an NF of one network impersonating an NF of another network:

"It is necessary to enable the receiving SEPP to reliably verify the source of the request and response message, before forwarding the message including the PLMN information to the target NF for the subsequent authorization verification procedure."

To fulfill the security requirement, SA3 proposed, in a subsequent LS to CT3 and CT4 (S3-190559), including PLMN ID of NF consumer in the access token in plaintext:

"It is proposed to enable verification of the PLMN-ID of the NF consumer by including the consumer's PLMN-ID unencrypted into the access token specified in TS 29.510, e.g. as part of existing claims."

In Rel-15, as a result of the above LSes, CT4 added NF consumer's PLMN ID in the Access Token Claims (CR C4-191528).

Rel-16 work in CT4:

In Rel-16, to consistently define authorization parameters for different services/procedures in the NRF, in the Access Token Request message, CT4 included a list of PLMN IDs instead of a single PLMN ID when NF supports multiple PLMN IDs (CR C4-203256).

Issues with the Rel-15/16 work:

However, access tokens can include only one PLMN ID of the NF consumer. In the current specification, it is unclear how the NRF chooses one PLMN ID of the NF consumer to provide in the access token claim when it receives an access token request with a list of PLMN IDs of the NF consumer.

Therefore, it may happen that the NF consumer obtains an access token from the NRF with a PLMN ID of the NF consumer that does not match with the PLMN ID that the NF consumer intends to use in the service request API message sent to the target NF producer. Also, the NF consumer may send a service request by including a PLMN ID in the API message that does not match with the PLMN ID in the associated access token — causing the target NF producer rejecting the service request.

Resolution in SA3 from Rel-17 onwards:

SA3 has agreed on the attached CRs to address the problem in Rel-17, Rel-18, and Rel-19.

Complete solution requires CT4 input:

However, it is not clear how to address the problem in Rel-16.

# 2	Actions

To CT4

ACTION:

1) SA3 kindly asks CT4 to take the above observations and the attached agreed SA3 CRs into account.

2) SA3 kindly asks CT4 to consider solutions under the CT4 scope.

# 3	Dates of next TSG SA WG 3 meetings

SA3#121	7 – 11 April 2025		Goteborg, Sweden

SA3#122	19 - 23 May 2025		Fukuoka, Japan