3GPP TSG-SA3 Meeting #120	S3-250844

Athens, Greece, 17 - 21 February 2025

Title:	[draft] Reply LS on security handling for inter-CU LTM in non-DC cases

Response to:	Reply LS S3-250325/R2-2411136 on security handling for inter-CU LTM in non-DC cases from RAN2

Release:	Rel-19

Work Item:	NR_Mob_Ph4-Core

Source:	Xiaomi [to be SA3]

To:	RAN2

Cc:

Contact person:	Wei Lu

luwei10(at)xiaomi(dot)com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1	Overall description

SA3 thanks RAN2 for the reply LS (R2-2411136) on security handling for inter-CU LTM in non-DC cases.

Based on the response from RAN2, SA3 revisited all the solutions for key handling in inter-CU LTM scenario and would like to provide the following feedback:

- For AS security parameter not changing per inter-CU LTM handover (i.e. UE’s 5G security capabilities, selected AS security algorithms, UE’s UP security policy, UE’s UP security activation status):

  - These parameters are synchronized via LTM candidate preparation procedure.

  - When the gNB receives mismatched UE security context (e.g. UE’s 5G security capabilities and/or UE’s UP security policy) in the Path-Switch Acknowledge message after LTM switch, subsequent inter-CU LTM switch will be broken off and turn to an initial inter-CU LTM, because:

    - intra-cell handover procedure needs to be initiated by the gNB towards the UE.

    - UE’s 5G security capabilities and/or UE’s UP security policy for the active PDU session need to be updated by the gNB to all candidate gNBs.

- For AS security parameters changing per inter-CU LTM handover (i.e. NCC, keySetChangeIndicator):

  - These parameters are synchronized via cell switch command MAC CE. NCC value and keySetChangeIndicator value in MAC CE can be encoded against being tampered.

2	Actions

To CT1 and CT6

ACTION: 	SA3 kindly asks RAN2 to take the above feedback into account.

3	Dates of next TSG SA WG 3 meetings

SA3#121	7 – 11 April 2025		Goteborg, Sweden

SA3#122	19 - 23 May 2025		Fukuoka, Japan