3GPP TSG-SA3 Meeting #120	draft_S3-251005-r1

# Athens, Greece, 17 - 21 February 2025                               merger of S3-250863 and S3-250913

Source:	Xiaomi, Nokia

Title:	Pseudo-CR on Resolve ENs in TR 33.700-22 Solution #19

Document for:	Approval

Agenda item:	5.19

Spec:	3GPP TR 33.700-22

Version:	0.5.0

Work Item:	FS_CAPIF_Ph3-sec

Comments

This contribution proposes to resolve ENs in TR 33.700-22 Solution #19.

For SA6 alignment issue, a statement is added to clarify that this solution reuses service API discovery procedure defined in clause 8.25.3.3 of TS 23.222. Then the following ENs are resolved.

Editor’s Note: The alignment with SA6 is FFS.

Editor’s Note: The alignment with clause 8.25.3.2 and 8.25.3.3 in TS 23.222 is FFS.

Moreover, a statement is added to clarify that the authorization related request/response in this solution can be the enhancement of existing access token request/response defined in clause 6.5.2.3 of TS 33.122. Then the following EN is resolved.

Editor’s Note: whether to enhance the Access token request specified in C.3.2 in TS 33.122 or defind new authorization request is FFS.

Considering a comprehensive evaluation has been derived for this solution, the following EN can also be removed.

Editor’s Note: Further evaluation is FFS.

* * * First Change * * * *

6.19	Solution #19: API invoker authorization mechanism in CAPIF interconnection scenarios

6.19.1	Introduction

This solution addresses part of KI#2 (i.e., The CAPIF should support authorization of the API invoker in CAPIF interconnection scenarios).

In this solution, if the requested AEF is in the domain corresponding to the CCF-B while the API invoker 's onboarding is completed in CCF-A, CCF-B is used to generate token for the API invoker.

6.19.2	Solution details

Pre-conditions:

API invoker's onboarding is completed in CCF-A

The CCF-B is connected with the AEF

The authorization related request in this solution can be the enhancement of OAuth 2.0 based access token request defined in clause 6.5.2.3 of TS 33.122[4].

The authorization related response in this solution can be the enhancement of access token response defined in clause 6.5.2.3 of TS 33.122[4].

This solution reuses the service API discovery procedure defined in clause 8.25.3.3 of TS 23.222 [2]

Figure 6.19.2 API invoker authorization mechanism in CAPIF interconnection scenarios

0.	API invoker and CCF-A have established the TLS tunnel and completed the mutual authentication.

1.	API invoker sends the service API discovery related request to the CCF-A.

2.	CCF-A determines to send the service API discovery request to CCF-B based on local policy (e.g., A specific category of API needs to send the request to CCF-B).

3.	CCF-B sends the discovered service API information to the CCF-A.

4.	CCF-A sends the discovered service API information to the API invoker ID.

5.	API invoker sends the authorization related request to the CCF-A. The request also includes the expected resource owner ID, expected service/service operation/service API, date type, and the corresponding data processing purpose.

6.	To check if the authorization related request should be sent to the CCF-B, the following rules apply.

-	If the CCF-A finds that the expected service/service operation/service API cannot be provided by AEF in its domain and finds that the expected service/service operation matches the discovered service API from CCA-B received in step 3, the CCF-A sends the authorization related request to the CCF-B.

-	If the CCF-A finds that the expected service/service operation/service API is previously published by CCF-B, the CCF-A sends the authorization related request to the CCF-B.

The CCF-A may use the date type and the corresponding data processing purpose in addition to the expected service/service operation to identify the matched service API. Then the CCF-A can determine how to handle the authorization related request.

The request sent to CCF-B also includes the API invoker ID, expected resource owner ID, expected service/service API/service operation, date type, and the corresponding data processing purpose. Based on the trusted business relationship between CCF-A and CCF-B, the CCF-B trusts that the API invoker indicated by the API invoker ID is already authenticated and onboarded in the CCF-A.

7.	If API invoker is authorized, CCF-B sends the token to the CCF-A. The token includes the API invoker ID, expected resource owner ID, expected service/service operation, date type, and the corresponding data processing purpose.

8.	The CCF-A sends the token to the API invoker.

9.	The API invoker authenticates to the AEF by establishing a TLS session with the AEF.

10.	With successful authentication to the AEF, the API invoker shall initiate invocation of a 3GPP northbound API with the AEF by sending service operation request with the requested date type, and the corresponding data processing purpose. The access token received from the CCF-B is sent along with the northbound API invocation request as per OAuth 2.0.

11.	The AEF validates the access token using CCF-B certificate or a key pre-shared with CCF-B.

12.	After successful verification of the access token of the API invoker, the requested northbound API is invoked and the appropriate response shall be returned to the API invoker.

6.19.3	Evaluation

This solution addresses part of KI#2 (i.e., The CAPIF should support authorization of the API invoker in CAPIF interconnection scenarios).

The benefit of this solution is that it realizes API invoker authorization via enhancing existing client credentials flow.

Impacts to CCF-A:

If the CCF-A finds that the expected service/service operation/service API cannot be provided by AEF in its domain and finds that the expected service/service operation matches the discovered service API from CCA-B, the CCF-A sends the authorization related request to the CCF-B.

If the CCF-A finds that the expected service/service operation/service API is previously published by CCF-B, the CCF-A sends the authorization related request to the CCF-B.

Impacts to CCF-B:

CCF-B is required to receive authorization request from the CCF-A.

CCF-B is required to send the token to the API invoker via the CCF-A.

* * * End of Change * * * *