3GPP TSG-SA3 Meeting #122	S3-251938

# Fukuoka, Japan,  19 - 23 May 2025

Source:	ZTE

Title:	Update conclusion for KI#1

Document for:	Approval

Agenda item:	4.12

Spec:

Version:

Work Item:	Security aspects of NR mobility enhancement Phase 4

Comments

This pCR proposes to update the conclusion for KI#1. According to the response from RAN3 (R3-252449), the new source gNB needs to provide the new KgNB*(s) to the corresponding candidate gNB via a LTM Configuration Update procedure. Therefore, it is proposed to conclude on KI#1 based on this principle.

Proposed Changes

* * * First Change * * * *

5	Conclusions

5.1	Conclusion for key issue #1

For the case where CU is acting as MN and DC is not configured, it is agreed that the normative work is based on the following principles:

-	In LTM cell switch execution phase, the NCC value is included in the LTM cell switch command  MAC CE.

-	In subsequent LTM phase, the new source gNB provides the new KgNB* to the corresponding candidate gNB via a LTM Configuration Update procedure.

-	For the parameters in AS security context, which are not changing per inter-CU LTM handover (including UE’s 5G security capabilities, selected AS security algorithms, UE’s UP security policy, UE’s UP security activation status):

-	If the gNB receives mismatched UE security context (e.g. UE’s 5G security capabilities and/or UE’s UP security policy) in the Path-Switch Acknowledge message after LTM switch,

-	it initiates an intra-cell handover procedure which includes RRC Connection Reconfiguration procedure indicating the change of selected algorithms (as defined in clause 6.7.3.1) and/or UE’s UP activation status (as defined in clause 6.6.1) to the UE.

-	it updates the UE’s 5G security capabilities and/or UE’s UP security policy for the active PDU session to all candidate gNBs.

* * * End of Changes * * * *