3GPP TSG-SA3 Meeting #122	S3-252000

Fukuoka, Japan, 19 - 23 May 2025

# Source:	Huawei, HiSilicon

# Title:	Discussion paper for certificate lifecycle management

# Document for:	Discussion

Agenda Item:	4.1.15

1	Decision/action requested

It is proposed to discuss the certificate lifecycle management for SBA.

2	References

[1]	3GPP TS 33.310: "Network Domain Security (NDS); Authentication Framework (AF)".

3	Rationale

3.1 Certificate lifecycle management for SBA

In Release 18, SA3 specified certificate management procedures in SBA for 5GC NFs in TS 33.310 [1]. In addition to the certificate enrollment, update and revocation procedures, the certificate lifecycle management has also been considered and informative guidance to implementations was also documented in the TS.

As an example, “during the NF discovery procedure, the NRF may check that the potential producers, to be included in the response, do have valid certificates”. How such a check is performed is left to implementation, which may cause some confusion for implementers who want to implement this feature.

3.2	Potential issues for certificate validation check implementation

How to do the certificate validation check has mature technologies, for example, checking the validity and the signature carried in the certificate and checking whether the certificate is revoked or not based on the CRL or OCSP mechanisms. However, NRF may not hold the NF producers’ certificates during the NF discovery procedure, and cannot do the certificate validation check. Implementers may need to design solutions to check the validity of certificates.

In fact, the most straightforward approach is to make the NF certificate as an optional profile for NF registration. Since the certificates are registered to the NRF during the NF Service Registration procedure, the NRF can check the potential NF producers’ certificate straightly durint the NF discovery procedure.

As another option, the NRF can subscribe NF certificate status from other network functions like CMNE or OCSP server, in that case, the NRF may keep an extra optional NF certificate status profile.

4	Detailed proposal

Based on the rationale above, clarification on certificate life cycle management may need to be added. An associated CR S3-251999 has been submitted.