3GPP TSG-SA3 Meeting #122	S3-252321

Fukuoka, Japan, 19 – 23 May 2025

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.216 | CR | 0033 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clean up and correction of test names in 33.216 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | BSI (DE) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SCAS_5G_Maint |  |  |  |  |  | Date: |  |  | 2025-05-20 |
|  | D |  |  |  |  |  |  |  |  |  |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Inconsistent test naming, some format glitches and typos |  |  |  |  |  |  |  |  |
| Summary of change: |  | Test names, format glitches and typos corrected |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Non-standard test naming, formats and typos remain and test automation may be affected |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.2.1.5, 4.2.2.1.6, 4.2.2.1.7, 4.2.2.1.8, 4.2.2.1.9, 4.2.2.1.10, 4.2.2.1.11, 4.2.2.1.12, 4.2.2.1.14, 4.2.2.1.18, 4.2.2.1.19, 4.2.2.1.20 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

********** START OF 1st CHANGE **********

##### 4.2.2.1.5	AS algorithms selection

Requirement Name: AS algorithms selection

Requirement Reference: TS 33.401 [3], clause 7.2.4.1; TS 33.401 [3], clause 7.2.4.2.1

Requirement Description: "The serving network shall select the algorithms to use dependent on: the UE security capabilities of the UE, and the configured allowed list of security capabilities of the currently serving network entity." as specified in TS 33.401 [3], clause 7.2.4.1".

"Each eNB shall be configured via network management with lists of algorithms which are allowed for usage. There shall be one list for integrity algorithms, and one for ciphering algorithms. These lists shall be ordered according to a priority decided by the operator." as specified in TS 33.401 [3], clause 7.2.4.2.1.

Threat References: TBA

Test Case:

Test Name: TC_ENB_AS_ALGO_SELECTION

Purpose:

Verify that the eNB selects the algorithms with the highest priority in its configured list.

Pre-Conditions:

Test environment with the eNB has been pre-configured with allowed security algorithms with priority.

Execution Steps

1)	The UE sends attach request message to the eNB.

2)	The eNB receives S1 context setup request message.

3)	The eNB sends the SECURITY MODE COMMAND message.

4)	The UE replies with the AS SECURITY MODE COMPLETE message.

Expected Results:

The eNB initiates the SECURITY MODE COMMAND message that includes the chosen algorithm with the highest priority according to the ordered lists and is contained in the UE EPS security capabilities.

The MAC in the AS SECURITY MODE COMPLETE message is verified, and the AS protection algorithms are selected and applied correctly.

Expected format of evidence:

Sample copies of the log files.

********** END OF 1st CHANGE **********

********** START OF 2nd CHANGE **********

##### 4.2.2.1.6	Verify RRC integrity protection

Requirement Name: The check of RRC integrity

Requirement Reference: TS 33.401 [3], clause 7.4.1

Requirement Description: " The supervision of failed RRC integrity checks shall be performed both in the ME and the eNB. In case of failed integrity check (i.e. faulty or missing MAC-I) is detected after the start of integrity protection, the concerned message shall be discarded. " as specified in TS 33.401 [3], clause 7.4.1.

Security Objective References: TBA

Test Case:

Test Name: TC_ENB_RRC_INTEGRITY_PROTECTION

Purpose:

Verify that the message is discarded in case of failed integrity check (i.e. faulty or missing MAC-I).

Pre-Conditions:

Test environment with RRC Protection is activated at the eNB.

Execution Steps

Positive:

The eNB receives a RRC message with a right MAC-I.

Negative:

The eNB receives a RRC message with a wrong MAC-I or missing MAC-I.

Expected Results:

The RRC message is discarded in the negative test.

Expected format of evidence:

Sample copies of the log files.

********** END OF 2nd CHANGE **********

********** START OF 3rd CHANGE **********

##### 4.2.2.1.7	The selection of EIA0

Requirement Name: The selection of EIA0

Requirement Reference: TS 33.401 [3], clause 5.1.4.2

Requirement Description: " EIA0 is only allowed for unauthenticated emergency calls " as specified in TS 33.401 [3], clause 5.1.4.2.

Threat References: TBA

Test Case:

Test Name: TC_ENB_SELECTION_OF_EIA0

Purpose:

Verify that AS NULL integrity algorithm is used correctly.

Pre-Conditions:

Test environment with a UE. The UE may be simulated.

The vendor shall provide documentation describing how EIA0 is disabled or enabled.

Execution Steps

Positive:

1)	The eNB receives a UE security capability only containing EIA0 from S1 context setup message.

2)	The eNB sends AS SMC to the UE.

Negative:

1)	The eNB receives a UE security capability that contains EIA0 and other integrity algorithm(s).

2)	The eNB sends AS SMC to the UE.

Expected Results:

EIA0 is only selected in the Positive test.

Expected format of evidence:

Sample copies of the log files.

********** END OF 3rd CHANGE **********

********** START OF 4th CHANGE **********

##### 4.2.2.1.8	Key refresh at the eNB

Requirement Name: Key refresh at the eNB

Requirement Reference: TS 33.401 [3], clause 7.2.9.1; TS 36.331 [5], clause 5.3.1.2.

Requirement Description: "Key refresh shall be possible for KeNB, KRRC-enc, KRRC-int, KUP-int, and KUP-enc and shall be initiated by the eNB when a PDCP COUNTs is about to be re-used with the same Radio Bearer identity and with the same KeNB. " as specified in TS 33.401 [3], clause 7.2.9.1.

Moreover, "The eNB is responsible for avoiding reuse of the COUNT with the same RB identity and with the same KeNB, e.g. due to the transfer of large volumes of data, release and establishment of new RBs. In order to avoid such re-use, the eNB may e.g. use different RB identities for successive RB establishments, trigger an intra cell handover or by triggering a transition from RRC_CONNECTED to RRC_IDLE or RRC_INACTIVE and then back to RRC_CONNECTED." as specified in TS 36.331 [x], clause 5.3.1.2.

Threat References: TR 33.926[4], clause C.2.3.1 – Key reuse for eavesdropping

Test Case 1:

Test Name: TC_ENB_KEY_REFRESH_ PDCP_COUNT

Purpose:

Verify that the eNB performs KeNB refresh when PDCP COUNTs are about to wrap around.

Pre-Conditions:

The UE may be simulated.

Execution Steps

1)	The eNB sends AS Security Mode Command message to the UE, and the UE responds with the AS Security Mode Complete message.

2)	The UE sends RRC messages or UP messages to the eNB with an increasing PDCP COUNT until the value wraps around.

Expected Results:

The eNB triggers an intra-cell handover and takes a new KeNB into use.

Expected format of evidence:

Part of log that shows the PDCP COUNT wrapping around and the intra-cell handover. This part can be presented, for example as a screenshot.

Test Case 2:

Test Name: TC_ENB_KEY_REFRESH_DRB_ID

Purpose:

Verify that the eNB performs KeNB refresh when DRB-IDs are about to be reused under the following conditions:

-	the successive Radio Bearer establishment uses the same RB identity while the PDCP COUNT is reset to 0, or

-	the PDCP COUNT is reset to 0 but the RB identity is increased after multiple calls and wraps around.

Pre-Conditions:

The UE and MME may be simulated.

Execution Steps

1)	The eNB sends the AS Security Mode Command message to the UE.

2)		The UE responds with the AS Security Mode Complete message.

3)		A DRB is set up.

4)		DRB is set up and torn down for multiple times within one active radio connection without the UE going to idle (e.g. by the UE making multiple IMS calls, or by the MME requesting bearer setup and bearer deactivation), until the DRB ID is reused.

Expected Results:

Before DRB ID reuse, the eNB takes a new KeNB into use by e.g. triggering an intra-cell handover or triggering a transition from RRC_CONNECTED to RRC_IDLE or RRC_INACTIVE and then back to RRC_CONNECTED.

Expected format of evidence:

Part of log that shows all the DRB identities and the intra-cell handover or the transition from RRC_CONNECTED to RRC_IDLE or RRC_INACTIVE and then back to RRC_CONNECTED. This part can be presented, for example, as a screenshot.

********** END OF 4th CHANGE **********

********** START OF 5th CHANGE **********

##### 4.2.2.1.9	AS Security Mode Command Procedure

Requirement Name: AS integrity algorithm selection

Requirement Reference: TS 33.401 [3], clause 7.4.2

Requirement Description: The eNB shall protect the SECURITY MODE COMMAND message with the integrity algorithm, which has the highest priority according to the ordered lists.

Threat References: TBA

Test Case:

Test Name: TC_ENB_AS_SMC_INTEGRITY_PROTECTION

Purpose:

Verify that AS integrity protection algorithm is selected and applied correctly.

Pre-Conditions:

Test environment with UE. UE may be simulated.

Execution Steps:

The eNB sends the SECURITY MODE COMMAND message. The UE replies with the SECURITY MODE COMPLETE message.

Expected Results:

1.	The eNB has selected the integrity algorithm which has the highest priority according to the ordered lists and is contained in the UE EPS security capabilities. The eNB checks the message authentication code on the SECURITY MODE COMPLETE message.

2.	The MAC in the SECURITY MODE COMPLETE is verified, and the AS integrity protection algorithm is selected and applied correctly.

Expected format of evidence:

Snapshots containing the result.

********** END OF 5th CHANGE **********

********** START OF 6th CHANGE **********

##### 4.2.2.1.10	Bidding down prevention in X2-handovers

Requirement Reference: TS 33.401 [3], clause 7.2.4.2.2

Requirement Description: "In the path-switch message, the target eNB shall send the UE EPS security capabilities received from the source eNB to the MME." as specified in TS 33.401 [3], clause 7.2.4.2.2."

Threat References: TBA

Test Case:

Test Name: TC_ENB_BIDDING_DOWN_X2_HANDOVER

Purpose:

Verify that bidding down is prevented in X2-handovers.

Pre-Conditions:

Test environment with source eNB and target eNB, and the source eNB may be simulated.

Execution Steps:

The target eNB sends the path-switch message to the MME.

Expected Results:

The UE EPS security capabilities are in the path-switch message.

Expected format of evidence:

Snapshots containing the result

********** END OF 6th CHANGE **********

********** START OF 7th CHANGE **********

##### 4.2.2.1.11	AS protection algorithm selection in eNB change

Requirement Name: AS protection algorithm selection in eNB change.

Requirement Reference: TS 33.401 [3], clause 7.2.4.2.2, and clause 7.2.4.2.3

Requirement Description: "The target eNB shall select the algorithm with highest priority from the UE EPS security capabilities according to the prioritized locally configured list of algorithms (this applies for both integrity and ciphering algorithms). The chosen algorithms shall be indicated to the UE in the handover command if the target eNB selects different algorithms compared to the source eNB" as specified in TS 33.401 [3], clause 7.2.4.2.2, and clause 7.2.4.2.3.

Threat References: TBA

Test Case:

Test Name: TC_ENB_AS_ALGO_SELECTION_ENB_CHANGE

Purpose:

Verify that AS protection algorithm is selected correctly.

Pre-Conditions:

Test environment with source eNB, target eNB and MME. Source eNB and MME may be simulated.

Execution Steps:

Test Case 1:

Source eNB transfers the ciphering and integrity algorithms used in the source cell to the target eNB in the handover request message.

Target eNB verifies the algorithms and selects AS algorithms which have the highest priority according to the ordered lists. Target eNB includes the algorithm in the handover command.

Test Case 2:

MME sends the UE EPS security capability to the Target eNB.

The target eNB selects the AS algorithms which have the highest priority according to the ordered lists in the HANDOVER COMMAND.

The above test cases assume that the algorithms selected by the target eNB are different from the ones received from the source eNB.

Expected Results:

For both test cases:

1.	The UE checks the message authentication code on the handover command message.

2.	The MAC in the handover complete message is verified, and the AS integrity protection algorithm is selected and applied correctly.

Expected format of evidence:

Snapshots containing the result.

********** END OF 7th CHANGE **********

********** START OF 8th CHANGE **********

##### 4.2.2.1.12	RRC and UP downlink ciphering at the eNB

Requirement Name: RRC and UP downlink ciphering at the eNB.

Requirement Reference: TS 33.401 [3], clause 7.2.4.5

Requirement Description: "The eNB shall start RRC and UP downlink ciphering after sending the AS security mode command message" .

Threat References: TBA

Test Case:

Test Name: TC_eNB_DL_Cipher

Purpose: To verify that the eNB performs RRC and UP downlink ciphering after sending the AS security mode command message.

Pre-Condition:

-	The UE and eNB network products are connected in the test environment. UE may be simulated.

-	The tester shall have access to the AS security context and the corresponding cryptographic keys (e.g. RRC and UP cipher keys).

-	The tester has access to Uu interface and ability to capture the Uu interface messages with the debug port enabled in the UE.

Execution Steps:

1)	The tester shall POWER ON the UE to trigger the registration procedures (Attach and SMC).

2)	The tester performs packet capturing over the Uu interface using any packet analyser.

3)	The tester filters the AS SMC command message and the following RRC and UP downlink packets from eNB to UE.

4)	The tester proceeds the testing based on the parameters (algorithm identifier and algorithm distinguisher) present in the AS SMC command message.

Case 1: If the parameters refer to null ciphering algorithm, the tester verifies that the downlink packets filtered in step 3 are unciphered.

Case 2: If the parameters refer to algorithms such as SNOW, AES, ZUC, the tester verifies that the downlink packets filtered in step 3 are ciphered.

5)	The tester also checks if the packets are ciphered in accordance with the selected algorithm stated in the AS SMC command message.

NOTE:	The requirement mentioned in this clause is tested in accordance with the procedure mentioned in clause 4.2.3.2.4 of TS 33.117 [2].

Expected Results:

The downlink packets following the AS SMC command message are ciphered except NULL ciphering algorithm case.

Expected format of evidence:

Evidence suitable for the interface, e.g. Screenshot contains the operation results.

********** END OF 8th CHANGE **********

********** START OF 9th CHANGE **********

##### 4.2.2.1.14	UE NR security capability is only sent to a SgNB

Requirement Name: UE NR security capability is only sent to a SgNB

Requirement Reference: TS 33.401 [3], clause E.3.4.3

Requirement Description: "When adding SgNB while establishing an EN-DC connection, the MeNB shall send these created UE NR security capabilities to the SgNB. Other than for adding an SgNB, the created UE NR security capabilities shall not be sent from the MeNB." as specified in TS 33.401 [3], clause E.3.4.3.

Threat References: TBA

Test Case:

Test Name: TC_NR_SEC_CAP_SENT

Purpose: To verify that the UE NR security capabilities are only sent to a SgNB.

Pre-Condition:

-	The UE, gNB and eNB network products are connected in the test environment. UE and gNB may be simulated.

-	The tester has access to the X2 interface.

Execution Steps:

1)	The tester triggers MeNB to send SN addition Request message to a SgNB.

2)	The tester triggers UE HO from MeNB to another eNB.

3)	The tester checks if the UE NR security capabilities were sent in the X2 interface in both step 1 and step 2.

Expected Results:

The UE NR security capabilities are only sent to the SgNB.

Expected format of evidence:

Evidence suitable for the interface, e.g. Screenshot contains the operation results.

********** END OF 9th CHANGE **********

********** START OF 10th CHANGE **********

##### 4.2.2.1.18	UP IP policy selection

Requirement Name: Select the right UP IP policy.

Requirement Reference: TS 33.401 [2] clause 7.3.3

Requirement Description: " If the eNB receives UP integrity protection policy from the MME, the eNB shall use the received UP integrity protection policy, otherwise, the eNB shall use the locally configured UP integrity protection policy if EIA7 in the EPS security capability indicates that the UE supports user plane integrity protection with EPC. " in clause 7.3.3

Threat References: TBD

Test Case:

Test Name: TC_UP_IP_POLICY_Selection

Purpose: To verify that the UP IP policy sent from the MME is used by the eNB.

Pre-Condition:

-	The eNB network product is connected in emulated/real network environments. UE and MME may be simulated.

-	The eNB locally UP IP is set to NOT NEEDED.

-	Tester has knowledge of integrity algorithm and integrity protection keys.

-	The tester can capture the message via the Uu interface, or can capture the message at the UE.

-	The tester  disables EIA0 at the eNB.

Execution Steps:

1. 	The tester triggers the MME to send a EPS security capability message with EIA7 indicating the UP IP is supported by the UE to the eNB. But the tester also configures the MME to send a UP IP policy with REQUIRED to the eNB.

2. 	The eNB sends a RRCConnectionReconfiguration message with integrity protection indication "on" to the UE.

3. 	The tester checks that any user data sent by eNB after sending the RRCConnectionReconfiguration message and while the UE is in active state is integrity protected.

Expected Results:

Any user plane packets sent between UE and eNB over the Uu interface after eNB sending the RRCConnectionReconfiguration message is integrity protected according to the UP IP policy sent by the MME.

Expected format of evidence:

Evidence suitable for the interface e.g. Screenshot containing the operational results.

********** END OF 10th CHANGE **********

********** START OF 11th CHANGE **********

##### 4.2.2.1.19	UP IP policy selection in S1 Handover

Requirement Name: Select the right UP IP policy in S1 handover.

Requirement Reference: TS 33.401 [2] clause 7.3.3

Requirement Description: " At an S1-handover, the source MME shall send the UE's UP integrity protection policy and the UE EPS security capability to the target eNB via the target MME. Besides, the source eNB shall also send the UE's UP integrity protection policy if received from the source MME to the target eNB in a source-to-target container. The target eNB shall use the UE capability indicating support of UP IP in EPS together with the UP integrity protection policy received from the MME and ignore the UP integrity protection received in the source-to-target container. If the target eNB does not receive the UP integrity protection policy from the MME, the target eNB shall use the UE capability indicating support of UP IP in EPS together with the UP integrity protection policy received from the source eNB. If both policies from MME and source eNB are absent, but EIA7 in the EPS security capability indicates that the UE supports use of user plane protection with EPC, the eNB shall use locally configured UP integrity protection policy." in clause 7.3.3

Threat References: TR 33.926 [4], clause C.2.2.5, UP integrity protection policy selection

Test Case:

Test Name: TC_UP_IP_POLICY_Selection_S1_Handover

Purpose: To verify that the eNB has correct selection on UP IP policy in S1 handover

Pre-Condition:

-	The target eNB network product is connected in emulated/real network environments. UE, source eNB and MME may be simulated.

-	The target eNB locally UP IP is set to NOT NEEDED.

-	The tester has knowledge of integrity algorithm and integrity protection keys.

-	The tester can capture the message via the Uu interface, or can capture the message at the UE.

Execution Steps:

Test Case 1:

1)	The tester triggers the source MME to send EPS security capability with EIA7 to the target eNB indicating the UP IP is supported by the UE. Furthermore, the tester triggers the source MME to send a UP IP policy with REQUIRED to the target eNB.

2)	The source eNB sends UP IP policy with NOT NEEDED in the source-to-target container to the target eNB.

3)	The target eNB sends a RRCConnectionReconfiguration message with integrity protection indication "on" to the UE.

4)	The tester checks that any user data sent by the target eNB after sending the RRCConnectionReconfiguration message and before UE enters CM-Idle state is integrity protected.

Test Case 2:

1)	The tester triggers the source MME to send EPS security capability with EIA7 to the target eNB indicating the UP IP is supported by the UE. Furthermore, the tester prepares the MME to not send a UP IP policy to the target eNB.

2)	The source eNB sends UP IP policy with REQUIRED in the source-to-target container to the target eNB.

3)	The target eNB sends a RRCConnectionReconfiguration message with integrity protection indication "on" to the UE.

4)	The tester checks that any user data sent by eNB after sending the RRCConnectionReconfiguration message and before UE enters CM-Idle state is integrity protected.

Test Case 3:

1)	The tester configures the target eNB to make sure the local UP IP is set to REQUIRED.

2)	The tester triggers the source MME to send EPS security capability with EIA7 to the target eNB indicating the UP IP is supported by the UE. Furthermore, the tester prepares the MME to not send a UP IP policy to the target eNB.

3)	The source eNB does not send UP IP policy in the source-to-target container to the target eNB.

4)	The target eNB sends a RRCConnectionReconfiguration message with integrity protection indication "on" to the UE.

5)	The tester	checks that any user data sent by eNB after sending the RRCConnectionReconfiguration message and before UE enters CM-Idle state is integrity protected.

Expected Results:

For all test cases, any user plane packets sent between UE and eNB over the Uu interface after eNB sending the RRCConnectionReconfiguration message are integrity protected.

Expected format of evidence:

Evidence suitable for the interface e.g. Screenshot containing the operational results.

For each test case: Configuration of UP IP of target eNB, source eNB and UP IP policy sent by MME.

********** END OF 11th CHANGE **********

********** START OF 12th CHANGE **********

##### 4.2.2.1.20	Bidding down prevention for UP IP Policy

Requirement Reference: TS 33.401 [3], clause 7.3.3

Requirement Description: "Further, in the Path-Switch message, the target eNB shall send the UE's UP integrity protection policy and corresponding E-RAB ID to the MME. The sent UP integrity protection policy can either be the one received from source eNB or the locally configured one if the target eNB does not receive it from the source eNB, but the EIA7 in the EPS security capability indicates that the UE supports user plane integrity protection with EPC. " as specified in TS 33.401 [3], clause 7.3.3.

Threat References: TR 33.926 [4], clause C.2.2.a, bidding down for UP IP Policy

Test Case:

Test Name: TC_ENB_BIDDING_DOWN_UP_IP_POLICY

Purpose:

Verify that bidding down for UP IP policy is prevented in X2-handovers.

Pre-Conditions:

The target eNB network product is connected in emulated/real network environments. UE, source eNB and MME may be simulated.

Execution Steps:

Test Case 1:

1.	The tester configures the target eNB with UP IP set to NOT NEEDED.

2.	The tester triggers the source eNB to send EPS security capability with EIA7 to the MME indicating the UP IP is supported and UP IP policy with REQUIRED in Handover Request message to the target eNB.

3.	The target eNB sends path-switch request message with UP IP policy with REQUIRED to the MME.

Test Case 2:

1.	The tester configures the target eNB with UP IP set to REQUIRED.

2.	The tester triggers the source eNB to send EPS security capability with EIA7 to the MME indicating the UP IP is supported in Handover Request message to the target eNB. The tester prepares the source eNB to not send UP IP policy in the Handover Request message.

3.	The target eNB sends path-switch request message with UP IP policy with REQUIRED to the MME.

Expected Results:

For both test cases, the UP IP policy with REQUIRED is in the path-switch request message.

Expected format of evidence:

Snapshots containing the result.

For each test case: Configuration of UP IP of target eNB, source eNB and UP IP policy sent by MME.

********** END OF CHANGE **********