3GPP TSG-SA3 Meeting #122	S3-252403-r2

# Fukuoka, JP, 19 - 23 May 2025

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.122 | CR | DRAFT | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resource Owner authentication revocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Chinatelecom, Nokia, Lenovo, Huawei, CATT, Samsung, ZTE, HiSilicon, Xiaomi communications, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | CAPIF_Ph3_sec |  |  |  |  |  | Date: |  |  | 2025-02-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This CR defines detailed procedure about resource owner authorization revocation to keep align with the conclusions for KI#1.2 in TR 33.700-22. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Normative text provided to clarify resource owner authorization revocation information and corresponding procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Mis-implementation and cause incompatibility. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Baseline S3-251113 approved in SA3 #120 merging S3-250410 and S3-250479 Adding S3-251712 approved in SA3 #121 merging S3-251212, S3-251388, S3-251459, S3-251472 Adding S3-252291 approved in SA3 #122 merging S3-251931, S3-251945, S3-251973, S3-252040, S3-252076, S3-252218, S3-252233, S3-252252 |  |  |  |  |  |  |  |  |

* * * First Change * * * *

6.5.3.4	Revocation

The CCF can initiate the Authorization Revocation Request message as defined in clause 8.23.4 of TS 23.222 [3] with additional information to identify the RNAA-related revoked token.

NOTE:  The CCF can receive a revocation request message from the resource owner via the UE, resource owner function, web page etc.

AEF, storing the information about the RNAA-related revoked token, shall check whether the token presented by an API invoker is revoked or not, before responding to the API invoker’s invocation request.

The CCF provided notification message to the API invoker shall include the information to identify the RNAA-related revoked token.

The procedure illustrated in Figure 6.5.3.4-1 and explained below can be used for revoking RNAA-related token in RNAA scenarios.

Pre-conditions:

1.	The API invoker is authenticated and authorized to use the service API.

API invoker

Resource owner function

CAPIF Core function

AEF

0. TLS session establishment.

1. Resource owner authorization revocation request

2.  Identify the RNAA-related revoked token.

3. Revoke API invoker authorization request

4. Invalidate the authorization of API invoker for service API and resource

5-7: Same steps as the steps 4-6 in clause 8.23.4 of TS 23.222

8. Resource owner authorization revocation response

Figure 6.5.3.4-1: Procedure for revoking resource owner authorization

0.	CCF and ROF shall establish TLS session over CAPIF-8 reference point as specified in clause 6.11.

1.	Triggered by the resource owner, the resource owner function sends a resource owner authorization revocation request to the CCF.

The resource owner authorization revocation information in the request message shall include the API invoker information (where the API invoker may be either an application on a server, or an application on a UE as specified in TS 23.222 [3]) and information related to service API (e.g., service operation information, service information).

The request may include Resource Owner ID (e.g., GPSI), resource information and application identifier. The request may include other information specified in TS 23.222 [3].

Editor’s Note: Whether purpose of data processing is needed is FFS.

2.	The CCF determines the information of the resource owner ID, API invoker ID and the service API in the scope of the token based on the received resource owner authorization revocation information to identify the RNAA-related token to be revoked.

3.	The CCF sends the Revoke resource authorization request message to the AEF as defined in clause 8.35 of TS 23.222 [3] with information in RNAA-related token (e.g., token itself, token id) to identify the RNAA-related revoked token.

4.	The AEF, storing the information about the RNAA-related revoked token, checks whether the token presented by an API invoker is revoked or not, before responding to the API invoker’s invocation request.

NOTE:	The need of updating the resource due to the token revocation and how to do it if resource update is needed are not in the scope of the present document.

5-7. Step 5-7 are the same as in steps 4-6 in clause 8.23.4 of TS 23.222 [3].

8.   The CCF sends a resource owner authorization revocation response as an acknowledgement to the ROF that the resource owner authorization has been revoked.

* * * End of Changes * * * *