3GPP TSG-SA3 Meeting #122	S3-251852

Fukuoka, Japan, 19 – 23 May 2025

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.515 | CR | 0012 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of test names and clean up of 33.515 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | BSI (DE) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SCAS_5G_Maint |  |  |  |  |  | Date: |  |  | 2025-04-24 |
|  | D |  |  |  |  |  |  |  |  |  |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Incorrect test naming and a few format glitches and typos |  |  |  |  |  |  |  |  |
| Summary of change: |  | Test names, formats and typos corrected |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Non-standard test case naming, formats and typos remain and test automation may be affected |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.2.1.3, 4.2.2.1.4, 4.4.1 - 4.4.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

********** START OF 1st CHANGE **********

##### 4.2.2.1.3	Security functional requirements on the SMF checking UP security policy

Requirement Name: UP security policy check.

Requirement Reference: TS 33.501 [8], clause 6.6.1

Requirement Description: According to TS 33.501 [8], clause 6.6.1, the SMF verifies that the UE's UP security policy received from the target ng-eNB/gNB is the same as the UE's UP security policy that the SMF has locally stored. If there is a mismatch, the SMF sends its locally stored UE's UP security policy of the corresponding PDU sessions to the target gNB. This UP security policy information, if included by the SMF, is delivered to the target ng-eNB/gNB in the Path-Switch Acknowledge message. The SMF logs capabilities for this event and may take additional measures, such as raising an alarm.

Threat References: TR 33.926 [4], clause J.2.2.4, Unchecked UP security policy.

TEST CASE:

Test Name: TC_UP_SECURITY_POLICY_SMF

Purpose:

Verify that the SMF checks the UP security policy that is sent by the ng-eNB/gNB during handover.

Pre-Conditions:

The SMF under test is preconfigured with a UE UP security policy.

Execution

1.	The tester sends the Nsmf_PDUSession_UpdateSMContext Request message to the SMF under test. A UE UP security policy different than the one preconfigured at the SMF under test is included in the Request message.

2.	The tester captures the Nsmf_PDUSession_UpdateSMContext Response message sent from the SMF under test.

Expected Results:

The preconfigured UE security policy is contained in the ‘n2SmInfo’ IE in the captured Response message.

Expected format of evidence:

Files containing the triggered HTTP messages (e.g. pcap trace).

********** END OF 1st CHANGE **********

********** START OF 2nd CHANGE **********

##### 4.2.2.1.4	Charging ID Uniqueness

Requirement Name: Charging ID uniqueness.

Requirement Reference: TS 32.255 [6], clause 5.1.2

Requirement Description: According to TS 32.255 [6], clause 5.1.2:

-		The SMF supports PDU session charging using service based interface.

-		The SMF collects charging information per PDU session for UEs served under 3GPP access and non-3GPP access.

-	Every PDU session is assigned a unique identity number for billing purposes per PLMN. (i.e. the Charging Id).

Threat Reference: TR 33.926 [4], clause J.2.2.3, "Failure to assign unique Charging ID for a session"

TEST CASE:

Test Name: TC_CHARGING_ID_UNIQUENESS_SMF

Purpose:

Verify that the charging ID generated by the SMF for each PDU session is unique.

Pre-Conditions:

Test environment is set up with a Charging Function (CHF), which may be real or simulated, and the SMF under test. The tester is able to capture the traffic between the SMF under test and the CHF.

Execution Step

1)	The tester intercepts the traffic between the SMF under test and the CHF.

2)	The tester triggers the establishment of the maximum number of concurrent PDU sessions that the SMF under test can handle.

3)	The tester captures each Charging Data Request [initial] sent from the SMF under test to the CHF, and verifies the charging ID contained in the ‘PDU Session Charging Information’ IE in each Charging Data Request [initial] is unique.

Expected Results:

The charging ID in each Charging Data Request [initial] is unique.

Expected format of evidence:

Files containing the Charging Data Request [initial] messages (e.g. pcap trace).

********** END OF 2nd CHANGE **********

********** START OF 3rd CHANGE **********

### 4.4.1	Introduction

There are no SMF specific additions to clause 4.4.1 of TS 33.117 [2].

********** END OF 3rd CHANGE **********

********** START OF 4th CHANGE **********

### 4.4.2	Port Scanning

There are no SMF specific additions to clause 4.4.2 of TS 33.117 [2].

********** END OF 4th CHANGE **********

********** START OF 5th CHANGE **********

### 4.4.3	Vulnerability scanning

There are no SMF specific additions to clause 4.4.3 of TS 33.117 [2].

********** END OF 5th CHANGE **********

********** START OF 6th CHANGE **********

### 4.4.4	Robustness and fuzz testing

The test cases under clause 4.4.4 of TS 33.117 [2] are applicable to SMF.

The interfaces defined for the SMF are defined in clause 4.2.3 of TS 23.501 [1].

According to clause 4.4.4 of TS 33.117 [2], the transport protocols available on the interfaces providing IP-based protocols need to be robustness tested. Following TCP/IP layer model and considering all the protocols over transport layer, for SMF, the following interfaces and protocols are in the scope of the testing:

-	For Nsmf: The TCP, HTTP2 and JSON protocols.

-	For N4: The UDP and PFCP protocols.

NOTE:	There could be other interfaces and/or protocols requiring testing under clause 4.4.4 of TS 33.117 [2]

********** END OF CHANGE **********