3GPP TSG-SA3 Meeting #122																			S3-251908

Fukuoka, Japan, 19 – 23 May 2025

# Source:	KPN

# Title:	AIoT baseline requirements

# Document for:	Approval

Agenda Item:	4.26

# 1	Decision/action requested

It is requested to approve the following pCR.

# 2	References

[1]	3GPP TS 33.369, version 0.1.0, Security aspects of Ambient IoT service

[2]		3GPP TS 23.369, version 0.3.0, Architecture support for Ambient power-enabled Internet of Things;Stage 2

[3]	3GPP TS 38.300, NR; NR and NG-RAN Overall Description; Stage 2

[4]	3GPP TS 38.391, Ambient IoT Medium Access Control (MAC) Protocol specification

[5]	R2-2504398, draftCR for 38.300 v18.5.0, 38.300 running CR for Ambient IoT

[6]	R2-2503997, draft TS, TS 38.391 v0.0.2, Ambient IoT Medium Access Control Protocol specification

# 3	Rationale

The procedures and message contents of the security procedures shall be based on the SA2 and RAN2 baselines and agreements.

The SA2 baseline is specified in 3GPP TS 23.369 [2] , which currently has version 0.3.0. In particular the procedures in clause 6.2.2 (for Inventory Only) and clause 6.2.3 (for Inventory-and-Command) are relevant.

The RAN2 baseline and agreements consist of the specification 3GPP TS 38.300 [3], which needs to be updated with the endorsed draftCR R2-2504398 [5], and the new specification 3GPP TS 38.391 [4], which is captured in the endorsed document R2-2503997 [6]. In 3GPP TS 38.300, in particular the statements about paging, random access and data transmission given in clause 16.x.5 are relevant as well as the RAN2 agreements collected in the Annexes. In 3GPP TS 38.391, in particular the statements about architecture given in clause 4.2 and the statements about the paging message in clause 5.2 and clause 6.2.1.1 are relevant.

Note: from the current specifications of RAN2 the following can be observed:

- The paging message contains one Paging ID or no Paging ID. The Paging ID is the only IE that is visible to the upper layers. Other IEs in the paging message are not visible to the upper layers.

- The paging message does not contain an indication on the service type (Inventory Only or Inventory-and-Command) that the device is expected to perform.

- The random access messages are not visible to the upper layer.

- The (first) D2R message after the paging message contains a Data SDU. The Data SDU may contain a Device ID or other data.

# 4	Detailed proposal

It is proposed to make the following changes to 3GPP TS 33.369, version 0.1.0 [1]

FIRST CHANGE

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]		3GPP TS 23.369: "Architecture support for Ambient power-enabled Internet of Things".

[3]	3GPP TS 38.300: "NR; NR and NG-RAN Overall description; Stage-2".

[4]	3GPP TS 22.369: "Service requirements for Ambient power-enabled IoT".

[5]	3GPP TS 33.501: "Security architecture and procedures for 5G System".

[x]	3GPP TS 38.391: "Ambient IoT Medium Access Control Protocol specification".

NEXT CHANGE

# 5	Security procedures for Ambient IoT service

## 5.1	General

Editor’s Note: This clause contains the general security aspects.

# 5	Security procedures for Ambient IoT service

## 5.1	General

Editor’s Note: This clause contains the general security aspects.

The security procedures for Ambient IoT service are based on the SA2 architecture, procedures, and message content as specified in 3GPP TS 23.369 [2] and on the RAN2 procedures and message content as specified in 3GPP TS 38.300 [3] and 3GPP TS 38.391 [x].

END OF CHANGES