3GPP TSG-SA3 Meeting #122	S3-251941

# Fukuoka, Japan,  19 - 23 May 2025

Source:	ZTE

Title:	Resolving EN for Authentication and authorization for spatial localization services

Document for:	Approval

Agenda item:	4.21

Spec:	TS 33.434

Version:	18.2.0

Work Item:	Security aspects of 5G Mobile Metaverse services

Comments

This pCR proposes to resolve the ENs in clause 6.x.1 according to the progress in CAPIF_Ph3_Sec.

Proposed Changes

* * * First Change * * * *

6.x.1 Authentication and authorization for spatial anchor/map services when CAPIF is used

When CAPIF is used as specified in TS 23.434 [2], the security mechanism for CAPIF specified in TS 33.122 [29] is referred to protect interfaces between spatial anchors (SAn) client and spatial anchors server specified in TS 23.437 [x], between VAL server and spatial anchors server, between spatial map (SM) client and spatial map server specified in TS 23.437 [x], VAL server and spatial map server. Spatial anchors client, spatial map client and VAL server take role of API Invoker in CAPIF. Spatial anchors server and spatial map server take role of AEF in CAPIF.

The authentication and authorization mechanisms specified for CAPIF-2e reference point in TS 33.122 [29] clause 6.5.2 shall be followed for authenticating and authorizing a SAn/SM client to access the spatial anchor/map services defined in TS 23.437 [x] clauses 8 and 9.

If the owner of the spatial anchor or spatial map is provided in the spatial anchor/map service request, authentication and authorization for RNAA specified in clause 6.5.3 of TS 33.122 [29] are followed to authenticate and authorize a SAn/SM client to access the spatial anchor/map of a specific owner.

NOTE x:	when applying authentication and authorization for RNAA, the owner of the spatial anchor/map has to be a UE subscriber.

NOTE y: How CAPIF RNAA supports service operation level and resource level authorization using access token is specified in TS 33.122 [29].

* * * End of Changes * * * *