3GPP TSG-SA3 Meeting #122	S3-251944

# Fukuoka, Japan,  19 - 23 May 2025

Source:	ZTE

Title:	Resolving EN for Method 3: TLS with OAuth Token

Document for:	Approval

Agenda item:	4.22

Spec:	TS 33.122

Version:	19.0.0

Work Item:	Security aspects of CAPIF Phase3

Comments

This pCR proposes to resolve the EN in clause 6.X.3.3:

Editor’s Note: Whether CCF-A will have sufficient information to authenticate the API invoker before including API invoker ID into the access token is FFS.

Proposed Changes

* * * First Change * * * *

6.X.3.3		Method 3: TLS with OAuth Token

- The API invoker shall send the access token request message to the onboarded CCF-B, CCF-B determines that the service API requested is provided by the AEFs in CAPIF provider domain A.

- The CCF-B sends the access token request message to the CCF-A in CAPIF provider domain A. In interconnection scenario, the Onboard_secret is not included in the access token request message to the CCF-A. Upon receiving the access token request message, the CCF-A authenticates the API invoker. If CCF-A does not have sufficient information for authentication, CCF-A fetches additional information related to the API invoker from CCF-B.  If valid, the CCF-A generates and provides an access token specific to the API invoker and returns it in an Access Token Response message to the API invoker via CCF-B as specified in clause 6.5.2.3.

- On CAPIF-2e, the API invoker authenticates to the AEF by establishing a TLS session with the AEF as specified in clause 6.X.3.1 or 6.X.3.2.

- With successful authentication to the AEF on CAPIF-2e, the API invoker shall initiate invocation of a 3GPP northbound API with the AEF including the access token in northbound API invocation request as per OAuth 2.0. The AEF verifies the integrity of the access token by verifying the CCF-A’s signature to validate the access permission for the requested service API as specified in clause 6.5.2.3.

* * * End of Changes * * * *