3GPP TSG-SA3 Meeting #122	S3-251960

# Fukuoka, JP, 19 - 23 May 2025

Source:	Chinatelecom

Title:	KI#2 pCR on top of the baseline pCR- interconnect

Document for:	Approval

Agenda item:	4.22

Spec:	3GPP TS 33.122

Version:	v19.0.0

Work Item:	CAPIF_Ph3_sec

1	Decision/action requested

It is proposed to correct the access token request message in clause 6.x.3.3 of the baseline to make the procedure workable.

2	References

[1]	3GPP TR 33.700-22: "Study on security aspects of CAPIF Phase3".

3	Rationale

Based on the baseline pCR from pre-meeting, clause 6.5.2 of TS 33.122 is reused for authentication and authorization in CAPIF interconnection. Specified in Annex C.3.2 of TS 33.122, the parameter ‘scope’ in Access token request message is optional.

Table C.3.2-1: Access token request message parameters

| Parameter | Values |
| --- | --- |
| ….. | …… |
| scope | OPTIONAL. A string containing a space-delimited list, comprising of the following as scopes associated with this token: - List of Services per AEF (e.g. “AEF1:Service1,Service2,Service3,...,ServiceX; AEF2:Service1,Service2,Service3,...,ServiceZ”) |

And it’s mentioned in clause 6.x.3.3 of the baseline that:

-	The API invoker shall send the access token request message to the onboarded CCF-B, CCF-B determines that the service API requested is provided by the AEFs in CAPIF provider domain A.

-	The CCF-B sends the access token request message to the CCF-A in CAPIF provider domain A. In interconnection scenario, the Onboard_secret is not included in the access token request message to the CCF-A. The CCF-A generates and provides an access token specific to the API invoker and returns it in an Access Token Response message to the API invoker via CCF-B as specified in clause 6.5.2.3.

If the ‘scope’ in Access token request message is still optional in CAPIF interconnection, the CCF-B cannot determine which CAPIF provider domain provides the requested service API and which CCF to request for the access token.

So, to make the procedure for authorization in CAPIF interconnection workable, it’s proposed that the parameter ‘scope’ in Access token request message is required in CAPIF interconnection.

4	Detailed proposal

* * * First Change * * * *

6.X.3.3		Method 3: TLS with OAuth Token

- The API invoker shall send the access token request message to the onboarded CCF-B, CCF-B determines that the service API requested is provided by the AEFs in CAPIF provider domain A. In interconnection scenario, the parameter ‘scope’ in Access token request message is required.

- The CCF-B sends the access token request message to the CCF-A in CAPIF provider domain A. In interconnection scenario, the Onboard_secret is not included in the access token request message to the CCF-A. In interconnection scenario, the parameter ‘scope’ in Access token request message is required. The CCF-A generates and provides an access token specific to the API invoker and returns it in an Access Token Response message to the API invoker via CCF-B as specified in clause 6.5.2.3.

- On CAPIF-2e, the API invoker authenticates to the AEF by establishing a TLS session with the AEF as specified in clause 6.X.3.1 or 6.X.3.2.

- With successful authentication to the AEF on CAPIF-2e, the API invoker shall initiate invocation of a 3GPP northbound API with the AEF including the access token in northbound API invocation request as per OAuth 2.0. The AEF verifies the integrity of the access token by verifying the CCF’s signature to validate the access permission for the requested service API as specified in clause 6.5.2.3.

Editor’s Note: Whether CCF-A will have sufficient information to authenticate the API invoker before including API invoker ID into the access token is FFS.

* * * End of Change * * * *