3GPP TSG-SA3 Meeting #122	S3-252001

Fukuoka, Japan, 19 - 23 May 2025

# Source:	Huawei, HiSilicon

# Title:	Discussion paper of the certificate identifier for public key distribution

# Document for:	Discussion

Agenda Item:	4.11

1	Decision/action requested

It is proposed to discuss the certificate identifier for public key distribution.

2	References

[1]	IETF RFC 7515: "JSON Web Signature (JWS) ".

[2]	IETF RFC 5280: "Internet X.509 Public Key Infrastructure Certificate and Certificate Revocation List (CRL) Profile".

3	Rationale

3.1 Public key distribution service support discussion

As discussed during SA3#121, the public key used to verify the access token signature can be provided by the NRF in a raw public key format or contained in the X.509 public key certificate or certificate chain. The Key ID in the access token can be used to identify the NRF’s raw public key that can be used to verify the signature of the access token. However, how to identify the X.509 public key certificate is still under discussion. Two options are being discussed: the certificate fingerprint "x5t#S256" versus the key ID

3.2	Observations

Observation 1: JSON Web Signature support both key ID and x5t#S256 as optional head parameters.

As specified in IETF RFC 7515 [1], JSON Web Signature supports both "kid" (Key ID) and "x5t#S256" (X.509 Certificate SHA-256 Thumbprint) Header Parameters, and for both of these header parameters, use is OPTIONAL.

Observation 2: The issuer name and serial number of the certificate can be used to construct the key ID to identify the NRF X.509 public key certificate.

As specified in clause 4.1.2.2 of IETF RFC 5280 [2],

"The serial number MUST be a positive integer assigned by the CA to each certificate.  It MUST be unique for each certificate issued by a given CA (i.e., the issuer name and serial number identify a unique certificate). "

Therefor, the issuer name and serial number profile of the X.509 certificate can be used to identify a unique certificate, and can be used to construct a Key ID which is used to identify the NRF X.509 public key certificate.

Observation 3: Using certificate fingerprint x5t#S256 incurs additional complexity.

Since there is no fingerprint in X.509 certificate, the processor has to calculate the fingerprint and maintain such data somehow. Such burden increases, if there are many certificates.

4	Detailed proposal

Based on the rationale above, it is proposed to support key ID to identify a X.509 certificate in JWS profile, "x5t#S256" can also be supported but can not be a mandatory option.