3GPP TSG-SA3 Meeting #122	S3-252059

# Fukuoka, Japan, 19 – 23 May 2025

Source:	China Mobile

Title:	Pseudo-CR on resolving ENs of sequence number of figure in solution #45 of TR 33.713

Document for:	Approval

Agenda item:	5.9

Spec:	3GPP TR 33.713

Version:	0.8.0

Work Item:	FS_Ambient_IoT_Sec

Comments

This contribution provides updates to solution #45 to address the following Editor’s Notes.

Editor’s note: The sequence number of message needs to be added in Figure 6.45.2-1.

Editor’s note: The sequence number of message needs to be added in Figure.

Editor’s note: The sequence number of message needs to be added in Figure 6.45.2-2.

Proposed Changes

* * * First Change * * * *

6.45.2	Solution details

Temp ID generation and sync up procedure between device and 5GC

Figure 6.45.2-1: Procedure for Temp ID generation and sync up between device and 5GC

Procedure:

1. When AF wants to trigger inventory request to an AIoT device, it sends request with AIoT permanent device ID and AF ID.

2. When 5G network receives the request, it enables privacy protection based on AF ID. After that, it checks whether there is temporary ID mapped to the permanent device ID. If no, it sends inventory request with AIoT permanent device ID and ID response indication, which is set to represent "report both Temp ID and Permanent device ID".

3. When AIoT device receives this request, it generates temp ID based on the indication, and write it locally. After that, it sends inventory response with its permanent device ID and temp ID together.

4. 5G network gets the binding relationship between permanent ID and temp ID from received response. Then 5G network forward permanent ID to AF in inventory response message.

Editor’s note: It’s not clear how the permanent ID is protected in up and downlink.

Editor’s note: It is FFS how to keep uniqueness of temp ID.

Downlink and Uplink Privacy protection based on Temp ID

Figure 6.45.2-2: Procedure for Downlink and Uplink Privacy protection based on Temp ID

Procedure:

- When AF wants to trigger inventory request to an AIoT device, it sends request with AIoT permanent device ID and AF ID.

- When 5G network receives the request, it enables privacy protection based on AF ID. After that, it checks whether there is temporary ID mapped to the permanent device ID. If yes, it sends inventory request with temp ID to provide privacy protection, and ID response indication which is set to represent "report Temp ID only".

- When AIoT device receives this request based on its temp ID, it sends inventory response with its temp ID.

3a. If 5G network could not retrieve AIoT device permanent ID or failed to get response. It needs to relaunch inventory request with AIoT permanent device ID and "report both Temp ID and Permanent device ID" ID response indication.

- 5G network gets the binding relationship between permanent ID and temp ID from received response retrieve its permanent ID by using temp ID from received response. Then 5G network forward permanent ID to AF in inventory response message.

Temp ID update and sync up procedure

Figure z: Procedure for Temp ID update and sync up

Procedure:

- When AF wants to trigger inventory request to an AIoT device, it sends request with AIoT permanent device ID and AF ID.

- When 5G network receives the request, it enables privacy protection based on AF ID. After that, it checks whether there is temporary ID mapped to the permanent device ID. If yes, it may decide to update temp ID based on local policy. It sends inventory request with temp ID to provide privacy protection, and ID response indication which is set to represent "update temp ID, and report both temp ID and permanent device ID ".

- When AIoT device receives this request based on its temp ID, it calculates a new temp ID and save it locally, then sends inventory response with its new temp ID and permanent ID together.

3a. If 5G network fails to get response. It needs to relaunch inventory request with AIoT permanent device ID and "report both Temp ID and Permanent device ID" ID response indication.

- 5G network gets the binding relationship between permanent ID and temp ID from received response and update its mapping table. Then 5G network forward permanent ID to AF in inventory response message.

Editor’s note: It is not clear how the ID is stored in the solution.

Editor’s note: Tracking or tracing of AIoT device using the unprotected ID response indicator is not addressed in the present document.

Editor’s note: It is FFS how to keep uniqueness of temp ID.

* * * End of Changes * * * *