3GPP TSG-SA3 Meeting #122	S3-252104

# Fukuoka, Japan, 19 - 23 May 2025

Source:	Apple

Title:	Security procedure for inter-CU LTM

Document for:	Approval

Agenda item:	4.12

Spec:	3GPP TS 33.501

Version:	19.2.0

Work Item:	NR_Mob_Ph4_Sec

Comments

This pCR proposes normative text to the draftCR for security procedure for inter-CU LTM.

* * * First Change * * * *

6.X.Y	Security mechanism and procedures for inter-CU LTM

Editor’s Notes: This clause contains the security procedure for inter-CU LTM in 5G.

6.X.Y.1	General

LTM is a procedure in which a gNB receives L1 or L3 measurement report(s) from a UE, and on their basis the gNB may change UE serving cell by a cell switch command signalled via a MAC CE, as specified in TS 38.300 [52]. The cell switch command indicates an LTM candidate configuration that the gNB previously prepared and provided to the UE through RRC signalling. Then the UE switches to the target configuration according to the cell switch command. This clause includes the security procedues involved in the LTM procedures.

6.X.Y.2	When DC is not configured

For the case where CU is acting as MN and DC is not configured as specified in TS 38.300 [52], in LTM preparation phase, the UE security context are sent by the initial/source gNB to all the candidate gNBs. The source gNB computes the KNG-RAN* (s) (per candidate cell) as indicated in Annex A.11 for the candidate gNB, and forwards the {KNG-RAN* , NCC} pair(s) to the candidate gNB(s) via a Handover Request message as legacy. The new source gNB needs to provide the new KNG-RAN* (s) to the corresponding candidate gNB(s) via a LTM configuration update procedure.

During LTM execution phase, the source gNB includes the NCC used for the derivation of KNG-RAN* in the Cell Switch Command MAC CE to the UE. UE switches to the target cell and use this NCC to derive the KNG-RAN* and apply this key for the communication with the target gNB

6.X.Y.3	When DC is configured

In the case where CU is acting as SN and MN is unchanged as specified in TS 37.340 [51], the security mechanism and procedures as specified in clause 6.10.2.4 of this specification for SCPAC shall be applied.

* * * End of Changes * * * *